Class 04: Location (time)

| FXY | ELEMENT NAME | BUFR UNIT | SCALE | REF VALUE | BITS | CREX UNIT | SCALE | CHARS | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 04 001 | Year | a | 0 | 0 | 12 | a | 0 | 4 |  |
| 0 04 002 | Month | mon | 0 | 0 | 4 | mon | 0 | 2 |  |
| 0 04 003 | Day | d | 0 | 0 | 6 | d | 0 | 2 |  |
| 0 04 004 | Hour | h | 0 | 0 | 5 | h | 0 | 2 |  |
| 0 04 005 | Minute | min | 0 | 0 | 6 | min | 0 | 2 |  |
| 0 04 006 | Second | s | 0 | 0 | 6 | s | 0 | 2 |  |
| 0 04 007 | Seconds within a minute (microsecond accuracy) | s | 6 | 0 | 26 | s | 6 | 8 |  |
| 0 04 011 | Time increment | a | 0 | -1024 | 11 | a | 0 | 4 |  |
| 0 04 012 | Time increment | mon | 0 | -1024 | 11 | mon | 0 | 4 |  |
| 0 04 013 | Time increment | d | 0 | -1024 | 11 | d | 0 | 4 |  |
| 0 04 014 | Time increment | h | 0 | -1024 | 11 | h | 0 | 4 |  |
| 0 04 015 | Time increment | min | 0 | -2048 | 12 | min | 0 | 4 |  |
| 0 04 016 | Time increment | s | 0 | -4096 | 13 | s | 0 | 4 |  |
| 0 04 017 | Reference time period for accumulated or extreme data | min | 0 | -1440 | 12 | min | 0 | 4 |  |
| 0 04 021 | Time period or displacement | a | 0 | -1024 | 11 | a | 0 | 4 |  |
| 0 04 022 | Time period or displacement | mon | 0 | -1024 | 11 | mon | 0 | 4 |  |
| 0 04 023 | Time period or displacement | d | 0 | -1024 | 11 | d | 0 | 4 |  |
| 0 04 024 | Time period or displacement | h | 0 | -2048 | 12 | h | 0 | 4 |  |
| 0 04 025 | Time period or displacement | min | 0 | -2048 | 12 | min | 0 | 4 |  |
| 0 04 026 | Time period or displacement | s | 0 | -4096 | 13 | s | 0 | 4 |  |
| 0 04 031 | Duration of time relating to following value | h | 0 | 0 | 8 | h | 0 | 3 |  |
| 0 04 032 | Duration of time relating to following value | min | 0 | 0 | 6 | min | 0 | 2 |  |
| 0 04 041 | Time difference, UTC - LMT | min | 0 | -1440 | 12 | min | 0 | 4 | Note 48: The latitude and longitude descriptors 0 05 001 and 0 06 001 indicate the position of the instrument and not the measurement itself (which is separately encoded in the associated data sequence). |
| 0 04 043 | Day of the year | d | 0 | 0 | 9 | d | 0 | 3 |  |
| 0 04 051 | Principal time of daily reading of maximum temperature | h | 0 | 0 | 5 | h | 0 | 2 |  |
| 0 04 052 | Principal time of daily reading of minimum temperature | h | 0 | 0 | 5 | h | 0 | 2 |  |
| 0 04 053 | Number of days with precipitation equal to or more than 1 mm | Numeric | 0 | 0 | 6 | Numeric | 0 | 2 |  |
| 0 04 059 | Times of observation used to compute the reported mean values | Flag table | 0 | 0 | 6 | Flag table | 0 | 2 |  |
| 0 04 065 | Short time increment | min | 0 | -128 | 8 | min | 0 | 2 |  |
| 0 04 066 | Short time increment | s | 0 | -128 | 8 | s | 0 | 2 |  |
| 0 04 073 | Short time period or displacement | d | 0 | -128 | 8 | d | 0 | 2 |  |
| 0 04 074 | Short time period or displacement | h | 0 | -128 | 8 | h | 0 | 2 |  |
| 0 04 075 | Short time period or displacement | min | 0 | -128 | 8 | min | 0 | 2 |  |
| 0 04 080 | Averaging period for following value | Code table | 0 | 0 | 4 | Code table | 0 | 2 |  |
| 0 04 086 | Long time period or displacement | s | 0 | -8192 | 15 | s | 0 | 5 |  |

Note 27: All times are Universal Time Coordinated (UTC) unless otherwise noted.

Note 105: Negative time periods or displacements shall be used to indicate time periods or displacements preceding the currently defined time.

Note 145: The significance of time periods or displacements may be indicated using the time significance code corresponding to table reference 0 08 021.

Note 149: The time location, when used with forecast values, shall indicate the time of the initial state for the forecast, or the beginning of the forecast period; when used with ensemble means of forecast values, the time location shall indicate the initial state or the beginning of the first forecast over which ensemble means are derived.

Note 158: Time periods or displacements and time increments require an initial time location to be defined prior to their use, followed where appropriate by a time significance definition.

Note 176: Where more than one time period or displacement is required to define complex time structures, they shall be defined in immediate succession, and the following ordering shall apply: ensemble period (if required), followed by forecast period (if required), followed by period for averaging or accumulation (if required).