Class 06: Location (horizontal - 2)

| FXY | ELEMENT NAME | BUFR UNIT | SCALE | REF VALUE | BITS | CREX UNIT | SCALE | CHARS | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 06 001 | Longitude (high accuracy) | deg | 5 | -18000000 | 26 | deg | 5 | 8 |  |
| 0 06 002 | Longitude (coarse accuracy) | deg | 2 | -18000 | 16 | deg | 2 | 5 |  |
| 0 06 011 | Longitude increment (high accuracy) | deg | 5 | -18000000 | 26 | deg | 5 | 8 |  |
| 0 06 012 | Longitude increment (coarse accuracy) | deg | 2 | -18000 | 16 | deg | 2 | 5 |  |
| 0 06 015 | Longitude displacement (high accuracy) | deg | 5 | -18000000 | 26 | deg | 5 | 8 |  |
| 0 06 016 | Longitude displacement (coarse accuracy) | deg | 2 | -18000 | 16 | deg | 2 | 5 |  |
| 0 06 021 | Distance | m | -1 | 0 | 13 | m | -1 | 4 |  |
| 0 06 029 | Wave number | /m | 1 | 0 | 22 | /m | 1 | 7 |  |
| 0 06 030 | Wave number (spectral) | rad/m | 5 | 0 | 13 | rad/m | 5 | 4 |  |
| 0 06 031 | Column number | Numeric | 0 | 0 | 12 | Numeric | 0 | 4 |  |
| 0 06 032 | X offset | m | 2 | -1073741824 | 31 | m | 2 | 11 | Note 183: X offset is the distance between the projection origin and the upper left corner of the upper left pixel in a map, as shown in the following diagram: {{image}}. |
| 0 06 033 | Pixel size on horizontal - 2 | m | -1 | 0 | 16 | m | -1 | 5 |  |
| 0 06 034 | Cross-track cell number | Numeric | 0 | 0 | 7 | Numeric | 0 | 3 |  |
| 0 06 035 | Maximum size of y-dimension | Numeric | 0 | 0 | 12 | Numeric | 0 | 4 |  |
| 0 06 040 | Radius of confidence | m | 0 | 0 | 13 | m | 0 | 4 |  |
| 0 06 041 | Earth scan sample number | Numeric | 0 | 0 | 11 | Numeric | 0 | 4 |  |
| 0 06 042 | Space/moon scan sample number | Numeric | 0 | 0 | 6 | Numeric | 0 | 2 |  |
| 0 06 043 | Sub-satellite point longitude | deg | 4 | -1800000 | 22 | deg | 4 | 8 |  |

Note 62: Distance shall only be used with respect to a stated location and a bearing, azimuth or elevation; it shall not redefine that location.

Note 66: East to west increments shall be assigned negative values.

Note 141: The pixel size on horizontal - 2 is given at location where map scale factor is unity.

Note 164: Values of longitude are limited to the range -180 degrees to +180 degrees.

Note 166: West longitude shall be assigned negative values.