Class 21: Radar data

| FXY | ELEMENT NAME | BUFR UNIT | SCALE | REF VALUE | BITS | CREX UNIT | SCALE | CHARS | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 21 001 | Horizontal reflectivity | dB | 0 | -64 | 7 | dB | 0 | 3 |  |
| 0 21 002 | Vertical reflectivity | dB | 0 | -64 | 7 | dB | 0 | 3 |  |
| 0 21 003 | Differential reflectivity | dB | 1 | -5 | 7 | dB | 1 | 3 |  |
| 0 21 004 | Differential reflectivity | dB | 2 | -800 | 11 | dB | 2 | 4 |  |
| 0 21 005 | Linear depolarization ratio | dB | 0 | -65 | 6 | dB | 0 | 2 |  |
| 0 21 006 | Circular depolarization ratio | dB | 0 | -65 | 6 | dB | 0 | 2 |  |
| 0 21 007 | Radar reflectivity factor | dB | 2 | -9000 | 15 | dB | 0 | 0 |  |
| 0 21 008 | Uncertainty in radar reflectivity factor | dB | 2 | 0 | 13 | dB | 0 | 0 |  |
| 0 21 009 | Vertical Doppler velocity | m/s | 2 | -10000 | 15 | m/s | 0 | 0 |  |
| 0 21 010 | Uncertainty in vertical Doppler velocity | m/s | 2 | -10000 | 15 | m/s | 0 | 0 |  |
| 0 21 011 | Doppler mean velocity in x-direction | m/s | 0 | -128 | 8 | m/s | 0 | 3 |  |
| 0 21 012 | Doppler mean velocity in y-direction | m/s | 0 | -128 | 8 | m/s | 0 | 3 |  |
| 0 21 013 | Doppler mean velocity in z-direction | m/s | 0 | -128 | 8 | m/s | 0 | 3 |  |
| 0 21 014 | Doppler mean velocity (radial) | m/s | 1 | -4096 | 13 | m/s | 1 | 4 |  |
| 0 21 017 | Doppler velocity spectral width | m/s | 1 | 0 | 8 | m/s | 1 | 3 |  |
| 0 21 018 | Extended NYQUIST velocity | m/s | 1 | 0 | 10 | m/s | 1 | 4 |  |
| 0 21 019 | High NYQUIST velocity | m/s | 1 | 0 | 10 | m/s | 1 | 3 |  |
| 0 21 020 | Dynamic atmospheric correction | Numeric | 3 | -65536 | 17 |  |  |  |  |
| 0 21 021 | Echo tops | m | -3 | 0 | 4 | m | -3 | 2 |  |
| 0 21 022 | Range bin offset | m | 1 | 0 | 14 | m | 1 | 5 |  |
| 0 21 023 | Range bin size | m | 0 | 0 | 14 | m | 0 | 5 |  |
| 0 21 024 | Azimuth offset | deg | 1 | 0 | 12 | deg | 1 | 4 |  |
| 0 21 025 | Azimuthal resolution | deg | 1 | 0 | 8 | deg | 1 | 3 |  |
| 0 21 026 | ASCAT sigma-0 usability | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 21 027 | Land contribution ratio | Numeric | 4 | 0 | 14 | Numeric | 4 | 5 |  |
| 0 21 028 | Differential phase | deg | 1 | 0 | 12 | deg | 1 | 4 |  |
| 0 21 029 | Cross-polarization correlation coefficient | Numeric | 2 | -100 | 8 | Numeric | 2 | 3 |  |
| 0 21 030 | Signal to noise ratio | dB | 0 | -32 | 8 | dB | 0 | 3 |  |
| 0 21 031 | Vertically integrated liquid-water content | kg m-2 | 0 | 0 | 7 | kg m-2 | 0 | 3 |  |
| 0 21 036 | Radar rainfall intensity | m/s | 7 | 0 | 12 | m/s | 7 | 4 |  |
| 0 21 041 | Bright-band height | m | -2 | 0 | 8 | m | -2 | 3 |  |
| 0 21 051 | Signal power above 1 mW | dB | 0 | -256 | 8 | dB | 0 | 3 |  |
| 0 21 062 | Backscatter | dB | 2 | -5000 | 13 | dB | 2 | 4 |  |
| 0 21 063 | Radiometric resolution (noise value) | % | 1 | 0 | 10 | % | 1 | 4 |  |
| 0 21 064 | Clutter noise estimate | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 |  |
| 0 21 065 | Missing packet counter | Numeric | 0 | -127 | 8 | Numeric | 0 | 3 |  |
| 0 21 066 | Wave scatterometer product confidence data | Flag table | 0 | 0 | 12 | Flag table | 0 | 4 |  |
| 0 21 067 | Wind product confidence data | Flag table | 0 | 0 | 13 | Flag table | 0 | 5 |  |
| 0 21 068 | Radar altimeter product confidence data | Flag table | 0 | 0 | 8 | Flag table | 0 | 3 |  |
| 0 21 069 | SST product confidence data | Flag table | 0 | 0 | 10 | Flag table | 0 | 4 |  |
| 0 21 070 | SST product confidence data (SADIST-2) | Flag table | 0 | 0 | 23 | Flag table | 0 | 6 |  |
| 0 21 071 | Peakiness | Numeric | 0 | 0 | 16 | Numeric | 0 | 5 |  |
| 0 21 072 | Satellite altimeter calibration status | Flag table | 0 | 0 | 4 | Flag table | 0 | 2 |  |
| 0 21 073 | Satellite altimeter instrument mode | Flag table | 0 | 0 | 9 | Flag table | 0 | 3 |  |
| 0 21 075 | Image spectrum intensity | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 |  |
| 0 21 076 | Representation of intensities | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 21 077 | Altitude correction (ionosphere) | m | 3 | 0 | 14 | m | 3 | 5 |  |
| 0 21 078 | Altitude correction (dry troposphere) | m | 3 | 0 | 9 | m | 3 | 3 |  |
| 0 21 079 | Altitude correction (wet troposphere) | m | 3 | 2000 | 10 | m | 3 | 4 |  |
| 0 21 080 | Altitude correction (calibration constant) | m | 3 | 0 | 11 | m | 3 | 4 |  |
| 0 21 081 | Open loop correction (height-time loop) | m | 3 | 0 | 10 | m | 3 | 4 |  |
| 0 21 082 | Open loop correction (auto gain control) | dB | 3 | -3000 | 14 | dB | 3 | 5 |  |
| 0 21 083 | Warm target calibration | Numeric | 0 | 0 | 16 | Numeric | 0 | 5 |  |
| 0 21 084 | Cold target calibration | Numeric | 0 | 0 | 16 | Numeric | 0 | 5 |  |
| 0 21 085 | ATSR sea-surface temperature across-track band number | Numeric | 0 | 0 | 4 | Numeric | 0 | 2 |  |
| 0 21 086 | Number of pixels in nadir only, average | Numeric | 0 | 0 | 9 | Numeric | 0 | 3 |  |
| 0 21 087 | Number of pixels in dual view, average | Numeric | 0 | 0 | 9 | Numeric | 0 | 3 |  |
| 0 21 088 | Wet backscatter | dB | 2 | -5000 | 13 | dB | 2 | 4 |  |
| 0 21 091 | Radar signal Doppler spectrum 0th moment | dB | 0 | -100 | 8 | dB | 0 | 3 |  |
| 0 21 092 | RASS signal Doppler spectrum 0th moment, referring to RASS signal | dB | 0 | -100 | 8 | dB | 0 | 3 |  |
| 0 21 093 | Ku band peakiness | Numeric | 3 | 0 | 16 | Numeric | 3 | 5 |  |
| 0 21 094 | S band peakiness | Numeric | 3 | 0 | 16 | Numeric | 3 | 5 |  |
| 0 21 095 | Kp coefficient A | Numeric | 6 | 0 | 20 | Numeric | 6 | 7 |  |
| 0 21 096 | Kp coefficient B | Numeric | 6 | 0 | 20 | Numeric | 6 | 7 |  |
| 0 21 097 | Kp coefficient C | Numeric | 6 | 0 | 20 | Numeric | 6 | 7 |  |
| 0 21 101 | Number of vector ambiguities | Numeric | 0 | 0 | 3 | Numeric | 0 | 1 |  |
| 0 21 102 | Index of selected wind vector | Numeric | 0 | 0 | 3 | Numeric | 0 | 1 |  |
| 0 21 103 | Total number of sigma-0 measurements | Numeric | 0 | 0 | 5 | Numeric | 0 | 2 |  |
| 0 21 104 | Likelihood computed for solution | Numeric | 3 | -30000 | 15 | Numeric | 3 | 5 |  |
| 0 21 105 | Normalized radar cross-section | dB | 2 | -10000 | 14 | dB | 2 | 5 |  |
| 0 21 106 | Kp variance coefficient (alpha) | Numeric | 3 | 0 | 14 | Numeric | 3 | 5 |  |
| 0 21 107 | Kp variance coefficient (beta) | Numeric | 8 | 0 | 16 | Numeric | 8 | 5 |  |
| 0 21 109 | SEAWINDS wind vector cell quality | Flag table | 0 | 0 | 17 | Flag table | 0 | 6 |  |
| 0 21 110 | Number of inner-beam sigma-0 (forward of satellite) | Numeric | 0 | 0 | 6 | Numeric | 0 | 2 |  |
| 0 21 111 | Number of outer-beam sigma-0 (forward of satellite) | Numeric | 0 | 0 | 6 | Numeric | 0 | 2 |  |
| 0 21 112 | Number of inner-beam sigma-0 (aft of satellite) | Numeric | 0 | 0 | 6 | Numeric | 0 | 2 |  |
| 0 21 113 | Number of outer-beam sigma-0 (aft of satellite) | Numeric | 0 | 0 | 6 | Numeric | 0 | 2 |  |
| 0 21 114 | Kp variance coefficient (gamma) | dB | 3 | -140000 | 18 | dB | 3 | 6 |  |
| 0 21 115 | SEAWINDS sigma-0 quality | Flag table | 0 | 0 | 17 | Flag table | 0 | 6 |  |
| 0 21 116 | SEAWINDS sigma-0 mode | Flag table | 0 | 0 | 17 | Flag table | 0 | 6 |  |
| 0 21 117 | Sigma-0 variance quality control | Numeric | 2 | 0 | 16 | Numeric | 2 | 5 |  |
| 0 21 118 | Attenuation correction on sigma-0 | dB | 2 | -10000 | 14 | dB | 2 | 5 |  |
| 0 21 119 | Wind scatterometer geophysical model function | Code table | 0 | 0 | 6 | Code table | 0 | 2 |  |
| 0 21 120 | Probability of rain | Numeric | 3 | 0 | 10 | Numeric | 3 | 4 |  |
| 0 21 121 | SEAWINDS NOF rain index | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 |  |
| 0 21 122 | Attenuation correction on sigma-0 (from tB) | dB | 2 | -10000 | 14 | dB | 2 | 5 |  |
| 0 21 123 | SEAWINDS normalized radar cross-section | dB | 2 | -30000 | 15 | dB | 2 | 5 |  |
| 0 21 128 | Number of valid points per second used to derive previous parameters | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 |  |
| 0 21 130 | Spectrum total energy | Numeric | 6 | 0 | 28 | Numeric | 6 | 9 |  |
| 0 21 131 | Spectrum max energy | Numeric | 6 | 0 | 28 | Numeric | 6 | 9 |  |
| 0 21 132 | Direction of spectrum max on higher resolution grid | deg | 3 | 0 | 19 | deg | 3 | 6 |  |
| 0 21 133 | Wavelength of spectrum max on higher resolution grid | m | 3 | 0 | 29 | m | 3 | 9 |  |
| 0 21 134 | Range resolution of cress covariance spectrum | rad/m | 3 | 0 | 19 | rad/m | 3 | 6 |  |
| 0 21 135 | Real part of cross spectra polar grid number of bins | Numeric | 3 | -524288 | 20 | Numeric | 3 | 7 |  |
| 0 21 136 | Imaginary part of cross spectra polar grid number of bins | Numeric | 3 | -524288 | 20 | Numeric | 3 | 7 |  |
| 0 21 137 | Ku band corrected ocean backscatter coefficient | dB | 2 | -32768 | 16 | dB | 2 | 5 |  |
| 0 21 138 | Std Ku band corrected ocean backscatter coefficient | dB | 2 | -32768 | 16 | dB | 2 | 5 |  |
| 0 21 139 | Ku band net instrumental correction for AGC | dB | 2 | -2048 | 12 | dB | 2 | 4 |  |
| 0 21 140 | S band corrected ocean backscatter coefficient | dB | 2 | -32768 | 16 | dB | 2 | 5 |  |
| 0 21 141 | Std S band corrected ocean backscatter coefficient | dB | 2 | -32768 | 16 | dB | 2 | 5 |  |
| 0 21 142 | S band net instrumental correction for AGC | dB | 2 | -1024 | 11 | dB | 2 | 4 |  |
| 0 21 143 | Ku band rain attenuation | dB | 2 | -1073741824 | 31 | dB | 2 | 10 |  |
| 0 21 144 | Altimeter rain flag | Flag table | 0 | 0 | 2 | Flag table | 0 | 1 |  |
| 0 21 145 | Ku band automatic gain control | dB | 2 | 0 | 13 | dB | 2 | 4 |  |
| 0 21 146 | RMS Ku band automatic gain control | dB | 2 | 0 | 8 | dB | 2 | 3 |  |
| 0 21 147 | Number of valid points for Ku band automatic gain control | Numeric | 0 | 0 | 5 | Numeric | 0 | 2 |  |
| 0 21 148 | Trailing edge variation flag | Flag table | 0 | 0 | 9 | Flag table | 0 | 3 |  |
| 0 21 150 | Beam co-location | Code table | 0 | 0 | 2 | Code table | 0 | 1 |  |
| 0 21 151 | Estimated error in sigma-0 at 40 degrees incidence angle | dB | 2 | 0 | 9 | dB | 2 | 3 |  |
| 0 21 152 | Slope at 40 degrees incidence angle | dB/deg | 2 | -80 | 7 | dB/deg | 2 | 2 |  |
| 0 21 153 | Estimated error in slope at 40 degrees incidence angle | dB/deg | 2 | -40 | 6 | dB/deg | 2 | 2 |  |
| 0 21 154 | Soil moisture sensitivity | dB | 2 | 0 | 12 | dB | 2 | 4 |  |
| 0 21 155 | Wind vector cell quality | Flag table | 0 | 0 | 24 | Flag table | 0 | 8 |  |
| 0 21 156 | Backscatter distance | Numeric | 1 | -4096 | 13 | Numeric | 1 | 4 |  |
| 0 21 157 | Loss per unit length of atmosphere used | dB/m | 10 | 0 | 22 | dB/m | 10 | 7 |  |
| 0 21 158 | ASCAT Kp estimate quality | Code table | 0 | 0 | 2 | Code table | 0 | 1 |  |
| 0 21 159 | ASCAT sigma-0 usability | Code table | 0 | 0 | 2 | Code table | 0 | 1 |  |
| 0 21 160 | ASCAT use of synthetic data | Numeric | 3 | 0 | 10 | Numeric | 3 | 4 |  |
| 0 21 161 | ASCAT synthetic data quantity | Numeric | 3 | 0 | 10 | Numeric | 3 | 4 |  |
| 0 21 162 | ASCAT satellite orbit and attitude quality | Numeric | 3 | 0 | 10 | Numeric | 3 | 4 |  |
| 0 21 163 | ASCAT solar array reflection contamination | Numeric | 3 | 0 | 10 | Numeric | 3 | 4 |  |
| 0 21 164 | ASCAT telemetry presence and quality | Numeric | 3 | 0 | 10 | Numeric | 3 | 4 |  |
| 0 21 165 | ASCAT extrapolated reference function presence | Numeric | 3 | 0 | 10 | Numeric | 3 | 4 |  |
| 0 21 166 | Land fraction | Numeric | 3 | 0 | 10 | Numeric | 3 | 4 |  |
| 0 21 167 | Subsurface scattering probability | % | 0 | 0 | 7 | % | 0 | 3 |  |
| 0 21 169 | Ice presence indicator | Code table | 0 | 0 | 2 | Code table | 0 | 1 |  |
| 0 21 170 | C band corrected ocean backscatter coefficient | dB | 2 | -32768 | 16 | dB | 2 | 5 |  |
| 0 21 171 | RMS C band corrected ocean backscatter coefficient | dB | 2 | -32768 | 16 | dB | 2 | 5 |  |
| 0 21 172 | C band net instrumental correction for AGC | dB | 2 | -2048 | 12 | dB | 2 | 4 |  |
| 0 21 173 | C band automatic gain control | dB | 2 | 0 | 13 | dB | 2 | 4 |  |
| 0 21 174 | RMS C band automatic gain control | dB | 2 | 0 | 9 | dB | 2 | 3 |  |
| 0 21 175 | Number of valid points for C band automatic gain control | Numeric | 0 | 0 | 10 | Numeric | 0 | 4 |  |
| 0 21 176 | High frequency variability correction | m | 3 | 0 | 16 | m | 3 | 5 |  |
| 0 21 177 | Corrected OCOG backscatter coefficient | dB | 2 | 0 | 16 | dB | 2 | 5 |  |
| 0 21 178 | STD of 20 Hz OCOG backscatter coefficient | dB | 2 | 0 | 16 | dB | 2 | 5 |  |
| 0 21 179 | Number of 20 Hz valid points for OCOG backscatter coefficient | Numeric | 0 | 0 | 16 | Numeric | 0 | 5 |  |
| 0 21 180 | Number of 20 Hz valid points for ocean backscatter coefficient | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 |  |
| 0 21 181 | 20 Hz ocean backscatter coefficient | dB | 2 | 0 | 16 | dB | 2 | 5 |  |
| 0 21 182 | 20 Hz Ku band peakiness | Numeric | 3 | 0 | 16 | Numeric | 3 | 5 |  |
| 0 21 183 | Specific band corrected ocean backscatter coefficient | dB | 2 | -32768 | 16 | dB | 2 | 5 |  |
| 0 21 184 | STD specific band corrected ocean backscatter coefficient | dB | 2 | -32768 | 16 | dB | 2 | 5 |  |
| 0 21 185 | Specific band net instrumental correction for AGC | dB | 2 | -2048 | 12 | dB | 2 | 4 |  |
| 0 21 186 | Specific band automatic gain control | dB | 2 | 0 | 13 | dB | 2 | 4 |  |
| 0 21 187 | RMS specific band automatic gain control | dB | 2 | 0 | 8 | dB | 2 | 3 |  |
| 0 21 188 | Number of valid points for specific band automatic gain control | Numeric | 0 | 0 | 7 | Numeric | 0 | 3 |  |
| 0 21 189 | Corrected OCOG backscatter coefficient (negative reference) | dB | 2 | -32768 | 16 | dB | 2 | 6 | Note 11: Descriptor 3 01 002 should not be used. |