Category 07: Surface report sequences (land)

3 07 001 (Low altitude station)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 031 | Identification and type of station, date/time, location (high accuracy), height of station |  |  |
| 3 02 011 | Low altitude station | Basic surface report |  |

3 07 002 (Low altitude station)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 032 | Identification and type of station, date/time, location (coarse accuracy), height of station |  |  |
| 3 02 011 | Low altitude station | Basic surface report |  |

3 07 003 (Low altitude station)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 07 001 | Low altitude station | Location (high accuracy) and basic report |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 02 005 | Cloud layer |  |  |

3 07 004 (Low altitude station)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 07 002 | Low altitude station | Location (coarse accuracy) and basic report |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 02 005 | Cloud layer |  |  |

3 07 005 (Low altitude station)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 07 001 | Low altitude station | Location (high accuracy) and basic report |  |
| 1 01 004 | Replicate 1 descriptor 4 times |  |  |
| 3 02 005 | Cloud layer | 4 layers |  |

3 07 006 (Low altitude station)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 07 002 | Low altitude station | Location (coarse accuracy) and basic report |  |
| 1 01 004 | Replicate 1 descriptor 4 times |  |  |
| 3 02 005 | Cloud layer | 4 layers |  |

3 07 007 (High altitude station)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 031 | Identification and type of station, date/time, location (high accuracy), height of station |  |  |
| 3 02 012 | High altitude station | Basic surface report |  |

3 07 008 (High altitude station)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 032 | Identification and type of station, date/time, location (coarse accuracy), height of station |  |  |
| 3 02 012 | High altitude station | Basic surface report |  |

3 07 009

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 031 | Identification and type of station, date/time, location (high accuracy), height of station |  |  |
| 3 02 013 | Basic surface report |  |  |

3 07 011 (Main part of data for representation of METAR/SPECI code in BUFR)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 01 063 | ICAO location indicator |  |  |
| 0 02 001 | Type of station |  |  |
| 3 01 011 | Year, month, day | YY |  |
| 3 01 012 | Hour, minute | GG, gg |  |
| 3 01 024 | Latitude/longitude (coarse accuracy), height of station |  |  |
| 0 07 006 | Height above station | Height of an anemometer |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 016 | Extreme counterclockwise wind direction of a variable wind |  |  |
| 0 11 017 | Extreme clockwise wind direction of a variable wind |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 11 041 | Maximum wind gust speed |  |  |
| 0 07 006 | Height above station | Height of a thermometer |  |
| 0 12 001 | Temperature/air temperature |  |  |
| 0 12 003 | Dewpoint temperature |  |  |
| 0 10 052 | Altimeter setting (QNH) |  |  |
| 0 20 009 | General weather indicator (TAF/METAR) |  |  |

3 07 012 (Horizontal visibility)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 1 03 000 | Delayed replication of 3 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor | Up to 3 |  |
| 0 08 023 | First-order statistics |  |  |
| 0 05 021 | Bearing or azimuth | Direction of visibility observed |  |
| 0 20 001 | Horizontal visibility |  |  |

3 07 013 (Runway visual range)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 1 06 000 | Delayed replication of 6 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor | Up to 4 |  |
| 0 01 064 | Runway designator |  |  |
| 0 08 014 | Qualifier for runway visual range |  |  |
| 0 20 061 | Runway visual range (RVR) |  |  |
| 0 08 014 | Qualifier for runway visual range |  |  |
| 0 20 061 | Runway visual range (RVR) |  |  |
| 0 20 018 | Tendency of runway visual range |  |  |

3 07 014 (Significant present or forecast weather)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor | Up to 3 |  |
| 0 20 019 | Significant present or forecast weather |  |  |

3 07 015 (Clouds group(s))

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 02 005 | Cloud layer | NsNsNs, CC, hshshs |  |
| 0 20 002 | Vertical visibility |  |  |

3 07 016 (Significant recent weather phenomena)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor | Up to 3 |  |
| 0 20 020 | Significant recent weather phenomena |  |  |

3 07 017 (Wind shear on runway(s))

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 11 070 | Designator of the runway affected by wind shear (including ALL) |  |  |

3 07 018 (Trend-type landing forecast)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 08 016 | Change qualifier of a trend-type forecast or an aerodrome forecast |  |  |
| 1 02 000 | Delayed replication of 2 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor | Up to 2 |  |
| 0 08 017 | Qualifier of the time when the forecast change is expected | FM, TL, AT |  |
| 3 01 012 | Hour, minute | GG, gg |  |
| 1 04 000 | Delayed replication of 4 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor | Up to 1 |  |
| 0 07 006 | Height above station |  |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 11 041 | Maximum wind gust speed |  |  |
| 0 20 009 | General weather indicator (TAF/METAR) |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor | Up to 1 |  |
| 0 20 001 | Horizontal visibility |  |  |
| 3 07 014 | Significant present or forecast weather | w´w´ |  |

3 07 020 (Short METAR/SPECI)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 07 011 | Main part of data for representation of METAR/SPECI code in BUFR |  |  |
| 3 07 014 | Significant present or forecast weather | w´w´ |  |
| 3 07 016 | Significant recent weather phenomena | REw´w´ |  |

3 07 021 (Total sequence for representation of METAR/SPECI code in BUFR)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 07 011 | Main part of data for representation of METAR/SPECI code in BUFR |  |  |
| 3 07 012 | Horizontal visibility | DvVVVV |  |
| 3 07 013 | Runway visual range | DRDR/VRVRVRVR |  |
| 3 07 014 | Significant present or forecast weather | w´w´ |  |
| 3 07 015 | Clouds group(s) |  |  |
| 3 07 016 | Significant recent weather phenomena | REw´w´ |  |
| 3 07 017 | Wind shear on runway(s) |  |  |
| 3 07 018 | Trend-type landing forecast |  |  |
| 3 07 015 | Clouds group(s) |  |  |

3 07 022 (Ground-based GNSS data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 01 015 | Station or site name |  |  |
| 3 01 011 | Year, month, day |  |  |
| 3 01 012 | Hour, minute |  |  |
| 3 01 022 | Latitude/longitude (high accuracy), height of station |  |  |
| 0 08 021 | Time significance | = 23 Monitoring period |  |
| 0 04 025 | Time period or displacement |  |  |
| 0 10 004 | Pressure |  |  |
| 0 12 001 | Temperature/air temperature |  |  |
| 0 13 003 | Relative humidity |  |  |
| 0 33 038 | Quality flags for ground-based GNSS data |  |  |
| 0 08 022 | Total number (with respect to accumulation or average) | Number of GNSS satellites used |  |
| 1 06 025 | Replicate 6 descriptors 25 times |  |  |
| 0 02 020 | Satellite classification |  |  |
| 0 01 050 | Platform transmitter ID number |  |  |
| 0 05 021 | Bearing or azimuth |  |  |
| 0 07 021 | Elevation |  |  |
| 0 15 031 | Atmospheric path delay in satellite signal |  |  |
| 0 15 032 | Estimated error in atmospheric path delay |  |  |
| 0 08 060 | Sample scanning mode significance | = 5 North/South |  |
| 0 15 033 | Difference in path delays for limb views at extremes of scan |  |  |
| 0 15 034 | Estimated error in path delay difference |  |  |
| 0 08 060 | Sample scanning mode significance | = 6 East/West |  |
| 0 15 033 | Difference in path delays for limb views at extremes of scan |  |  |
| 0 15 034 | Estimated error in path delay difference |  |  |
| 0 15 035 | Component of zenith path delay due to water vapour |  |  |
| 2 01 131 | Change data width |  |  |
| 2 02 129 | Change scale |  |  |
| 0 13 016 | Precipitable water |  |  |
| 2 02 000 | Change scale | Cancel |  |
| 2 01 000 | Change data width | Cancel |  |
| 0 15 011 | Log10 of integrated electron density |  |  |

3 07 024 (Ground-based GNSS data - slant total delay)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 150 | WIGOS identifier |  |  |
| 0 01 015 | Station or site name |  |  |
| 0 01 040 | Processing centre ID code | GNSS processing centre |  |
| 0 08 021 | Time significance | = 30 Time of occurrence |  |
| 3 01 011 | Year, month, day |  |  |
| 3 01 013 | Hour, minute, second |  |  |
| 3 01 022 | Latitude/longitude (high accuracy), height of station |  |  |
| 0 10 036 | Geoid undulation | Geoid height above WGS-84 ellipsoid, ±150 m to 1 cm |  |
| 0 25 061 | Software identification and version number | GNSS processing software, software version |  |
| 0 10 004 | Pressure |  |  |
| 0 12 001 | Temperature/air temperature |  |  |
| 0 13 003 | Relative humidity |  |  |
| 1 20 000 | Delayed replication of 20 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 25 060 | Software identification |  |  |
| 0 08 021 | Time significance | = 2 Time averaged |  |
| 0 04 025 | Time period or displacement |  |  |
| 1 15 000 | Delayed replication of 15 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 15 079 | Zenith path delay due to neutral atmosphere |  |  |
| 0 15 080 | Estimated error in neutral atmosphere zenith path delay |  |  |
| 0 08 022 | Total number (with respect to accumulation or average) |  |  |
| 0 33 093 | Extended quality flags for ground-based GNSS data |  |  |
| 0 15 089 | Zenith path delay due to neutral hydrostatic atmosphere |  |  |
| 0 15 035 | Component of zenith path delay due to water vapour |  |  |
| 1 02 002 | Replicate 2 descriptors 2 times |  |  |
| 0 08 060 | Sample scanning mode significance | = 5 north/south or =6 east/west |  |
| 0 15 083 | GNSS derived neutral atmosphere gradient |  |  |
| 2 01 131 | Change data width |  |  |
| 2 02 129 | Change scale |  |  |
| 0 13 016 | Precipitable water |  |  |
| 2 02 000 | Change scale | Cancel |  |
| 2 01 000 | Change data width | Cancel |  |
| 0 15 011 | Log10 of integrated electron density |  |  |
| 1 31 000 | Delayed replication of 31 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 25 060 | Software identification |  |  |
| 0 08 021 | Time significance | Set to missing |  |
| 0 04 025 | Time period or displacement | = 0 |  |
| 0 33 093 | Extended quality flags for ground-based GNSS data |  |  |
| 1 25 000 | Delayed replication of 25 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor | Number of GNSS satellites used |  |
| 0 02 020 | Satellite classification |  |  |
| 0 01 050 | Platform transmitter ID number |  |  |
| 0 01 150 | Coordinate reference system | = 5 ECF coordinate system |  |
| 2 02 127 | Change scale |  |  |
| 3 04 030 | Location of platform |  |  |
| 2 02 000 | Change scale |  |  |
| 0 05 021 | Bearing or azimuth |  |  |
| 0 07 021 | Elevation |  |  |
| 0 15 038 | Path delay due to neutral atmosphere |  |  |
| 0 15 039 | Estimated error in neutral atmosphere path delay |  |  |
| 0 15 090 | Path delay due to neutral hydrostatic atmosphere |  |  |
| 0 15 081 | Wet path delay due to neutral atmosphere |  |  |
| 0 15 082 | Path integrated water vapour |  |  |
| 0 15 079 | Zenith path delay due to neutral atmosphere |  |  |
| 0 15 089 | Zenith path delay due to neutral hydrostatic atmosphere |  |  |
| 0 15 035 | Component of zenith path delay due to water vapour |  |  |
| 1 02 002 | Replicate 2 descriptors 2 times |  |  |
| 0 08 060 | Sample scanning mode significance | = 5 north/south or = 6 east/west |  |
| 0 15 083 | GNSS derived neutral atmosphere gradient |  |  |
| 0 15 084 | GNSS least squares residual |  |  |
| 0 15 085 | GNSS multi-path delay |  |  |
| 0 15 086 | GNSS hydrostatic mapping function |  |  |
| 0 15 087 | GNSS wet mapping function |  |  |
| 0 15 088 | GNSS gradient mapping function |  |  |
| 0 15 011 | Log10 of integrated electron density |  |  |

3 07 030 (Ozone data - single observation)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 15 001 | Total ozone |  |  |
| 0 15 002 | Air mass (slant path at 22 km) |  |  |

3 07 031 (Ozone data - averaged observations)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 08 022 | Total number (with respect to accumulation or average) | Number of measurements |  |
| 0 08 023 | First-order statistics | = 4 Mean value |  |
| 0 15 001 | Total ozone | Average value of ozone measurement |  |
| 0 08 023 | First-order statistics | = 9 Best estimate of standard deviation |  |
| 0 15 001 | Total ozone | Best estimate of standard deviation of the ozone measurement |  |
| 0 08 023 | First-order statistics | = 11 Harmonic mean |  |
| 0 15 002 | Air mass (slant path at 22 km) | Harmonic mean value of the air-mass |  |

3 07 041 (Total ozone measurement from a Brewer ground-based spectrophotometer obtained from a single observation)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 001 | WMO block and station numbers |  |  |
| 0 01 015 | Station or site name |  |  |
| 3 01 024 | Latitude/longitude (coarse accuracy), height of station |  |  |
| 3 01 011 | Year, month, day | Ozone measurement |  |
| 3 01 012 | Hour, minute | Ozone measurement |  |
| 3 01 070 | Ozone instrumentation - Brewer spectrophotometer |  |  |
| 3 07 030 | Ozone data - single observation |  |  |

3 07 042 (Total ozone measurement from a Brewer ground-based spectrophotometer obtained from averaged observations)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 001 | WMO block and station numbers |  |  |
| 0 01 015 | Station or site name |  |  |
| 3 01 024 | Latitude/longitude (coarse accuracy), height of station |  |  |
| 3 01 011 | Year, month, day | Ozone measurement |  |
| 3 01 012 | Hour, minute | Ozone measurement |  |
| 0 08 021 | Time significance | = 8 Ensemble mean |  |
| 0 04 025 | Time period or displacement | Time period (minutes) for the computation of the average |  |
| 3 01 070 | Ozone instrumentation - Brewer spectrophotometer |  |  |
| 3 07 031 | Ozone data - averaged observations |  |  |

3 07 043 (Total ozone measurement from a Dobson ground-based spectrophotometer obtained from a single observation)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 001 | WMO block and station numbers |  |  |
| 0 01 015 | Station or site name |  |  |
| 3 01 024 | Latitude/longitude (coarse accuracy), height of station |  |  |
| 3 01 011 | Year, month, day | Ozone measurement |  |
| 3 01 012 | Hour, minute | Ozone measurement |  |
| 3 01 074 | Ozone instrumentation - Dobson spectrophotometer |  |  |
| 3 07 030 | Ozone data - single observation |  |  |

3 07 044 (Total ozone measurement from a Dobson ground-based spectrophotometer obtained from averaged observations)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 001 | WMO block and station numbers |  |  |
| 0 01 015 | Station or site name |  |  |
| 3 01 024 | Latitude/longitude (coarse accuracy), height of station |  |  |
| 3 01 011 | Year, month, day | Ozone measurement |  |
| 3 01 012 | Hour, minute | Ozone measurement |  |
| 0 08 021 | Time significance | = 8 Ensemble mean |  |
| 0 04 025 | Time period or displacement | Time period (minutes) for the computation of the average |  |
| 3 01 074 | Ozone instrumentation - Dobson spectrophotometer |  |  |
| 3 07 031 | Ozone data - averaged observations |  |  |

3 07 045 (Main part of METAR/SPECI), replacing 3 07 011

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 01 063 | ICAO location indicator | CCCC |  |
| 0 08 079 | Product status | METAR SPECI COR |  |
| 0 02 001 | Type of station | AUTO |  |
| 3 01 011 | Year, month, day | YY |  |
| 3 01 012 | Hour, minute | GGgg |  |
| 3 01 023 | Latitude/longitude (coarse accuracy) |  |  |
| 0 07 030 | Height of station ground above mean sea level |  |  |
| 0 07 031 | Height of barometer above mean sea level |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | = 10 m (if the actual value is not available) |  |
| 0 11 001 | Wind direction | ddd |  |
| 0 11 016 | Extreme counterclockwise wind direction of a variable wind | dndndn |  |
| 0 11 017 | Extreme clockwise wind direction of a variable wind | dxdxdx |  |
| 0 08 054 | Qualifier for wind speed or wind gusts | P |  |
| 0 11 083 | Wind speed | ff - km/h | Note 76: Within 3 07 045, 3 07 048 and 3 07 053, wind speed shall be reported in the same units as in the original TAC data and: 0 11 083 shall be set to missing, if wind speed is reported in knots or m s-1 in TAC data, 0 11 084 shall be set to missing, if wind speed is reported in km h-1 or m s-1 in TAC data. |
| 0 11 084 | Wind speed | ff - kt | Note 76: Within 3 07 045, 3 07 048 and 3 07 053, wind speed shall be reported in the same units as in the original TAC data and: 0 11 083 shall be set to missing, if wind speed is reported in knots or m s-1 in TAC data, 0 11 084 shall be set to missing, if wind speed is reported in km h-1 or m s-1 in TAC data. |
| 0 11 002 | Wind speed | ff - m/s | Note 76: Within 3 07 045, 3 07 048 and 3 07 053, wind speed shall be reported in the same units as in the original TAC data and: 0 11 083 shall be set to missing, if wind speed is reported in knots or m s-1 in TAC data, 0 11 084 shall be set to missing, if wind speed is reported in km h-1 or m s-1 in TAC data. |
| 0 08 054 | Qualifier for wind speed or wind gusts | P |  |
| 0 11 085 | Maximum wind gust speed | fmfm - km/h | Note 75: Within 3 07 045, 3 07 048 and 3 07 053, maximum wind speed (gusts) shall be reported in the same units as in the original TAC data and: 0 11 085 shall be set to missing, if maximum wind speed is reported in knots or m s-1 in TAC data, 0 11 086 shall be set to missing, if maximum wind speed is reported in km h-1 or m s-1 in TAC data. |
| 0 11 086 | Maximum wind gust speed | fmfm - kt | Note 75: Within 3 07 045, 3 07 048 and 3 07 053, maximum wind speed (gusts) shall be reported in the same units as in the original TAC data and: 0 11 085 shall be set to missing, if maximum wind speed is reported in knots or m s-1 in TAC data, 0 11 086 shall be set to missing, if maximum wind speed is reported in km h-1 or m s-1 in TAC data. |
| 0 11 041 | Maximum wind gust speed | fmfm - m/s | Note 75: Within 3 07 045, 3 07 048 and 3 07 053, maximum wind speed (gusts) shall be reported in the same units as in the original TAC data and: 0 11 085 shall be set to missing, if maximum wind speed is reported in knots or m s-1 in TAC data, 0 11 086 shall be set to missing, if maximum wind speed is reported in km h-1 or m s-1 in TAC data. |
| 0 08 054 | Qualifier for wind speed or wind gusts | Set to missing (cancel) |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | = 2 m (if the actual value is not available) |  |
| 0 12 023 | Temperature | T'T' - Celsius |  |
| 0 12 024 | Dewpoint temperature | T'dT'd - Celsius |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 0 10 052 | Altimeter setting (QNH) | QPHPHPHPH |  |
| 0 20 009 | General weather indicator (TAF/METAR) | CAVOK |  |

3 07 046 (METAR/SPECI visibility)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 20 060 | Prevailing horizontal visibility | VVVV or VVVVNDV |  |
| 1 02 000 | Delayed replication of 2 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor | Up to 2 |  |
| 0 05 021 | Bearing or azimuth | Direction of minimum visibility observed Dv |  |
| 0 20 059 | Minimum horizontal visibility | VNVNVNVN |  |

3 07 047 (METAR/SPECI/TAF clouds), replacing 3 07 015

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 1 05 000 | Delayed replication of 5 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 08 002 | Vertical significance (surface observations) |  |  |
| 0 20 011 | Cloud amount | NsNsNs |  |
| 0 20 012 | Cloud type | CC |  |
| 0 20 013 | Height of base of cloud | hshshs - m |  |
| 0 20 092 | Height of base of cloud | hshshs - ft |  |
| 0 20 002 | Vertical visibility | VVhshshs - m |  |
| 0 20 091 | Vertical visibility | VVhshshs - ft |  |

3 07 048 (Trend type forecast), replacing 3 07 018

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 08 016 | Change qualifier of a trend-type forecast or an aerodrome forecast | TTTTT NOSIG |  |
| 1 02 000 | Delayed replication of 2 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor | = 0, 1 or 2 |  |
| 0 08 017 | Qualifier of the time when the forecast change is expected | TT |  |
| 3 01 012 | Hour, minute | GGgg |  |
| 1 12 000 | Delayed replication of 12 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor | = 0 or 1 |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | = 10 m (if the actual value is not available) |  |
| 0 11 001 | Wind direction | ddd |  |
| 0 08 054 | Qualifier for wind speed or wind gusts | P |  |
| 0 11 083 | Wind speed | ff - km/h | Note 76: Within 3 07 045, 3 07 048 and 3 07 053, wind speed shall be reported in the same units as in the original TAC data and: 0 11 083 shall be set to missing, if wind speed is reported in knots or m s-1 in TAC data, 0 11 084 shall be set to missing, if wind speed is reported in km h-1 or m s-1 in TAC data. |
| 0 11 084 | Wind speed | ff - kt | Note 76: Within 3 07 045, 3 07 048 and 3 07 053, wind speed shall be reported in the same units as in the original TAC data and: 0 11 083 shall be set to missing, if wind speed is reported in knots or m s-1 in TAC data, 0 11 084 shall be set to missing, if wind speed is reported in km h-1 or m s-1 in TAC data. |
| 0 11 002 | Wind speed | ff - m/s | Note 76: Within 3 07 045, 3 07 048 and 3 07 053, wind speed shall be reported in the same units as in the original TAC data and: 0 11 083 shall be set to missing, if wind speed is reported in knots or m s-1 in TAC data, 0 11 084 shall be set to missing, if wind speed is reported in km h-1 or m s-1 in TAC data. |
| 0 08 054 | Qualifier for wind speed or wind gusts | P |  |
| 0 11 085 | Maximum wind gust speed | fmfm - km/h | Note 75: Within 3 07 045, 3 07 048 and 3 07 053, maximum wind speed (gusts) shall be reported in the same units as in the original TAC data and: 0 11 085 shall be set to missing, if maximum wind speed is reported in knots or m s-1 in TAC data, 0 11 086 shall be set to missing, if maximum wind speed is reported in km h-1 or m s-1 in TAC data. |
| 0 11 086 | Maximum wind gust speed | fmfm - kt | Note 75: Within 3 07 045, 3 07 048 and 3 07 053, maximum wind speed (gusts) shall be reported in the same units as in the original TAC data and: 0 11 085 shall be set to missing, if maximum wind speed is reported in knots or m s-1 in TAC data, 0 11 086 shall be set to missing, if maximum wind speed is reported in km h-1 or m s-1 in TAC data. |
| 0 11 041 | Maximum wind gust speed | fmfm - m/s | Note 75: Within 3 07 045, 3 07 048 and 3 07 053, maximum wind speed (gusts) shall be reported in the same units as in the original TAC data and: 0 11 085 shall be set to missing, if maximum wind speed is reported in knots or m s-1 in TAC data, 0 11 086 shall be set to missing, if maximum wind speed is reported in km h-1 or m s-1 in TAC data. |
| 0 08 054 | Qualifier for wind speed or wind gusts | Set to missing (cancel) |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 0 20 009 | General weather indicator (TAF/METAR) | CAVOK NSW NSC |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor | = 0 or 1 |  |
| 0 20 060 | Prevailing horizontal visibility | VVVV |  |
| 3 07 014 | Significant present and forecast weather | Weather intensity and phenomena w´w´ |  |
| 3 07 047 | METAR/SPECI/TAF clouds, replacing 3 07 015 | NsNsNshshshs |  |

3 07 049 (Sea conditions)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 1 02 000 | Delayed replication of 2 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor | = 0 or 1 |  |
| 0 22 043 | Sea/water temperature | TsTs |  |
| 0 22 021 | Height of waves | S´ |  |

3 07 050 (Runway state)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor | = 0 or 1 |  |
| 0 20 085 | General condition of runway | SNOCLO |  |
| 1 02 000 | Delayed replication of 2 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 01 064 | Runway designator | DRDR |  |
| 0 20 085 | General condition of runway | CLRD// |  |
| 1 05 000 | Delayed replication of 5 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 01 064 | Runway designator | DRDR |  |
| 0 20 086 | Runway deposits | ER |  |
| 0 20 087 | Runway contamination | CR |  |
| 0 20 088 | Depth of runway deposits | eReR |  |
| 0 20 089 | Runway friction coefficient | BRBR |  |

3 07 051 (Full METAR/SPECI), replacing 3 07 021

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 07 045 | Main part of METAR/SPECI, replacing 3 07 011 |  |  |
| 3 07 046 | METAR/SPECI visibility | VVVV or VVVVNDV VNVNVNVNDV |  |
| 3 07 013 | Runway visual range | RDRDR/VRVRVRVR |  |
| 3 07 014 | Significant present and forecast weather | Weather intensity and phenomena w´w´ |  |
| 3 07 047 | METAR/SPECI/TAF clouds, replacing 3 07 015 | NsNsNshshshs |  |
| 3 07 016 | Significant recent weather phenomena | REw´w´ |  |
| 3 07 017 | Wind shear on runway(s) | WS RDRDR |  |
| 3 07 049 | Sea conditions | WTsTs/SS’ |  |
| 3 07 050 | Runway state | RDRDR/ERCReReRBRBR |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor | = 0 to 3 normally |  |
| 3 07 048 | Trend type forecast, replacing 3 07 018 |  |  |

3 07 052 (Aerodrome forecast identification and time interval)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 01 063 | ICAO location indicator | CCCC |  |
| 0 08 039 | Time significance (aviation forecast) | = 0 Issue time of forecast |  |
| 3 01 011 | Year, month, day | YY |  |
| 3 01 012 | Hour, minute | GGgg |  |
| 0 08 079 | Product status | COR CNL AMD NIL |  |
| 0 08 039 | Time significance (aviation forecast) | = 1 Time of commencement of period of the forecast |  |
| 3 01 011 | Year, month, day | Y1Y1 |  |
| 3 01 012 | Hour, minute | G1G1 |  |
| 0 08 039 | Time significance (aviation forecast) | = 2 Time of ending of period of the forecast |  |
| 3 01 011 | Year, month, day | Y2Y2 |  |
| 3 01 012 | Hour, minute | G2G2 |  |
| 3 01 023 | Latitude/longitude (coarse accuracy) |  |  |
| 0 07 030 | Height of station ground above mean sea level |  |  |
| 0 07 031 | Height of barometer above mean sea level |  |  |

3 07 053 (Forecast weather at an aerodrome)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | = 10 m (if the actual value is not available) |  |
| 0 11 001 | Wind direction | ddd |  |
| 0 08 054 | Qualifier for wind speed or wind gusts | P |  |
| 0 11 083 | Wind speed | ff - km/h | Note 76: Within 3 07 045, 3 07 048 and 3 07 053, wind speed shall be reported in the same units as in the original TAC data and: 0 11 083 shall be set to missing, if wind speed is reported in knots or m s-1 in TAC data, 0 11 084 shall be set to missing, if wind speed is reported in km h-1 or m s-1 in TAC data. |
| 0 11 084 | Wind speed | ff - kt | Note 76: Within 3 07 045, 3 07 048 and 3 07 053, wind speed shall be reported in the same units as in the original TAC data and: 0 11 083 shall be set to missing, if wind speed is reported in knots or m s-1 in TAC data, 0 11 084 shall be set to missing, if wind speed is reported in km h-1 or m s-1 in TAC data. |
| 0 11 002 | Wind speed | ff - m/s | Note 76: Within 3 07 045, 3 07 048 and 3 07 053, wind speed shall be reported in the same units as in the original TAC data and: 0 11 083 shall be set to missing, if wind speed is reported in knots or m s-1 in TAC data, 0 11 084 shall be set to missing, if wind speed is reported in km h-1 or m s-1 in TAC data. |
| 0 08 054 | Qualifier for wind speed or wind gusts | P |  |
| 0 11 085 | Maximum wind gust speed | fmfm - km/h | Note 75: Within 3 07 045, 3 07 048 and 3 07 053, maximum wind speed (gusts) shall be reported in the same units as in the original TAC data and: 0 11 085 shall be set to missing, if maximum wind speed is reported in knots or m s-1 in TAC data, 0 11 086 shall be set to missing, if maximum wind speed is reported in km h-1 or m s-1 in TAC data. |
| 0 11 086 | Maximum wind gust speed | fmfm - kt | Note 75: Within 3 07 045, 3 07 048 and 3 07 053, maximum wind speed (gusts) shall be reported in the same units as in the original TAC data and: 0 11 085 shall be set to missing, if maximum wind speed is reported in knots or m s-1 in TAC data, 0 11 086 shall be set to missing, if maximum wind speed is reported in km h-1 or m s-1 in TAC data. |
| 0 11 041 | Maximum wind gust speed | fmfm - m/s | Note 75: Within 3 07 045, 3 07 048 and 3 07 053, maximum wind speed (gusts) shall be reported in the same units as in the original TAC data and: 0 11 085 shall be set to missing, if maximum wind speed is reported in knots or m s-1 in TAC data, 0 11 086 shall be set to missing, if maximum wind speed is reported in km h-1 or m s-1 in TAC data. |
| 0 08 054 | Qualifier for wind speed or wind gusts | Set to missing (cancel) |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 0 20 009 | General weather indicator (TAF/METAR) | CAVOK NSW NSC |  |
| 0 20 060 | Prevailing horizontal visibility | VVVV |  |
| 3 07 014 | Significant present and forecast weather | w´w´ |  |
| 3 07 047 | METAR/SPECI/TAF clouds, replacing 3 07 015 | NsNsNshshshs |  |

3 07 054 (Forecast of extreme temperatures)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | = 2 m (if the actual value is not available) |  |
| 0 08 039 | Time significance (aviation forecast) | = 3 Forecast time of maximum temperature |  |
| 0 04 003 | Day |  |  |
| 0 04 004 | Hour | GFGF |  |
| 0 08 023 | First-order statistics | = 3 Minimum |  |
| 0 12 023 | Temperature | TFTF - Celsius |  |
| 0 08 039 | Time significance (aviation forecast) | = 4 Forecast time of minimum temperature |  |
| 0 04 003 | Day |  |  |
| 0 04 004 | Hour | GFGF |  |
| 0 08 023 | First-order statistics | = 2 Maximum |  |
| 0 12 023 | Temperature | TFTF - Celsius |  |
| 0 08 023 | First-order statistics | Set to missing (cancel) |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |

3 07 055 (Change indicator and forecast changes)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 33 045 | Probability of following event | C2C2 |  |
| 0 08 016 | Change qualifier of a trend-type forecast or an aerodrome forecast | TTTTTT |  |
| 0 08 039 | Time significance (aviation forecast) | = 5 Time of beginning of the forecast change |  |
| 0 04 003 | Day |  |  |
| 3 01 012 | Hour, minute | GGgg |  |
| 0 08 039 | Time significance (aviation forecast) | = 6 Time of ending of the forecast change |  |
| 0 04 003 | Day |  |  |
| 3 01 012 | Hour, minute | GeGe |  |
| 3 07 053 | Forecast weather at an aerodrome | During or after change |  |

3 07 056 (Aerodrome forecast - full TAF)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 07 052 | Aerodrome forecast identification and time interval |  |  |
| 3 07 053 | Forecast weather at an aerodrome |  |  |
| 3 07 054 | Forecast of extreme temperatures |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 07 055 | Change indicator and forecast changes |  |  |

3 07 060 (Soil temperature below land surface)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 061 | Depth below land surface |  |  |
| 0 12 030 | Soil temperature |  |  |

3 07 061 (Soil temperature data at number of depths not exceeding five - high accuracy position)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 031 | Identification and type of station, date/time, location (high accuracy), height of station |  |  |
| 1 01 005 | Replicate 1 descriptor 5 times |  |  |
| 3 07 060 | Soil temperature below land surface |  |  |

3 07 062 (Soil temperature data at number of depths not exceeding five - coarse accuracy position)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 032 | Identification and type of station, date/time, location (coarse accuracy), height of station |  |  |
| 1 01 005 | Replicate 1 descriptor 5 times |  |  |
| 3 07 060 | Soil temperature below land surface |  |  |

3 07 063 (Depth below land surface and soil temperature)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 061 | Depth below land surface |  |  |
| 0 12 130 | Soil temperature | Scale: 2 |  |

3 07 064 (Soil temperature profile including permafrost)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 04 025 | Time period or displacement |  |  |
| 1 06 000 | Delayed replication of 6 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 07 061 | Depth below land surface |  |  |
| 2 01 131 | Change data width |  |  |
| 2 02 129 | Change scale |  |  |
| 0 12 130 | Soil temperature |  |  |
| 2 01 000 | Change data width | Cancel |  |
| 2 02 000 | Change scale | Cancel |  |

3 07 071 (Monthly values of a land station)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 090 | Surface station identification; time, horizontal and vertical coordinates |  | Note 59: The time identification refers to the beginning of the one-month period. |
| 0 04 074 | Short time period or displacement | = UTC - LT | Note 59: The time identification refers to the beginning of the one-month period. |
| 0 04 023 | Time period or displacement | Number of days in the month |  |
| 0 08 023 | First-order statistics | = 4 Mean value |  |
| 0 10 004 | Pressure |  |  |
| 0 10 051 | Pressure reduced to mean sea level |  |  |
| 0 07 004 | Pressure | Standard level ¦ Set to missing for lowland stations |  |
| 0 10 009 | Geopotential height | Standard level ¦ Set to missing for lowland stations |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  | Note 25: If the height of the sensor was changed during the period specified, the value shall be that which existed for the greater part of the period. |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 02 051 | Indicator to specify observing method for extreme temperatures |  |  |
| 0 04 051 | Principal time of daily reading of maximum temperature |  |  |
| 0 12 118 | Maximum temperature at height specified, past 24 hours |  |  |
| 0 04 052 | Principal time of daily reading of minimum temperature |  |  |
| 0 12 119 | Minimum temperature at height specified, past 24 hours |  |  |
| 0 13 004 | Vapour pressure |  |  |
| 0 08 023 | First-order statistics | Set to missing |  |
| 0 12 151 | Standard deviation of daily mean temperature |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 1 02 005 | Replicate 2 descriptors 5 times |  |  |
| 0 08 050 | Qualifier for number of missing values in calculation of statistic | = 1 Pressure, = 2 Temperature, = 4 Vapour pressure, = 7 Maximum temperature, = 8 Minimum temperature |  |
| 0 08 020 | Total number of missing entities (with respect to accumulation or average) | Days |  |
| 0 14 032 | Total sunshine |  |  |
| 0 14 033 | Total sunshine |  |  |
| 0 08 050 | Qualifier for number of missing values in calculation of statistic | = 6 Sunshine duration |  |
| 0 08 020 | Total number of missing entities (with respect to accumulation or average) | Days |  |
| 1 02 018 | Replicate 2 descriptors 18 times |  |  |
| 0 08 052 | Condition for which number of days of occurrence follows |  |  |
| 0 08 022 | Total number (with respect to accumulation or average) | Days |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  | Note 25: If the height of the sensor was changed during the period specified, the value shall be that which existed for the greater part of the period. |
| 0 08 053 | Day of occurrence qualifier | = 0 On 1 day only, = 1 On 2 or more days |  |
| 0 04 003 | Day |  |  |
| 0 12 152 | Highest daily mean temperature |  |  |
| 0 08 053 | Day of occurrence qualifier | = 0 On 1 day only, = 1 On 2 or more days |  |
| 0 04 003 | Day |  |  |
| 0 12 153 | Lowest daily mean temperature |  |  |
| 0 08 053 | Day of occurrence qualifier | = 0 On 1 day only, = 1 On 2 or more days |  |
| 0 04 003 | Day |  |  |
| 0 08 023 | First-order statistics | = 2 Maximum value |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 08 053 | Day of occurrence qualifier | = 0 On 1 day only, = 1 On 2 or more days |  |
| 0 04 003 | Day |  |  |
| 0 08 023 | First-order statistics | = 3 Minimum value |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 08 023 | First-order statistics | Set to missing |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  | Note 25: If the height of the sensor was changed during the period specified, the value shall be that which existed for the greater part of the period. |
| 0 02 002 | Type of instrumentation for wind measurement |  |  |
| 0 08 053 | Day of occurrence qualifier | = 0 On 1 day only, = 1 On 2 or more days |  |
| 0 04 003 | Day |  |  |
| 0 11 046 | Maximum instantaneous wind speed |  |  |
| 0 08 053 | Day of occurrence qualifier | Set to missing (cancel) |  |
| 0 04 003 | Day | = 1 | Note 27: In case of precipitation measurements, the one-month period begins at 06 UTC on the first day of the month and ends at 06 UTC on the first day of the following month. |
| 0 04 004 | Hour | = 6 | Note 27: In case of precipitation measurements, the one-month period begins at 06 UTC on the first day of the month and ends at 06 UTC on the first day of the following month. |
| 0 04 023 | Time period or displacement | Number of days in the month | Note 27: In case of precipitation measurements, the one-month period begins at 06 UTC on the first day of the month and ends at 06 UTC on the first day of the following month. |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  | Note 25: If the height of the sensor was changed during the period specified, the value shall be that which existed for the greater part of the period. |
| 0 13 060 | Total accumulated precipitation |  |  |
| 0 13 051 | Frequency group, precipitation |  |  |
| 0 04 053 | Number of days with precipitation equal to or more than 1 mm |  |  |
| 0 08 050 | Qualifier for number of missing values in calculation of statistic | = 5 Precipitation |  |
| 0 08 020 | Total number of missing entities (with respect to accumulation or average) | Days |  |
| 1 02 006 | Replicate 2 descriptors 6 times |  |  |
| 0 08 052 | Condition for which number of days of occurrence follows |  |  |
| 0 08 022 | Total number (with respect to accumulation or average) | Days |  |
| 0 08 053 | Day of occurrence qualifier | = 0 On 1 day only, = 1 On 2 or more days |  |
| 0 04 003 | Day |  |  |
| 0 13 052 | Highest daily amount of precipitation |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |

3 07 072 (Monthly normals for a land station)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 04 001 | Year | Beginning of the reference period |  |
| 0 04 001 | Year | Ending of the reference period |  |
| 0 04 002 | Month |  |  |
| 0 04 003 | Day | = 1 | Note 59: The time identification refers to the beginning of the one-month period. |
| 0 04 004 | Hour | = 0 | Note 59: The time identification refers to the beginning of the one-month period. |
| 0 04 074 | Short time period or displacement | = UTC - LT | Note 59: The time identification refers to the beginning of the one-month period. |
| 0 04 022 | Time period or displacement | = 1 |  |
| 0 08 023 | First-order statistics | = 4 Mean value |  |
| 0 10 004 | Pressure |  |  |
| 0 10 051 | Pressure reduced to mean sea level |  |  |
| 0 07 004 | Pressure | Standard level |  |
| 0 10 009 | Geopotential height | Standard level |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  | Note 25: If the height of the sensor was changed during the period specified, the value shall be that which existed for the greater part of the period. |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 02 051 | Indicator to specify observing method for extreme temperatures | = 2 |  |
| 0 04 051 | Principal time of daily reading of maximum temperature |  |  |
| 0 12 118 | Maximum temperature at height specified, past 24 hours |  |  |
| 0 04 052 | Principal time of daily reading of minimum temperature |  |  |
| 0 12 119 | Minimum temperature at height specified, past 24 hours |  |  |
| 0 13 004 | Vapour pressure |  |  |
| 0 12 151 | Standard deviation of daily mean temperature |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 0 14 032 | Total sunshine |  |  |
| 0 08 023 | First-order statistics | Set to missing |  |
| 0 04 001 | Year | Beginning of the reference period |  |
| 0 04 001 | Year | Ending of the reference period |  |
| 0 04 002 | Month |  |  |
| 0 04 003 | Day | = 1 | Note 27: In case of precipitation measurements, the one-month period begins at 06 UTC on the first day of the month and ends at 06 UTC on the first day of the following month. |
| 0 04 004 | Hour | = 6 | Note 27: In case of precipitation measurements, the one-month period begins at 06 UTC on the first day of the month and ends at 06 UTC on the first day of the following month. |
| 0 04 022 | Time period or displacement | = 1 |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  | Note 25: If the height of the sensor was changed during the period specified, the value shall be that which existed for the greater part of the period. |
| 0 08 023 | First-order statistics | = 4 Mean value |  |
| 0 13 060 | Total accumulated precipitation |  |  |
| 0 04 053 | Number of days with precipitation equal to or more than 1 mm |  |  |
| 0 08 023 | First-order statistics | Set to missing |  |
| 1 02 008 | Replicate 2 descriptors 8 times |  |  |
| 0 08 050 | Qualifier for number of missing values in calculation of statistic | = 1 Pressure, = 2 Temperature, = 3 Extreme temperature, = 4 Vapour pressure, = 5 Precipitation, = 6 Sunshine duration, = 7 Maximum temperature, = 8 Minimum temperature | Note 51: The number of missing years within the reference period from the calculation of normal for mean extreme air temperature should be given, if available, for both the calculation of normal maximum temperature and for the calculation of normal minimum temperature in addition to the number of missing years for the extreme air temperatures reported under 0 08 020 preceded by 0 08 050 in which figure 3 is used. |
| 0 08 020 | Total number of missing entities (with respect to accumulation or average) | Years | Note 51: The number of missing years within the reference period from the calculation of normal for mean extreme air temperature should be given, if available, for both the calculation of normal maximum temperature and for the calculation of normal minimum temperature in addition to the number of missing years for the extreme air temperatures reported under 0 08 020 preceded by 0 08 050 in which figure 3 is used. |

3 07 073 (Representation of CLIMAT data of the actual month and for monthly normals)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 07 071 | Monthly values of a land station |  |  |
| 3 07 072 | Monthly normals for a land station |  |  |

3 07 074 (Supplemental daily temperature and precipitation values for monthly climate report)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 001 | WMO block and station numbers |  |  |
| 0 04 001 | Year |  |  |
| 0 04 002 | Month |  |  |
| 3 01 021 | Latitude/longitude (high accuracy) |  |  |
| 0 07 030 | Height of station ground above mean sea level |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 1 12 000 | Delayed replication of 12 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor | Set to the number of days in the particular month for which data are being reported |  |
| 0 04 003 | Day |  |  |
| 0 04 004 | Hour |  |  |
| 0 04 024 | Time period or displacement | Typically set to -24 to denote the time period beginning 24 hours prior to and ending at the specified time |  |
| 1 02 003 | Replicate 2 descriptors 3 times |  |  |
| 0 08 023 | First-order statistics | = 2 Daily maximum temperature, = 3 Daily minimum temperature, = 4 Daily mean temperature |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 08 023 | First-order statistics | Set to missing (cancel) |  |
| 0 04 004 | Hour |  |  |
| 0 04 024 | Time period or displacement |  |  |
| 0 13 060 | Total accumulated precipitation |  |  |
| 0 13 012 | Depth of fresh snow |  |  |
| 0 13 013 | Total snow depth |  |  |

3 07 075 (Supplemental daily temperature and precipitation values with the time of occurrence for monthly climate report (DAYCLI))

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 150 | WIGOS identifier |  |  |
| 3 01 001 | WMO block and station number |  |  |
| 3 01 021 | Latitude/longitude (high accuracy) |  |  |
| 0 07 030 | Height of station ground above mean sea level |  |  |
| 0 08 095 | Siting and measurement quality classification for temperature |  |  |
| 0 08 096 | Siting and measurement quality classification for precipitation |  |  |
| 0 08 094 | Method used to calculate the average daily temperature |  |  |
| 3 01 011 | Year, month, day |  |  |
| 0 04 023 | Time period or displacement | = 0 when beginning time of the period is on the same day as reference date, = -1 when beginning time of the period is on the day before reference date |  |
| 3 01 013 | Hour, minute, second | Beginning time of the period |  |
| 2 04 008 | Add associated field | 8 bits long |  |
| 0 31 021 | Associated field significance | Set as 5 for 8-bit indicator of quality control |  |
| 0 13 060 | Total accumulated precipitation |  |  |
| 2 04 000 | Add associated field | Cancel |  |
| 0 04 023 | Time period or displacement | = 0 when beginning time of the period is on the same day as reference date, = -1 when beginning time of the period is on the day before reference date |  |
| 3 01 013 | Hour, minute, second | Beginning time of the period |  |
| 2 04 008 | Add associated field | 8 bits long |  |
| 0 31 021 | Associated field significance | Set as 5 for 8-bit indicator of quality control |  |
| 0 13 012 | Depth of fresh snow |  |  |
| 2 04 000 | Add associated field | Cancel |  |
| 0 04 023 | Time period or displacement | = 0 when beginning time of the period is on the same day as reference date, = -1 when beginning time of the period is on the day before reference date |  |
| 3 01 013 | Hour, minute, second | Beginning time of the period |  |
| 2 04 008 | Add associated field | 8 bits long |  |
| 0 31 021 | Associated field significance | Set as 5 for 8-bit indicator of quality control |  |
| 0 13 013 | Total snow depth |  |  |
| 2 04 000 | Add associated field | Cancel |  |
| 0 07 032 | Height of sensor above local ground | For temperature measurement |  |
| 1 07 003 | Replicate 7 descriptors 3 times |  |  |
| 0 04 023 | Time period or displacement | = 0 when beginning time of the period is on the same day as reference date, = -1 when beginning time of the period is on the day before reference date |  |
| 3 01 013 | Hour, minute, second | Beginning time of the period |  |
| 0 08 023 | First-order statistics | 2 - maximum; 3 - minimum; 4 - mean |  |
| 2 04 008 | Add associated field | 8 bits long |  |
| 0 31 021 | Associated field significance | Set as 5 for 8-bit indicator of quality control |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 2 04 000 | Add associated field | Cancel |  |
| 0 08 023 | First-order statistics | Set as missing value |  |

3 07 076 (Monthly values from a land station in compliance with regional or national reporting practices)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 090 | Surface station identification; time, horizontal and vertical coordinates |  |  |
| 0 04 074 | Short time period or displacement |  |  |
| 0 04 023 | Time period or displacement | Number of days in the month |  |
| 0 08 023 | First-order statistics | = 4 Mean value |  |
| 0 10 004 | Pressure |  |  |
| 0 10 051 | Pressure reduced to mean sea level |  |  |
| 0 07 004 | Pressure | Standard level ¦ Set to missing for lowland stations |  |
| 0 10 009 | Geopotential height | Standard level ¦ Set to missing for lowland stations |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 02 051 | Indicator to specify observing method for extreme temperatures |  |  |
| 0 04 051 | Principal time of daily reading of maximum temperature |  |  |
| 0 12 118 | Maximum temperature at height specified, past 24 hours |  |  |
| 0 04 052 | Principal time of daily reading of minimum temperature |  |  |
| 0 12 119 | Minimum temperature at height specified, past 24 hours |  |  |
| 0 13 004 | Vapour pressure |  |  |
| 0 08 023 | First-order statistics | Set to missing (cancel) |  |
| 0 12 151 | Standard deviation of daily mean temperature |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 1 02 005 | Replicate 2 descriptors 5 times |  |  |
| 0 08 050 | Qualifier for number of missing values in calculation of statistic | = 1 Pressure, = 2 Temperature, = 4 Vapour pressure, = 7 Maximum temperature, = 8 Minimum temperature |  |
| 0 08 020 | Total number of missing entities (with respect to accumulation or average) | Days |  |
| 0 14 032 | Total sunshine |  |  |
| 0 14 033 | Total sunshine |  |  |
| 0 08 050 | Qualifier for number of missing values in calculation of statistic | = 6 Sunshine duration |  |
| 0 08 020 | Total number of missing entities (with respect to accumulation or average) | Days |  |
| 1 02 018 | Replicate 2 descriptors 18 times |  |  |
| 0 08 052 | Condition for which number of days of occurrence follows | Wind >= 10 m/s, wind >= 20 m/s, wind >= 30 m/s, maximum temperature < 273.15 K, maximum temperature >= 298.15 K, maximum temperature >= 303.15 K, maximum temperature >= 308.15 K, maximum temperature >= 313.15 K, minimum temperature < 273.15 K, sss > 0.00 m, sss > 0.01 m, sss > 0.10 m, sss > 0.50 m, horizontal visibility < 50 m, horizontal visibility < 100 m, horizontal visibility < 1 000 m, hail, thunderstorm |  |
| 0 08 022 | Total number (with respect to accumulation or average) | Days |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 08 053 | Day of occurrence qualifier | = 0 On 1 day only, = 1 On 2 or more days |  |
| 0 04 003 | Day |  |  |
| 0 12 152 | Highest daily mean temperature |  |  |
| 0 08 053 | Day of occurrence qualifier | = 0 On 1 day only, = 1 On 2 or more days |  |
| 0 04 003 | Day |  |  |
| 0 12 153 | Lowest daily mean temperature |  |  |
| 0 08 053 | Day of occurrence qualifier | = 0 On 1 day only, = 1 On 2 or more days |  |
| 0 04 003 | Day |  |  |
| 0 08 023 | First-order statistics | = 2 Maximum value |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 08 053 | Day of occurrence qualifier | = 0 On 1 day only, = 1 On 2 or more days |  |
| 0 04 003 | Day |  |  |
| 0 08 023 | First-order statistics | = 3 Minimum value |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 08 023 | First-order statistics | Set to missing (cancel) |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 02 002 | Type of instrumentation for wind measurement |  |  |
| 0 08 053 | Day of occurrence qualifier | = 0 On 1 day only, = 1 On 2 or more days |  |
| 0 04 003 | Day |  |  |
| 0 11 046 | Maximum instantaneous wind speed |  |  |
| 0 08 053 | Day of occurrence qualifier | Set to missing (cancel) |  |
| 0 04 003 | Day | = 1 |  |
| 0 04 004 | Hour | = 0 |  |
| 0 04 074 | Short time period or displacement |  |  |
| 0 04 023 | Time period or displacement | Number of days in the month |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 13 060 | Total accumulated precipitation |  |  |
| 0 13 051 | Frequency group, precipitation |  |  |
| 0 04 053 | Number of days with precipitation equal to or more than 1 mm |  |  |
| 0 08 050 | Qualifier for number of missing values in calculation of statistic | = 5 Precipitation |  |
| 0 08 020 | Total number of missing entities (with respect to accumulation or average) | Days |  |
| 1 02 006 | Replicate 2 descriptors 6 times |  |  |
| 0 08 052 | Condition for which number of days of occurrence follows |  |  |
| 0 08 022 | Total number (with respect to accumulation or average) | Days |  |
| 0 08 053 | Day of occurrence qualifier | = 0 On 1 day only, = 1 On 2 or more days |  |
| 0 04 003 | Day |  |  |
| 0 13 052 | Highest daily amount of precipitation |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |

3 07 077 (Monthly normals for a land station in compliance with regional or national reporting practices)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 04 001 | Year | Beginning of the reference period |  |
| 0 04 001 | Year | Ending of the reference period |  |
| 0 04 002 | Month |  |  |
| 0 04 003 | Day | = 1 |  |
| 0 04 004 | Hour | = 0 |  |
| 0 04 074 | Short time period or displacement |  |  |
| 0 04 022 | Time period or displacement | = 1 |  |
| 0 08 023 | First-order statistics | = 4 Mean value |  |
| 0 10 004 | Pressure |  |  |
| 0 10 051 | Pressure reduced to mean sea level |  |  |
| 0 07 004 | Pressure | Standard level |  |
| 0 10 009 | Geopotential height | Standard level |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 02 051 | Indicator to specify observing method for extreme temperatures |  |  |
| 0 04 051 | Principal time of daily reading of maximum temperature |  |  |
| 0 12 118 | Maximum temperature at height specified, past 24 hours |  |  |
| 0 04 052 | Principal time of daily reading of minimum temperature |  |  |
| 0 12 119 | Minimum temperature at height specified, past 24 hours |  |  |
| 0 13 004 | Vapour pressure |  |  |
| 0 12 151 | Standard deviation of daily mean temperature |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 0 14 032 | Total sunshine |  |  |
| 0 08 023 | First-order statistics | Set to missing (cancel) |  |
| 0 04 001 | Year | Beginning of the reference period |  |
| 0 04 001 | Year | Ending of the reference period |  |
| 0 04 002 | Month |  |  |
| 0 04 003 | Day | = 1 |  |
| 0 04 004 | Hour | = 0 |  |
| 0 04 074 | Short time period or displacement |  |  |
| 0 04 022 | Time period or displacement | = 1 |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 08 023 | First-order statistics | = 4 Mean value |  |
| 0 13 060 | Total accumulated precipitation |  |  |
| 0 04 053 | Number of days with precipitation equal to or more than 1 mm |  |  |
| 0 08 023 | First-order statistics | Set to missing (cancel) |  |
| 1 02 008 | Replicate 2 descriptors 8 times |  |  |
| 0 08 050 | Qualifier for number of missing values in calculation of statistic | Pressure, temperature, extreme temperatures, vapour pressure, precipitation, sunshine duration, maximum temperature, minimum temperature |  |
| 0 08 020 | Total number of missing entities (with respect to accumulation or average) | Years |  |

3 07 078 (Sequence for representation of monthly values suitable for CLIMAT data in compliance with regional or national reporting practices)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 07 076 | Monthly values from a land station in compliance with regional or national reporting practices |  |  |
| 3 07 077 | Monthly normals for a land station in compliance with regional or national reporting practices |  |  |

3 07 079 (Sequence for representation of synoptic reports from fixed land stations suitable for SYNOP data and for maritime data from coastal stations)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 090 | Surface station identification; time, horizontal and vertical coordinates |  |  |
| 3 02 031 | Pressure information |  |  |
| 3 02 035 | Basic synoptic "instantaneous" data |  |  |
| 3 02 036 | Clouds with bases below station level |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 047 | Direction of cloud drift |  |  |
| 0 08 002 | Vertical significance (surface observations) |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 048 | Direction and elevation of cloud |  |  |
| 3 02 037 | State of ground, snow depth, ground minimum temperature |  |  |
| 1 02 000 | Delayed replication of 2 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 22 061 | State of the sea |  |  |
| 0 20 058 | Visibility seawards from a coastal station |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 056 | Sea/water temperature | Sea/water surface temperature, method of measurement, depth below water surface |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 055 | Icing and ice |  |  |
| 3 02 043 | Basic synoptic "period" data |  |  |
| 3 02 044 | Evaporation data |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 02 045 | Radiation data (from 1 hour and 24-hour period) |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 046 | Temperature change |  |  |

3 07 080 (Sequence for representation of synoptic reports from a fixed land station suitable for SYNOP data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 090 | Surface station identification; time, horizontal and vertical coordinates |  |  |
| 3 02 031 | Pressure information |  |  |
| 3 02 035 | Basic synoptic "instantaneous" data |  |  |
| 3 02 036 | Clouds with bases below station level |  |  |
| 3 02 047 | Direction of cloud drift |  |  |
| 0 08 002 | Vertical significance (surface observations) |  |  |
| 3 02 048 | Direction and elevation of cloud |  |  |
| 3 02 037 | State of ground, snow depth, ground minimum temperature |  |  |
| 3 02 043 | Basic synoptic "period" data |  |  |
| 3 02 044 | Evaporation data |  |  |
| 1 01 002 | Replicate 1 descriptor 2 times |  |  |
| 3 02 045 | Radiation data (from 1 hour and 24-hour period) |  |  |
| 3 02 046 | Temperature change |  |  |

3 07 081 (Sequence for representation of synoptic reports from a fixed land station suitable for SYNOP data in compliance with reporting practices in RA I)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 090 | Surface station identification; time, horizontal and vertical coordinates |  |  |
| 3 02 031 | Pressure information |  |  |
| 3 02 035 | Basic synoptic "instantaneous" data |  |  |
| 3 02 036 | Clouds with bases below station level |  |  |
| 3 02 047 | Direction of cloud drift |  |  |
| 0 08 002 | Vertical significance (surface observations) | Set to missing (cancel) |  |
| 3 02 048 | Direction and elevation of cloud |  |  |
| 3 02 037 | State of ground, snow depth, ground minimum temperature |  |  |
| 0 12 122 | Ground minimum temperature of the preceding night |  |  |
| 0 13 056 | Character and intensity of precipitation |  |  |
| 0 13 057 | Time of beginning or end of precipitation |  |  |
| 0 20 101 | Locust (acridian) name |  |  |
| 0 20 102 | Locust (maturity) colour |  |  |
| 0 20 103 | Stage of development of locusts |  |  |
| 0 20 104 | Organization state of swarm or band of locusts |  |  |
| 0 20 105 | Size of swarm or band of locusts and duration of passage of swarm |  |  |
| 0 20 106 | Locust population density |  |  |
| 0 20 107 | Direction of movements of locust swarm |  |  |
| 0 20 108 | Extent of vegetation |  |  |
| 3 02 043 | Basic synoptic "period" data |  |  |
| 3 02 044 | Evaporation data |  |  |
| 1 01 002 | Replicate 1 descriptor 2 times |  |  |
| 3 02 045 | Radiation data (from 1 hour and 24-hour period) |  |  |
| 3 02 046 | Temperature change |  |  |

3 07 082 (Sequence for representation of synoptic reports from a fixed land station suitable for SYNOP data in compliance with reporting practices in RA II)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 090 | Surface station identification; time, horizontal and vertical coordinates |  | Note 2: 3 07 082 is deprecated. |
| 3 02 031 | Pressure information |  | Note 2: 3 07 082 is deprecated. |
| 3 02 035 | Basic synoptic "instantaneous" data |  | Note 2: 3 07 082 is deprecated. |
| 3 02 036 | Clouds with bases below station level |  | Note 2: 3 07 082 is deprecated. |
| 3 02 047 | Direction of cloud drift |  | Note 2: 3 07 082 is deprecated. |
| 0 08 002 | Vertical significance (surface observations) | Set to missing (cancel) | Note 2: 3 07 082 is deprecated. |
| 3 02 048 | Direction and elevation of cloud |  | Note 2: 3 07 082 is deprecated. |
| 3 02 037 | State of ground, snow depth, ground minimum temperature |  | Note 2: 3 07 082 is deprecated. |
| 0 12 121 | Ground minimum temperature | At the time of observation | Note 2: 3 07 082 is deprecated. |
| 0 12 122 | Ground minimum temperature of the preceding night |  | Note 2: 3 07 082 is deprecated. |
| 3 02 043 | Basic synoptic "period" data |  | Note 2: 3 07 082 is deprecated. |
| 3 02 044 | Evaporation data |  | Note 2: 3 07 082 is deprecated. |
| 1 01 002 | Replicate 1 descriptor 2 times |  | Note 2: 3 07 082 is deprecated. |
| 3 02 045 | Radiation data (from 1 hour and 24-hour period) |  | Note 2: 3 07 082 is deprecated. |
| 3 02 046 | Temperature change |  | Note 2: 3 07 082 is deprecated. |

3 07 083 (Sequence for representation of synoptic reports from a fixed land station suitable for SYNOP data in compliance with reporting practices in RA III)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 090 | Surface station identification; time, horizontal and vertical coordinates |  | Note 2: 3 07 082 is deprecated. |
| 3 02 031 | Pressure information |  | Note 2: 3 07 082 is deprecated. |
| 3 02 035 | Basic synoptic "instantaneous" data |  |  |
| 3 02 036 | Clouds with bases below station level |  |  |
| 3 02 047 | Direction of cloud drift |  |  |
| 0 08 002 | Vertical significance (surface observations) | Set to missing (cancel) |  |
| 3 02 048 | Direction and elevation of cloud |  |  |
| 3 02 037 | State of ground, snow depth, ground minimum temperature |  |  |
| 0 12 122 | Ground minimum temperature of the preceding night |  |  |
| 3 02 043 | Basic synoptic "period" data |  |  |
| 3 02 044 | Evaporation data |  |  |
| 1 01 002 | Replicate 1 descriptor 2 times |  |  |
| 3 02 045 | Radiation data (from 1 hour and 24-hour period) |  |  |
| 3 02 046 | Temperature change |  |  |

3 07 084 (Sequence for representation of synoptic reports from a fixed land station suitable for SYNOP data in compliance with reporting practices in RA IV)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 090 | Surface station identification; time, horizontal and vertical coordinates |  |  |
| 3 02 031 | Pressure information |  |  |
| 3 02 035 | Basic synoptic "instantaneous" data |  |  |
| 3 02 036 | Clouds with bases below station level |  |  |
| 3 02 047 | Direction of cloud drift |  |  |
| 0 08 002 | Vertical significance (surface observations) | Set to missing (cancel) |  |
| 3 02 048 | Direction and elevation of cloud |  |  |
| 3 02 037 | State of ground, snow depth, ground minimum temperature |  |  |
| 0 20 055 | State of sky in the tropics |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 2 05 001 | Signify character | Character field of 1 character |  |
| 3 02 043 | Basic synoptic "period" data |  |  |
| 3 02 044 | Evaporation data |  |  |
| 1 01 002 | Replicate 1 descriptor 2 times |  |  |
| 3 02 045 | Radiation data (from 1 hour and 24-hour period) |  |  |
| 3 02 046 | Temperature change |  |  |

3 07 086 (Sequence for representation of synoptic reports from a fixed land station suitable for SYNOP data in compliance with reporting practices in RA VI)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 090 | Surface station identification; time, horizontal and vertical coordinates |  |  |
| 3 02 031 | Pressure information |  |  |
| 3 02 035 | Basic synoptic "instantaneous" data |  |  |
| 3 02 036 | Clouds with bases below station level |  |  |
| 0 08 002 | Vertical significance (surface observations) | Set to missing (cancel) |  |
| 3 02 037 | State of ground, snow depth, ground minimum temperature |  |  |
| 3 02 066 | Dangerous weather phenomena |  |  |
| 3 02 043 | Basic synoptic "period" data |  |  |
| 3 02 044 | Evaporation data |  |  |
| 1 01 002 | Replicate 1 descriptor 2 times |  |  |
| 3 02 045 | Radiation data (from 1 hour and 24-hour period) |  |  |

3 07 087 ("Instantaneous" parameters of sequence 3 07 089)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 001 | WMO block and station numbers | IIiii |  |
| 0 02 001 | Type of station | ix |  |
| 3 01 011 | Year, month, day | YY |  |
| 3 01 012 | Hour, minute | GG, gg |  |
| 3 01 023 | Latitude/longitude (coarse accuracy) |  |  |
| 0 07 030 | Height of station ground above mean sea level |  |  |
| 0 07 031 | Height of barometer above mean sea level |  |  |
| 3 02 001 | Pressure and 3-hour pressure change | PoPoPoPo, PPPP, ppp, a |  |
| 0 10 062 | 24-hour pressure change | p24p24p24 |  |
| 0 07 004 | Pressure | Standard level a3 = 925, 850, 700, ..hPa ¦ Set to missing for lowland stations |  |
| 0 10 009 | Geopotential height | Standard level hhh ¦ Set to missing for lowland stations |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Temperature measurement |  |
| 0 12 101 | Temperature/air temperature | snTTT ¦ Scale: 2 |  |
| 0 12 103 | Dewpoint temperature | snTdTdTd ¦ Scale: 2 |  |
| 0 13 003 | Relative humidity |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 0 20 001 | Horizontal visibility | VV |  |
| 3 02 004 | General cloud information | Cloud cover (total) N: If N = 9, then 0 20 010 = 113, if N = /, then 0 20 010 = missing ¦ Vertical significance: If CL are observed, then 0 08 002 = 7 ¦ Low cloud: If CL are not observed and CM are observed, then 0 08 002 = 8 ¦ Middle cloud: If only CH are observed, 0 08 002 = 0, if N = 9, then 0 08 002 = 5, if N = 0, then 0 08 002 = 62, if N = /, then 0 08 002 = missing ¦ Cloud amount (of low or middle clouds) Nh: If N = 0, then 0 20 011 = 0, if N = 9, then 0 20 011 = 9, if N = /, then 0 20 011 = missing ¦ Height of base of cloud h: If N = 0 or /, then 0 20 013 = missing ¦ Cloud type (low clouds) CL: 0 20 012 = CL + 30, if N = 0, then 0 20 012 = 30, if N = 9 or /, then 0 20 012 = 62 ¦ Cloud type (middle clouds) CM: 0 20 012 = CM + 20, if N = 0, then 0 20 012 = 20, if N = 9 or / or CM = /, then 0 20 012 = 61 ¦ Cloud type (high clouds) CH: 0 20 012 = CH + 10, if N = 0, then 0 20 012 = 10, if N = 9 or / or CH = /, then 0 20 012 = 60 |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 02 005 | Cloud layer | Vertical significance: In any Cb layer, 0 08 002 = 4, else in the first replication, if N = 9, then 0 08 002 = 5, if N = /, then 0 08 002 = missing, else 0 08 002 = 1, in the other replications 0 08 002 = 2, 3, 4 ¦ Cloud amount Ns: In the first replication, if N = /, then 0 20 011 = missing, else 0 20 011 = Ns, in the other replications 0 20 011 = Ns ¦ Cloud type C: If N = 9 or /, then 0 20 012 = missing, else 0 20 012 = C ¦ Height of base of cloud hshs |  |

3 07 088 ("Period" parameters of sequence 3 07 089)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 20 003 | Present weather | ww |  |
| 0 04 024 | Time period or displacement | = -6 at 00, 06, 12, 18 UTC, = -3 at 03, 09,15, 21 UTC |  |
| 0 20 004 | Past weather (1) | W1 |  |
| 0 20 005 | Past weather (2) | W2 |  |
| 0 04 024 | Time period or displacement | = -24 (hours) |  |
| 0 02 004 | Type of instrumentation for evaporation measurement or type of crop for which evapotranspiration is reported | iE |  |
| 0 13 033 | Evaporation/evapotranspiration | EEE |  |
| 1 02 002 | Replicate 2 descriptors 2 times |  |  |
| 0 04 024 | Time period or displacement | = -24 (hours) in the first replication, = -1 (hour) in the second replication |  |
| 0 14 031 | Total sunshine | SSS in the first replication, SS in the second replication |  |
| 1 02 002 | Replicate 2 descriptors 2 times |  |  |
| 0 04 024 | Time period or displacement | tR |  |
| 0 13 011 | Total precipitation/total water equivalent | RRR ¦ = 0 No precipitation, = -0.1 Trace |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Temperature measurement |  |
| 0 04 024 | Time period or displacement | = -12 (hours) |  |
| 0 12 111 | Maximum temperature, at height and over period specified | snTxTxTx |  |
| 0 04 024 | Time period or displacement | = -12 (hours) |  |
| 0 12 112 | Minimum temperature, at height and over period specified | snTnTnTn |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Wind measurement |  |
| 0 02 002 | Type of instrumentation for wind measurement | iw |  |
| 0 08 021 | Time significance | = 2 Time averaged |  |
| 0 04 025 | Time period or displacement | = -10 (minutes) or number of minutes after a significant change of wind, if any |  |
| 0 11 001 | Wind direction | dd ¦ If dd = 00 Calm or dd = 99 Variable, 0 11 001 = 0 |  |
| 0 11 002 | Wind speed | ff |  |
| 0 08 021 | Time significance | Set to missing (cancel) |  |

3 07 089 (Sequence for representation of synoptic reports from a fixed land station suitable for SYNOP data manually encoded in CREX)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 07 087 | "Instantaneous" parameters of sequence 3 07 089 |  |  |
| 3 07 088 | "Period" parameters of sequence 3 07 089 |  |  |

3 07 090 (Sequence for representation of synoptic reports from a mobile land station suitable for SYNOP MOBIL data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 092 | Mobile surface station identification, date/time, horizontal and vertical coordinates |  |  |
| 3 02 031 | Pressure information |  |  |
| 3 02 035 | Basic synoptic "instantaneous" data |  |  |
| 3 02 036 | Clouds with bases below station level |  |  |
| 3 02 047 | Direction of cloud drift |  |  |
| 0 08 002 | Vertical significance (surface observations) |  |  |
| 3 02 048 | Direction and elevation of cloud |  |  |
| 3 02 037 | State of ground, snow depth, ground minimum temperature |  |  |
| 3 02 043 | Basic synoptic "period" data |  |  |
| 3 02 044 | Evaporation data |  |  |
| 1 01 002 | Replicate 1 descriptor 2 times |  |  |
| 3 02 045 | Radiation data (from 1 hour and 24-hour period) |  |  |
| 3 02 046 | Temperature change |  |  |

3 07 091 (BUFR template for surface observations from one-hour period with national and WMO station identification)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 089 | National station identification |  |  |
| 3 01 090 | Surface station identification; time, horizontal and vertical co-ordinates |  |  |
| 0 08 010 | Surface qualifier (temperature data) |  |  |
| 3 01 091 | Surface station instrumentation |  |  |
| 3 02 001 | Pressure and 3-hour pressure change |  |  |
| 0 07 004 | Pressure | Standard level |  |
| 0 10 009 | Geopotential height | Standard level |  |
| 3 02 072 | Temperature and humidity data |  |  |
| 1 03 000 | Delayed replication of 3 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 1 01 005 | Replicate 1 descriptor 5 times |  |  |
| 3 07 063 | Depth below land surface and soil temperature |  |  |
| 0 07 061 | Depth below land surface | Set to missing (cancel) |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 069 | Visibility data |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 0 07 033 | Height of sensor above water surface | Set to missing (cancel) |  |
| 1 05 000 | Delayed replication of 5 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 20 031 | Ice deposit (thickness) |  |  |
| 0 20 032 | Rate of ice accretion (estimated) |  |  |
| 0 02 038 | Method of water temperature and/or salinity measurement |  |  |
| 0 22 043 | Sea/water temperature | Scale: 2 |  |
| 3 02 021 | Waves |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 078 | State of ground and snow depth measurement |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 073 | Cloud data |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 074 | Present and past weather |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 175 | Intensity of precipitation, size of precipitation element |  |  |
| 1 02 000 | Delayed replication of 2 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 04 025 | Time period or displacement | = -10 (minutes) |  |
| 3 02 076 | Precipitation, obscuration and other phenomena |  |  |
| 3 02 071 | Wind data from one-hour period |  |  |
| 3 02 077 | Extreme temperature data |  |  |
| 0 07 033 | Height of sensor above water surface | Set to missing (cancel) |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 079 | Precipitation measurement |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 080 | Evaporation measurement |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 081 | Total sunshine data |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 082 | Radiation data |  |  |
| 1 02 000 | Delayed replication of 2 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 04 025 | Time period or displacement | = -10 (minutes) |  |
| 0 13 059 | Number of flashes (thunderstorm) |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 083 | First-order statistics of P, W, T, U data |  |  |
| 0 33 005 | Quality information (AWS data) |  |  |
| 0 33 006 | Internal measurement status information (AWS) |  |  |

3 07 092 (BUFR template for surface observations from n-minute period)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 150 | WIGOS identifier |  |  |
| 3 01 001 | WMO block and station numbers |  |  |
| 2 08 040 | Change width of CCITT IA5 field |  |  |
| 0 01 019 | Long station or site name | 40 characters |  |
| 2 08 000 | Change width of CCITT IA5 field |  |  |
| 3 01 011 | Year, month, day | The time identification refers to the end of the n-minute period. |  |
| 3 01 012 | Hour, minute |  |  |
| 3 01 021 | Latitude/longitude (high accuracy) |  |  |
| 0 07 030 | Height of station ground above mean sea level |  |  |
| 0 01 023 | Observation sequence number |  |  |
| 1 08 000 | Delayed replication of 8 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 07 031 | Height of barometer above mean sea level |  |  |
| 2 04 018 | Add associated field |  |  |
| 0 31 021 | Associated field significance |  |  |
| 0 10 004 | Pressure | Measured value of the air pressure at the sensor location and sensor height |  |
| 0 10 051 | Pressure reduced to mean sea level |  |  |
| 0 07 004 | Pressure | Standard level |  |
| 0 10 009 | Geopotential height | Standard level |  |
| 2 04 000 | Add associated field | Cancel |  |
| 1 15 000 | Delayed replication of 15 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 08 010 | Surface qualifier (temperature data) |  |  |
| 2 04 018 | Add associated field |  |  |
| 0 31 021 | Associated field significance |  |  |
| 0 12 101 | Temperature/air-temperature | Scale: 2 |  |
| 0 12 103 | Dewpoint temperature | Scale: 2 |  |
| 2 02 129 | Change scale |  |  |
| 2 01 132 | Change data width |  |  |
| 0 13 003 | Relative humidity | Mandatory to report (presuming a humidity sensor is installed), data width 11 bits |  |
| 2 01 000 | Change data width | Cancel |  |
| 2 02 000 | Change scale | Cancel |  |
| 0 13 009 | Relative humidity | Original measured value |  |
| 2 04 000 | Add associated field | Cancel |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 0 08 010 | Surface qualifier (temperature data) | Set to missing (cancel) |  |
| 1 07 000 | Delayed replication of 7 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 07 061 | Depth below land surface |  |  |
| 2 04 018 | Add associated field |  |  |
| 0 31 021 | Associated field significance |  |  |
| 0 12 130 | Soil temperature |  |  |
| 0 13 111 | Soil moisture |  |  |
| 2 04 000 | Add associated field | Cancel |  |
| 0 07 061 | Depth below land surface | Set to missing (cancel) |  |
| 1 05 000 | Delayed replication of 5 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 33 041 | Attribute of following value |  |  |
| 2 04 018 | Add associated field |  |  |
| 0 31 021 | Associated field significance |  |  |
| 0 20 001 | Horizontal visibility |  |  |
| 2 04 000 | Add associated field | Cancel |  |
| 1 13 000 | Delayed replication of 13 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 2 04 018 | Add associated field |  |  |
| 0 31 021 | Associated field significance |  |  |
| 0 20 010 | Cloud cover (total) |  |  |
| 2 04 000 | Add associated field | Cancel |  |
| 1 07 000 | Delayed replication of 7 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 08 002 | Vertical significance (surface observations) |  |  |
| 2 04 018 | Add associated field |  |  |
| 0 31 021 | Associated field significance |  |  |
| 0 20 011 | Cloud amount |  |  |
| 0 20 013 | Height of base of cloud |  |  |
| 2 04 000 | Add associated field | Cancel |  |
| 0 08 002 | Vertical significance (surface observations) | Set to missing (cancel) |  |
| 1 05 000 | Delayed replication of 5 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 2 04 018 | Add associated field |  |  |
| 0 31 021 | Associated field significance |  |  |
| 0 20 062 | State of ground (with or without snow) |  |  |
| 0 13 013 | Total snow depth |  |  |
| 2 04 000 | Add associated field | Cancel |  |
| 1 05 000 | Delayed replication of 5 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 04 025 | Time period or displacement | = -n minutes |  |
| 2 04 018 | Add associated field |  |  |
| 0 31 021 | Associated field significance |  |  |
| 0 20 003 | Present weather |  |  |
| 2 04 000 | Add associated field | Cancel |  |
| 1 05 000 | Delayed replication of 5 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 04 025 | Time period or displacement | = -n minutes |  |
| 2 04 018 | Add associated field |  |  |
| 0 31 021 | Associated field significance |  |  |
| 0 13 011 | Total precipitation/total water equivalent | Snow |  |
| 2 04 000 | Add associated field | Cancel |  |
| 1 15 000 | Delayed replication of 15 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 08 021 | Time significance | = 2 Time averaged |  |
| 0 04 025 | Time period or displacement | = -10 minutes, or number of minutes after a significant change of wind |  |
| 2 04 018 | Add associated field |  |  |
| 0 31 021 | Associated field significance |  |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 2 04 000 | Add associated field | Cancel |  |
| 0 08 021 | Time significance | Set to missing (cancel) |  |
| 2 04 018 | Add associated field |  |  |
| 0 31 021 | Associated field significance |  |  |
| 0 11 043 | Maximum wind gust direction |  |  |
| 0 11 041 | Maximum wind gust speed |  |  |
| 2 04 000 | Add associated field | Cancel |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 1 05 000 | Delayed replication of 5 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 04 025 | Time period or displacement | = -n minutes (default n = 10) |  |
| 2 04 018 | Add associated field |  |  |
| 0 31 021 | Associated field significance |  |  |
| 0 14 031 | Total sunshine |  |  |
| 2 04 000 | Add associated field | Cancel |  |
| 1 10 000 | Delayed replication of 10 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor | Open or close (1/0) |  |
| 0 04 025 | Time period or displacement | = -n minutes (default n = 10) |  |
| 2 04 018 | Add associated field |  |  |
| 0 31 021 | Associated field significance |  |  |
| 0 14 002 | Long-wave radiation, integrated over period specified | Upward long-wave radiation ¦ according to BUFR Table B, under Class 14, Note 2: negative values |  |
| 0 14 002 | Long-wave radiation, integrated over period specified | Downward long-wave radiation ¦ according to BUFR Table B, under Class 14, Note 1: positive values |  |
| 0 14 004 | Short-wave radiation, integrated over period specified | Upward short-wave radiation ¦ according to BUFR Table B, under Class 14, Note 2: negative values |  |
| 0 14 028 | Global solar radiation (high accuracy), integrated over period specified |  |  |
| 0 14 029 | Diffuse solar radiation (high accuracy), integrated over period specified |  |  |
| 0 14 030 | Direct solar radiation (high accuracy), integrated over period specified |  |  |
| 2 04 000 | Add associated field | Cancel |  |
| 1 13 000 | Delayed replication of 13 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 04 025 | Time period or displacement | = -n minutes (default n = 10) |  |
| 0 02 071 | Spectrographic wavelength | UV-A: 315 nm |  |
| 0 02 072 | Spectrographic width | UV-A: 85 nm |  |
| 2 04 018 | Add associated field |  |  |
| 0 31 021 | Associated field significance |  |  |
| 0 14 072 | Global UV irradiation | Integrated over period specified ¦ UV-A irradiation ¦ according to BUFR Table B under Class 14, Note 8 (ISO 21348: UV-A wave length range 315 ≤ λ ≤400 nm) |  |
| 2 04 000 | Add associated field | Cancel |  |
| 0 02 071 | Spectrographic wavelength | UV-B: 280 nm |  |
| 0 02 072 | Spectrographic width | UV-B: 35 nm |  |
| 2 04 018 | Add associated field |  |  |
| 0 31 021 | Associated field significance | Quality flag |  |
| 0 14 072 | Global UV irradiation | Integrated over period specified ¦ UV-B irradiation ¦ according to BUFR Table B under Class 14, Note 8 (ISO 21348 UV-B wave length range 280 ≤ λ ≤ 315 nm) |  |
| 2 04 000 | Add associated field | Cancel |  |

3 07 096 (Sequence for representation of SYNOP with supplementary information on one-hour observations)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 090 | Surface station identification; time, horizontal and vertical coordinates |  |  |
| 3 01 089 | National station identification |  |  |
| 0 08 010 | Surface qualifier (temperature data) |  |  |
| 3 01 091 | Surface station instrumentation |  |  |
| 3 02 084 | "Instantaneous" data of sequence 3 07 096 |  |  |
| 3 02 085 | "Period" data of sequence 3 07 096 |  |  |
| 0 33 005 | Quality information (AWS data) |  |  |
| 0 33 006 | Internal measurement status information (AWS) |  |  |

3 07 101 (Snow observation)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 089 | National station identification |  |  |
| 0 01 019 | Long station or site name |  |  |
| 0 02 001 | Type of station |  |  |
| 3 01 011 | Year, month, day |  |  |
| 3 01 012 | Hour, minute |  |  |
| 3 01 021 | Latitude/longitude (high accuracy) |  |  |
| 0 07 030 | Height of station ground above mean sea level |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 02 177 | Method of snow depth measurement |  |  |
| 0 20 062 | State of the ground (with or without snow) |  |  |
| 0 13 013 | Total snow depth |  |  |

3 07 102 (Road weather information)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 089 | National station identification |  |  |
| 0 01 018 | Short station or site name | For identification of the road weather monitoring site |  |
| 0 01 015 | Station or site name |  |  |
| 0 01 104 | State/federal state identifier |  |  |
| 0 01 105 | Highway designator |  |  |
| 0 01 106 | Location along highway as indicated by position markers |  |  |
| 0 03 017 | Extended type of station |  |  |
| 0 03 018 | Type of road |  |  |
| 0 03 019 | Type of construction |  |  |
| 3 01 011 | Year, month, day |  |  |
| 3 01 012 | Hour, minute |  |  |
| 3 01 021 | Latitude/longitude (high accuracy) |  |  |
| 0 07 030 | Height of station ground above mean sea level |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 12 103 | Dewpoint temperature |  |  |
| 0 13 003 | Relative humidity |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 0 20 001 | Horizontal visibility |  |  |
| 1 09 000 | Delayed replication of 9 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 03 016 | Position of road sensors |  | Note 17: Each position of road sensors includes the measurements of: - Road surface temperature - Road subsurface temperatures - Water film thickness - Road surface condition |
| 0 12 128 | Road surface temperature |  |  |
| 1 02 000 | Delayed replication of 2 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 07 061 | Depth below land surface | = 0.30 m in the first replication, = e.g. 0.15 or 0.07 m in the second replication |  |
| 0 12 129 | Road subsurface temperature |  |  |
| 0 07 061 | Depth below land surface | Set to missing (cancel) |  |
| 0 13 116 | Water film thickness |  |  |
| 0 20 138 | Road surface condition |  | Note 45: Some types of stations do not have the ability to identify the surface condition accurately. They can only report conditions such as "not dry" or "glazed". The code table for road surface conditions has been adjusted accordingly. |
| 0 04 025 | Time period or displacement | = -15 minutes |  |
| 0 20 024 | Intensity of phenomena | Intensity (light, moderate, heavy) of precipitation |  |
| 0 13 055 | Intensity of precipitation |  | Note 74: To represent Intensity of precipitation, type of precipitation and state of functionality, 0 20 024 (Code table), 0 20 021 (Flag table) and 0 33 005 (Flag table) are used, respectively. |
| 0 20 021 | Type of precipitation |  | Note 74: To represent Intensity of precipitation, type of precipitation and state of functionality, 0 20 024 (Code table), 0 20 021 (Flag table) and 0 33 005 (Flag table) are used, respectively. |
| 0 13 011 | Total precipitation/total water equivalent |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 08 021 | Time significance | = 2 Time averaged |  |
| 0 04 025 | Time period or displacement | = -10 minutes |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 08 021 | Time significance | Set to missing (cancel) |  |
| 0 04 025 | Time period or displacement | In minutes |  |
| 0 11 043 | Maximum wind gust direction |  |  |
| 0 11 041 | Maximum wind gust speed |  |  |
| 0 33 005 | Quality information (AWS data) |  |  |

3 07 103 (Snow observation, snow density, snow water equivalent)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 150 | WIGOS identifier |  |  |
| 3 07 101 | Snow observation |  |  |
| 0 13 117 | Snow density (liquid water content) |  |  |
| 0 03 028 | Method of snow water equivalent measurement |  |  |
| 0 13 163 | Snow water equivalent |  |  |

3 07 104 (Cryo station temperature profile)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 150 | WIGOS identifier |  |  |
| 0 01 019 | Long station or site name |  |  |
| 0 02 001 | Type of station |  |  |
| 3 01 011 | Year, month, day |  |  |
| 3 01 013 | Hour, minute, second |  |  |
| 3 01 021 | Latitude/longitude (high accuracy) |  |  |
| 0 07 030 | Height of station ground above mean sea level |  |  |
| 0 08 029 | Surface type |  |  |
| 0 13 115 | Ice thickness |  |  |
| 0 20 062 | State of the ground (with or without snow) |  |  |
| 0 13 119 | Snow depth |  |  |
| 0 02 177 | Method of snow depth measurement |  |  |
| 0 08 010 | Surface qualifier (for temperature data) |  |  |
| 0 12 161 | Skin temperature |  |  |
| 1 11 000 | Delayed replication of 11 descriptor |  |  |
| 0 31 001 | Delayed replication factor |  |  |
| 0 07 034 | Vertical distance of sensor |  |  |
| 0 02 096 | Type of temperature sensor |  |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 12 131 | Snow temperature |  |  |
| 2 01 131 | Change data width |  |  |
| 2 02 129 | Change scale |  |  |
| 0 12 130 | Soil temperature |  |  |
| 2 02 000 | Change scale | Cancel |  |
| 2 01 000 | Change data width | Cancel |  |
| 0 12 133 | Ice temperature |  |  |
| 0 13 082 | Water temperature |  |  |

3 07 182 (Sequence for representation of synoptic reports from a fixed land station suitable for SYNOP data in compliance with reporting practices in RA II)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 090 | Surface station identification, time, horizontal and vertical coordinates |  |  |
| 3 02 031 | Pressure information |  |  |
| 3 02 035 | Basic synoptic “instantaneous” data |  |  |
| 3 02 036 | Clouds with bases below station level |  |  |
| 3 02 047 | Direction of cloud drift |  |  |
| 0 08 002 | Vertical significance (surface observations) | Set to missing (cancel) |  |
| 3 02 048 | Direction and elevation of cloud |  |  |
| 3 02 037 | State of ground, snow depth, ground minimum temperature | Past 12 hours |  |
| 0 12 120 | Ground temperature | snT’gT’g |  |
| 0 12 122 | Ground minimum temperature of the preceding night | snTgTg |  |
| 3 02 043 | Basic synoptic “period” data |  |  |
| 3 02 044 | Evaporation data |  |  |
| 1 01 002 | Replicate 1 descriptor 2 times |  |  |
| 3 02 045 | Radiation data (from 1 hour and 24-hour period) |  |  |
| 3 02 046 | Temperature change |  |  |

Note 4: Additional descriptors are required to reduce the workload with respect to the station database, e.g. for identification of the federal state and identification of the highway.

Note 49: The majority of stations has only one position on the road and one subsurface temperature sensor. Delayed Replications have been introduced to increase flexibility and volume efficiency.