Class 24: Radiological elements

| FXY | ELEMENT NAME | BUFR UNIT | SCALE | REF VALUE | BITS | CREX UNIT | SCALE | CHARS | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 24 001 | Estimate of amount of radioactivity released up to specified time | Bq | -11 | 0 | 28 | Bq | -11 | 9 |  |
| 0 24 002 | Estimated maximum potential release | Bq | -11 | 0 | 28 | Bq | -11 | 9 |  |
| 0 24 003 | Composition of release | Code table | 0 | 0 | 5 | Code table | 0 | 2 |  |
| 0 24 004 | Element name | CCITT IA5 | 0 | 0 | 16 | Character | 0 | 2 |  |
| 0 24 005 | Isotope mass | Numeric | 0 | 0 | 9 | Numeric | 0 | 3 |  |
| 0 24 011 | Dose | mSv | 2 | 0 | 32 | mSv | 2 | 10 |  |
| 0 24 012 | Trajectory dose (defined location and expected time of arrival) | mSv | 2 | 0 | 32 | mSv | 2 | 10 |  |
| 0 24 013 | Gamma dose in air along the main transport path (defined location and time period) | mSv | 2 | 0 | 32 | mSv | 2 | 10 |  |
| 0 24 014 | Gamma radiation dose rate | nSv/h | 1 | 0 | 14 | nSv/h | 1 | 4 | Note B81: Gamma radiation dose rate 0 24 014 is intended to be used for reporting of this element under normal conditions, nuclear accidents excluded. |
| 0 24 021 | Air concentration (of named isotope type including gross beta) | Bq m-3 | 2 | 0 | 32 | Bq m-3 | 2 | 10 |  |
| 0 24 022 | Concentration in precipitation (of named isotope type) | Bq/l | 2 | 0 | 32 | Bq/l | 2 | 10 |  |
| 0 24 023 | Pulse rate of beta radiation | /s | 1 | 0 | 14 | /s | 1 | 4 |  |
| 0 24 024 | Pulse rate of gamma radiation | /s | 1 | 0 | 14 | /s | 1 | 4 |  |

Note B162: Useful ranges used above:1011 Bq to 1019 Bq for releases;10-2 Bq to 107 Bq and 10-2 mSv to 107 mSv for concentration and doses.