Class 27: Non-coordinate location (horizontal - 1)

| FXY | ELEMENT NAME | BUFR UNIT | SCALE | REF VALUE | BITS | CREX UNIT | SCALE | CHARS | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 27 001 | Latitude (high accuracy) | deg | 5 | -9000000 | 25 | deg | 5 | 7 |  |
| 0 27 002 | Latitude (coarse accuracy) | deg | 2 | -9000 | 15 | deg | 2 | 4 |  |
| 0 27 003 | Alternate latitude (coarse accuracy) | deg | 2 | -9000 | 15 | deg | 2 | 4 | Note B132: The alternate latitude may be used when the computation of the position yields multiple solutions and there is no a priori way to distinguish between them. |
| 0 27 004 | Alternate latitude (high accuracy) | deg | 5 | -9000000 | 25 | deg | 5 | 7 | Note B132: The alternate latitude may be used when the computation of the position yields multiple solutions and there is no a priori way to distinguish between them. |
| 0 27 010 | Footprint axis 1 | m | -1 | 0 | 14 | m | -1 | 5 |  |
| 0 27 020 | Satellite location counter | Numeric | 0 | 0 | 16 | Numeric | 0 | 5 | Note B143: The satellite location counter is calculated as:counter = superswath No. x 1000 + box No. x 10 + minibox No. |
| 0 27 021 | Satellite sublocation dimension | Numeric | 0 | 0 | 16 | Numeric | 0 | 5 | Note B144: The satellite sublocation dimension is calculated as:dimension = minibox dimension + box dimension where: minibox dimension = lines x 1000 + spots x 100 box dimension = lines x 10 + spots |
| 0 27 031 | In direction of 0 degrees longitude, distance from the Earth's centre | m | 2 | -1073741824 | 31 | m | 2 | 10 | Note B96: Left-handed x, y and z axes have been chosen for descriptor 0 27 031. ¦ Note B151: The value for descriptor 0 27 031 has been chosen to be suitable for polar orbiting satellites in approximately Sun-synchronous orbits. Geostationary orbits would require greater data widths for distance and slightly less for speed. |
| 0 27 079 | Horizontal width of sampled volume | m | 0 | 0 | 18 | m | 0 | 6 |  |
| 0 27 080 | Viewing azimuth angle | degree true | 2 | 0 | 16 | degree true | 0 | 5 |  |