Class 28: Non-coordinate location (horizontal - 2)

| FXY | ELEMENT NAME | BUFR UNIT | SCALE | REF VALUE | BITS | CREX UNIT | SCALE | CHARS | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 28 001 | Longitude (high accuracy) | deg | 5 | -18000000 | 26 | deg | 5 | 8 |  |
| 0 28 002 | Longitude (coarse accuracy) | deg | 2 | -18000 | 16 | deg | 2 | 5 |  |
| 0 28 003 | Alternate longitude (coarse accuracy) | deg | 2 | -18000 | 16 | deg | 2 | 5 | Note B133: The alternate longitude may be used when the computation of the position yields multiple solutions and there is no a priori way to distinguish between them. |
| 0 28 004 | Alternate longitude (high accuracy) | deg | 5 | -18000000 | 26 | deg | 5 | 8 | Note B133: The alternate longitude may be used when the computation of the position yields multiple solutions and there is no a priori way to distinguish between them. |
| 0 28 010 | Footprint axis 2 | m | -1 | 0 | 14 | m | -1 | 5 |  |
| 0 28 031 | In direction 90 degrees East, distance from the Earth's centre | m | 2 | -1073741824 | 31 | m | 2 | 10 | Note B94: Left handed x, y and z axes have been chosen for descriptor 0 28 031. ¦ Note B152: The value for descriptor 0 28 031 has been chosen to be suitable for polar orbiting satellites in approximately Sun-synchronous orbits. Geostationary orbits would require greater data widths for distance and slightly less for speed. |