Category 00: BUFR table entries sequences

3 00 002

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 00 002 | Table A: data category description, line 1 |  |  |
| 0 00 003 | Table A: data category description, line 2 |  |  |

3 00 003 (F, X, Y of descriptor to be added or defined)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 00 010 | F descriptor to be added or defined |  |  |
| 0 00 011 | X descriptor to be added or defined |  |  |
| 0 00 012 | Y descriptor to be added or defined |  |  |

3 00 004

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 00 003 | F, X, Y of descriptor to be added or defined |  |  |
| 0 00 013 | Element name, line 1 |  |  |
| 0 00 014 | Element name, line 2 |  |  |
| 0 00 015 | Units name |  |  |
| 0 00 016 | Units scale sign |  |  |
| 0 00 017 | Units scale |  |  |
| 0 00 018 | Units reference sign |  |  |
| 0 00 019 | Units reference value |  |  |
| 0 00 020 | Element data width |  |  |

3 00 010

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 00 003 | F, X, Y of descriptor to be added or defined |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 00 030 | Descriptor defining sequence |  |  |

3 00 015 (Code table definition)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 00 030 | Descriptor defining sequence |  |  |
| 1 02 000 | Delayed replication of 2 descriptors |  |  |
| 0 31 002 | Extended delayed descriptor replication factor |  |  |
| 0 00 024 | Code figure |  |  |
| 0 00 025 | Code figure meaning |  |  |

3 00 016 (Flag table definition)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 00 030 | Descriptor defining sequence |  |  |
| 1 02 000 | Delayed replication of 2 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 00 026 | Bit number |  |  |
| 0 00 027 | Bit number meaning |  |  |

Note D32: It could be argued that, as only additions are possible, only complete lines should be allowed; but it is conceivable that local areas will require changes as well as additions, so it is better and in any case clearer to provide descriptions for all the fields.

Note D33: It is better to use different Class 00 descriptors for the defining and defined elements, in the same way as different descriptors correspond to pressure considered as a coordinate and pressure measured at a given point; otherwise special rules would be needed to interpret such message. Entries 0 00 010 to 0 00 012 define F, X and Y for Tables B and D; entry 0 00 030 is a descriptor used as data and provides the F, X and Y values defining a sequence for Table D entries.

Note D63: These entries include the facility to update the Table A code figure and data description.