Class 21: Radar data

| FXY | ELEMENT NAME | BUFR UNIT | SCALE | REF VALUE | BITS | CREX UNIT | SCALE | CHARS | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 21 001 | Horizontal reflectivity | dB | 0 | -64 | 7 | dB | 0 | 3 |  |
| 0 21 002 | Vertical reflectivity | dB | 0 | -64 | 7 | dB | 0 | 3 |  |
| 0 21 003 | Differential reflectivity | dB | 1 | -5 | 7 | dB | 1 | 3 |  |
| 0 21 004 | Differential reflectivity | dB | 2 | -800 | 11 | dB | 2 | 4 |  |
| 0 21 005 | Linear depolarization ratio | dB | 0 | -65 | 6 | dB | 0 | 2 |  |
| 0 21 006 | Circular depolarization ratio | dB | 0 | -65 | 6 | dB | 0 | 2 |  |
| 0 21 007 | Radar reflectivity factor | dB | 2 | -9000 | 15 | dB | 0 | 0 |  |
| 0 21 008 | Uncertainty in radar reflectivity factor | dB | 2 | 0 | 13 | dB | 0 | 0 |  |
| 0 21 009 | Vertical Doppler velocity | m/s | 2 | -10000 | 15 | m/s | 0 | 0 |  |
| 0 21 010 | Uncertainty in vertical Doppler velocity | m/s | 2 | -10000 | 15 | m/s | 0 | 0 |  |
| 0 21 011 | Doppler mean velocity in x-direction | m/s | 0 | -128 | 8 | m/s | 0 | 3 |  |
| 0 21 012 | Doppler mean velocity in y-direction | m/s | 0 | -128 | 8 | m/s | 0 | 3 |  |
| 0 21 013 | Doppler mean velocity in z-direction | m/s | 0 | -128 | 8 | m/s | 0 | 3 |  |
| 0 21 014 | Doppler mean velocity (radial) | m/s | 1 | -4096 | 13 | m/s | 1 | 4 |  |
| 0 21 017 | Doppler velocity spectral width | m/s | 1 | 0 | 8 | m/s | 1 | 3 |  |
| 0 21 018 | Extended NYQUIST velocity | m/s | 1 | 0 | 10 | m/s | 1 | 4 |  |
| 0 21 019 | High NYQUIST velocity | m/s | 1 | 0 | 10 | m/s | 1 | 3 |  |
| 0 21 020 | Dynamic atmospheric correction | Numeric | 3 | -65536 | 17 |  |  |  |  |
| 0 21 021 | Echo tops | m | -3 | 0 | 4 | m | -3 | 2 |  |
| 0 21 022 | Range bin offset | m | 1 | 0 | 14 | m | 1 | 5 |  |
| 0 21 023 | Range bin size | m | 0 | 0 | 14 | m | 0 | 5 |  |
| 0 21 024 | Azimuth offset | deg | 1 | 0 | 12 | deg | 1 | 4 |  |
| 0 21 025 | Azimuthal resolution | deg | 1 | 0 | 8 | deg | 1 | 3 |  |
| 0 21 026 | ASCAT sigma-0 usability | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 21 027 | Land contribution ratio | Numeric | 4 | 0 | 14 | Numeric | 4 | 5 |  |
| 0 21 028 | Differential phase | deg | 1 | 0 | 12 | deg | 1 | 4 |  |
| 0 21 029 | Cross-polarization correlation coefficient | Numeric | 2 | -100 | 8 | Numeric | 2 | 3 |  |
| 0 21 030 | Signal to noise ratio | dB | 0 | -32 | 8 | dB | 0 | 3 |  |
| 0 21 031 | Vertically integrated liquid-water content | kg m-2 | 0 | 0 | 7 | kg m-2 | 0 | 3 |  |
| 0 21 036 | Radar rainfall intensity | m/s | 7 | 0 | 12 | m/s | 7 | 4 |  |
| 0 21 041 | Bright-band height | m | -2 | 0 | 8 | m | -2 | 3 |  |
| 0 21 051 | Signal power above 1 mW | dB | 0 | -256 | 8 | dB | 0 | 3 |  |
| 0 21 062 | Backscatter | dB | 2 | -5000 | 13 | dB | 2 | 4 |  |
| 0 21 063 | Radiometric resolution (noise value) | % | 1 | 0 | 10 | % | 1 | 4 |  |
| 0 21 064 | Clutter noise estimate | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 |  |
| 0 21 065 | Missing packet counter | Numeric | 0 | -127 | 8 | Numeric | 0 | 3 |  |
| 0 21 066 | Wave scatterometer product confidence data | Flag table | 0 | 0 | 12 | Flag table | 0 | 4 |  |
| 0 21 067 | Wind product confidence data | Flag table | 0 | 0 | 13 | Flag table | 0 | 5 |  |
| 0 21 068 | Radar altimeter product confidence data | Flag table | 0 | 0 | 8 | Flag table | 0 | 3 |  |
| 0 21 069 | SST product confidence data | Flag table | 0 | 0 | 10 | Flag table | 0 | 4 |  |
| 0 21 070 | SST product confidence data (SADIST-2) | Flag table | 0 | 0 | 23 | Flag table | 0 | 6 |  |
| 0 21 071 | Peakiness | Numeric | 0 | 0 | 16 | Numeric | 0 | 5 |  |
| 0 21 072 | Satellite altimeter calibration status | Flag table | 0 | 0 | 4 | Flag table | 0 | 2 |  |
| 0 21 073 | Satellite altimeter instrument mode | Flag table | 0 | 0 | 9 | Flag table | 0 | 3 |  |
| 0 21 075 | Image spectrum intensity | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 |  |
| 0 21 076 | Representation of intensities | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 21 077 | Altitude correction (ionosphere) | m | 3 | 0 | 14 | m | 3 | 5 |  |
| 0 21 078 | Altitude correction (dry troposphere) | m | 3 | 0 | 9 | m | 3 | 3 |  |
| 0 21 079 | Altitude correction (wet troposphere) | m | 3 | 2000 | 10 | m | 3 | 4 |  |
| 0 21 080 | Altitude correction (calibration constant) | m | 3 | 0 | 11 | m | 3 | 4 |  |
| 0 21 081 | Open loop correction (height-time loop) | m | 3 | 0 | 10 | m | 3 | 4 |  |
| 0 21 082 | Open loop correction (auto gain control) | dB | 3 | -3000 | 14 | dB | 3 | 5 |  |
| 0 21 083 | Warm target calibration | Numeric | 0 | 0 | 16 | Numeric | 0 | 5 |  |
| 0 21 084 | Cold target calibration | Numeric | 0 | 0 | 16 | Numeric | 0 | 5 |  |
| 0 21 085 | ATSR sea-surface temperature across-track band number | Numeric | 0 | 0 | 4 | Numeric | 0 | 2 |  |
| 0 21 086 | Number of pixels in nadir only, average | Numeric | 0 | 0 | 9 | Numeric | 0 | 3 |  |
| 0 21 087 | Number of pixels in dual view, average | Numeric | 0 | 0 | 9 | Numeric | 0 | 3 |  |
| 0 21 088 | Wet backscatter | dB | 2 | -5000 | 13 | dB | 2 | 4 |  |
| 0 21 091 | Radar signal Doppler spectrum 0th moment | dB | 0 | -100 | 8 | dB | 0 | 3 |  |
| 0 21 092 | RASS signal Doppler spectrum 0th moment, referring to RASS signal | dB | 0 | -100 | 8 | dB | 0 | 3 |  |
| 0 21 093 | Ku band peakiness | Numeric | 3 | 0 | 16 | Numeric | 3 | 5 |  |
| 0 21 094 | S band peakiness | Numeric | 3 | 0 | 16 | Numeric | 3 | 5 |  |
| 0 21 095 | Kp coefficient A | Numeric | 6 | 0 | 20 | Numeric | 6 | 7 |  |
| 0 21 096 | Kp coefficient B | Numeric | 6 | 0 | 20 | Numeric | 6 | 7 |  |
| 0 21 097 | Kp coefficient C | Numeric | 6 | 0 | 20 | Numeric | 6 | 7 |  |
| 0 21 101 | Number of vector ambiguities | Numeric | 0 | 0 | 3 | Numeric | 0 | 1 |  |
| 0 21 102 | Index of selected wind vector | Numeric | 0 | 0 | 3 | Numeric | 0 | 1 |  |
| 0 21 103 | Total number of sigma-0 measurements | Numeric | 0 | 0 | 5 | Numeric | 0 | 2 |  |
| 0 21 104 | Likelihood computed for solution | Numeric | 3 | -30000 | 15 | Numeric | 3 | 5 |  |
| 0 21 105 | Normalized radar cross-section | dB | 2 | -10000 | 14 | dB | 2 | 5 |  |
| 0 21 106 | Kp variance coefficient (alpha) | Numeric | 3 | 0 | 14 | Numeric | 3 | 5 |  |
| 0 21 107 | Kp variance coefficient (beta) | Numeric | 8 | 0 | 16 | Numeric | 8 | 5 |  |
| 0 21 109 | SEAWINDS wind vector cell quality | Flag table | 0 | 0 | 17 | Flag table | 0 | 6 |  |
| 0 21 110 | Number of inner-beam sigma-0 (forward of satellite) | Numeric | 0 | 0 | 6 | Numeric | 0 | 2 |  |
| 0 21 111 | Number of outer-beam sigma-0 (forward of satellite) | Numeric | 0 | 0 | 6 | Numeric | 0 | 2 |  |
| 0 21 112 | Number of inner-beam sigma-0 (aft of satellite) | Numeric | 0 | 0 | 6 | Numeric | 0 | 2 |  |
| 0 21 113 | Number of outer-beam sigma-0 (aft of satellite) | Numeric | 0 | 0 | 6 | Numeric | 0 | 2 |  |
| 0 21 114 | Kp variance coefficient (gamma) | dB | 3 | -140000 | 18 | dB | 3 | 6 |  |
| 0 21 115 | SEAWINDS sigma-0 quality | Flag table | 0 | 0 | 17 | Flag table | 0 | 6 |  |
| 0 21 116 | SEAWINDS sigma-0 mode | Flag table | 0 | 0 | 17 | Flag table | 0 | 6 |  |
| 0 21 117 | Sigma-0 variance quality control | Numeric | 2 | 0 | 16 | Numeric | 2 | 5 |  |
| 0 21 118 | Attenuation correction on sigma-0 | dB | 2 | -10000 | 14 | dB | 2 | 5 |  |
| 0 21 119 | Wind scatterometer geophysical model function | Code table | 0 | 0 | 6 | Code table | 0 | 2 |  |
| 0 21 120 | Probability of rain | Numeric | 3 | 0 | 10 | Numeric | 3 | 4 |  |
| 0 21 121 | SEAWINDS NOF rain index | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 |  |
| 0 21 122 | Attenuation correction on sigma-0 (from tB) | dB | 2 | -10000 | 14 | dB | 2 | 5 |  |
| 0 21 123 | SEAWINDS normalized radar cross-section | dB | 2 | -30000 | 15 | dB | 2 | 5 |  |
| 0 21 128 | Number of valid points per second used to derive previous parameters | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 |  |
| 0 21 130 | Spectrum total energy | Numeric | 6 | 0 | 28 | Numeric | 6 | 9 |  |
| 0 21 131 | Spectrum max energy | Numeric | 6 | 0 | 28 | Numeric | 6 | 9 |  |
| 0 21 132 | Direction of spectrum max on higher resolution grid | deg | 3 | 0 | 19 | deg | 3 | 6 |  |
| 0 21 133 | Wavelength of spectrum max on higher resolution grid | m | 3 | 0 | 29 | m | 3 | 9 |  |
| 0 21 134 | Range resolution of cress covariance spectrum | rad/m | 3 | 0 | 19 | rad/m | 3 | 6 |  |
| 0 21 135 | Real part of cross spectra polar grid number of bins | Numeric | 3 | -524288 | 20 | Numeric | 3 | 7 |  |
| 0 21 136 | Imaginary part of cross spectra polar grid number of bins | Numeric | 3 | -524288 | 20 | Numeric | 3 | 7 |  |
| 0 21 137 | Ku band corrected ocean backscatter coefficient | dB | 2 | -32768 | 16 | dB | 2 | 5 |  |
| 0 21 138 | Std Ku band corrected ocean backscatter coefficient | dB | 2 | -32768 | 16 | dB | 2 | 5 |  |
| 0 21 139 | Ku band net instrumental correction for AGC | dB | 2 | -2048 | 12 | dB | 2 | 4 |  |
| 0 21 140 | S band corrected ocean backscatter coefficient | dB | 2 | -32768 | 16 | dB | 2 | 5 |  |
| 0 21 141 | Std S band corrected ocean backscatter coefficient | dB | 2 | -32768 | 16 | dB | 2 | 5 |  |
| 0 21 142 | S band net instrumental correction for AGC | dB | 2 | -1024 | 11 | dB | 2 | 4 |  |
| 0 21 143 | Ku band rain attenuation | dB | 2 | -1073741824 | 31 | dB | 2 | 10 |  |
| 0 21 144 | Altimeter rain flag | Flag table | 0 | 0 | 2 | Flag table | 0 | 1 |  |
| 0 21 145 | Ku band automatic gain control | dB | 2 | 0 | 13 | dB | 2 | 4 |  |
| 0 21 146 | RMS Ku band automatic gain control | dB | 2 | 0 | 8 | dB | 2 | 3 |  |
| 0 21 147 | Number of valid points for Ku band automatic gain control | Numeric | 0 | 0 | 5 | Numeric | 0 | 2 |  |
| 0 21 148 | Trailing edge variation flag | Flag table | 0 | 0 | 9 | Flag table | 0 | 3 |  |
| 0 21 150 | Beam co-location | Code table | 0 | 0 | 2 | Code table | 0 | 1 |  |
| 0 21 151 | Estimated error in sigma-0 at 40 degrees incidence angle | dB | 2 | 0 | 9 | dB | 2 | 3 |  |
| 0 21 152 | Slope at 40 degrees incidence angle | dB/deg | 2 | -80 | 7 | dB/deg | 2 | 2 |  |
| 0 21 153 | Estimated error in slope at 40 degrees incidence angle | dB/deg | 2 | -40 | 6 | dB/deg | 2 | 2 |  |
| 0 21 154 | Soil moisture sensitivity | dB | 2 | 0 | 12 | dB | 2 | 4 |  |
| 0 21 155 | Wind vector cell quality | Flag table | 0 | 0 | 24 | Flag table | 0 | 8 |  |
| 0 21 156 | Backscatter distance | Numeric | 1 | -4096 | 13 | Numeric | 1 | 4 |  |
| 0 21 157 | Loss per unit length of atmosphere used | dB/m | 10 | 0 | 22 | dB/m | 10 | 7 |  |
| 0 21 158 | ASCAT Kp estimate quality | Code table | 0 | 0 | 2 | Code table | 0 | 1 |  |
| 0 21 159 | ASCAT sigma-0 usability | Code table | 0 | 0 | 2 | Code table | 0 | 1 |  |
| 0 21 160 | ASCAT use of synthetic data | Numeric | 3 | 0 | 10 | Numeric | 3 | 4 |  |
| 0 21 161 | ASCAT synthetic data quantity | Numeric | 3 | 0 | 10 | Numeric | 3 | 4 |  |
| 0 21 162 | ASCAT satellite orbit and attitude quality | Numeric | 3 | 0 | 10 | Numeric | 3 | 4 |  |
| 0 21 163 | ASCAT solar array reflection contamination | Numeric | 3 | 0 | 10 | Numeric | 3 | 4 |  |
| 0 21 164 | ASCAT telemetry presence and quality | Numeric | 3 | 0 | 10 | Numeric | 3 | 4 |  |
| 0 21 165 | ASCAT extrapolated reference function presence | Numeric | 3 | 0 | 10 | Numeric | 3 | 4 |  |
| 0 21 166 | Land fraction | Numeric | 3 | 0 | 10 | Numeric | 3 | 4 |  |
| 0 21 167 | Subsurface scattering probability | % | 0 | 0 | 7 | % | 0 | 3 |  |
| 0 21 169 | Ice presence indicator | Code table | 0 | 0 | 2 | Code table | 0 | 1 |  |
| 0 21 170 | C band corrected ocean backscatter coefficient | dB | 2 | -32768 | 16 | dB | 2 | 5 |  |
| 0 21 171 | RMS C band corrected ocean backscatter coefficient | dB | 2 | -32768 | 16 | dB | 2 | 5 |  |
| 0 21 172 | C band net instrumental correction for AGC | dB | 2 | -2048 | 12 | dB | 2 | 4 |  |
| 0 21 173 | C band automatic gain control | dB | 2 | 0 | 13 | dB | 2 | 4 |  |
| 0 21 174 | RMS C band automatic gain control | dB | 2 | 0 | 9 | dB | 2 | 3 |  |
| 0 21 175 | Number of valid points for C band automatic gain control | Numeric | 0 | 0 | 10 | Numeric | 0 | 4 |  |
| 0 21 176 | High frequency variability correction | m | 3 | 0 | 16 | m | 3 | 5 |  |
| 0 21 177 | Corrected OCOG backscatter coefficient | dB | 2 | 0 | 16 | dB | 2 | 5 |  |
| 0 21 178 | STD of 20 Hz OCOG backscatter coefficient | dB | 2 | 0 | 16 | dB | 2 | 5 |  |
| 0 21 179 | Number of 20 Hz valid points for OCOG backscatter coefficient | Numeric | 0 | 0 | 16 | Numeric | 0 | 5 |  |
| 0 21 180 | Number of 20 Hz valid points for ocean backscatter coefficient | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 |  |
| 0 21 181 | 20 Hz ocean backscatter coefficient | dB | 2 | 0 | 16 | dB | 2 | 5 |  |
| 0 21 182 | 20 Hz Ku band peakiness | Numeric | 3 | 0 | 16 | Numeric | 3 | 5 |  |
| 0 21 183 | Specific band corrected ocean backscatter coefficient | dB | 2 | -32768 | 16 | dB | 2 | 5 |  |
| 0 21 184 | STD specific band corrected ocean backscatter coefficient | dB | 2 | -32768 | 16 | dB | 2 | 5 |  |
| 0 21 185 | Specific band net instrumental correction for AGC | dB | 2 | -2048 | 12 | dB | 2 | 4 |  |
| 0 21 186 | Specific band automatic gain control | dB | 2 | 0 | 13 | dB | 2 | 4 |  |
| 0 21 187 | RMS specific band automatic gain control | dB | 2 | 0 | 8 | dB | 2 | 3 |  |
| 0 21 188 | Number of valid points for specific band automatic gain control | Numeric | 0 | 0 | 7 | Numeric | 0 | 3 |  |
| 0 21 189 | Corrected OCOG backscatter coefficient (negative reference) | dB | 2 | -32768 | 16 | dB | 2 | 6 | Note 11: Descriptor 3 01 002 should not be used. |

Note 23: A precipitation value of -0.1 kg m-2 before scaling (-1 after scaling or in CREX) shall indicate a "trace" (non-measurable, less than 0.05 kg m-2).

Note 25: A previously defined significance may be cancelled by transmitting a "missing" from the appropriate code or flag table.

Note 27: All times are Universal Time Coordinated (UTC) unless otherwise noted.

Note 32: Bearing or azimuth shall only be used with respect to a stated location, and shall not redefine that location.

Note 62: Distance shall only be used with respect to a stated location and a bearing, azimuth or elevation; it shall not redefine that location.

Note 63: Downward radiation shall be assigned positive values.

Note 64: Downward w-components shall be assigned positive values where units are Pa s-1.

Note 66: East to west increments shall be assigned negative values.

Note 75: For CREX descriptors F = B, not 0.

Note 92: In order to distinguish unambiguously the cases of missing data and saturated pixels, n-bit image data should be encoded using a data width of n+1. Where such a descriptor is not already available in Class 30, operator descriptor 2 01 YYY should be used to modify the data width of the existing entry as required.

Note 105: Negative time periods or displacements shall be used to indicate time periods or displacements preceding the currently defined time.

Note 110: North to south increments shall be assigned negative values.

Note 115: Pixel data width can be changed with descriptor 2 01 YYY.

Note 121: Some indication of expected accuracy may be implied in conjunction with certain elements in this class.

Note 122: South latitude shall be assigned negative values.

Note 123: South to north v-components shall be assigned positive values.

Note 126: Surface wind direction measured at a station within 1° of the North Pole or within 1° of the South Pole shall be reported in such a way that the azimuth ring shall be aligned with its zero coinciding with the Greenwich 0° meridian.

Note 140: The pixel size on horizontal - 1 is given at location where map scale factor is unity.

Note 141: The pixel size on horizontal - 2 is given at location where map scale factor is unity.

Note 145: The significance of time periods or displacements may be indicated using the time significance code corresponding to table reference 0 08 021.

Note 149: The time location, when used with forecast values, shall indicate the time of the initial state for the forecast, or the beginning of the forecast period; when used with ensemble means of forecast values, the time location shall indicate the initial state or the beginning of the first forecast over which ensemble means are derived.

Note 155: This class may also contain elements relating to observational procedures.

Note 156: This class shall contain elements to describe the instrumentation used to obtain the meteorological elements reported.

Note 158: Time periods or displacements and time increments require an initial time location to be defined prior to their use, followed where appropriate by a time significance definition.

Note 160: Upward radiation shall be assigned negative values.

Note 161: Upward w-components shall be assigned positive values where units are m s-1.

Note 162: Useful ranges used above:1011 Bq to 1019 Bq for releases;10-2 Bq to 107 Bq and 10-2 mSv to 107 mSv for concentration and doses.

Note 163: Values of latitude and latitude increments are limited to the range -90 degrees to +90 degrees.

Note 164: Values of longitude are limited to the range -180 degrees to +180 degrees.

Note 165: Vertical elements and pressure shall be used to define values of these elements independent of the element or variable denoting the vertical coordinate.

Note 166: West longitude shall be assigned negative values.

Note 167: West to east u-components shall be assigned positive values.

Note 176: Where more than one time period or displacement is required to define complex time structures, they shall be defined in immediate succession, and the following ordering shall apply: ensemble period (if required), followed by forecast period (if required), followed by period for averaging or accumulation (if required).

Note 178: Where the expression "period specified" is entered under element name, an appropriate period shall be specified using descriptors from Class 04.

Note 179: Where values are accumulated or averaged (for example over a time period), the total number of values from which the accumulated or averaged values are obtained may be represented using reference 0 08 022.

Note 180: Wind reporting standards: Speed Direction No observation Missing Missing Calm 0 0 Normal observation > 0 1° - 360° Speed only > 0 Missing Direction only Missing 1°-360° "Light and variable" > 0 0

Note 181: Wind waves and waves reporting standards:Observation Speed Direction No observation Missing Missing Calm 0 0 Normal observation >0 1-360 Speed only >0 Missing Direction only Missing 1-360"Light and variable" >0 0