Class 40: Satellite data

| FXY | ELEMENT NAME | BUFR UNIT | SCALE | REF VALUE | BITS | CREX UNIT | SCALE | CHARS | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 40 001 | Surface soil moisture (ms) | % | 1 | 0 | 10 | % | 1 | 4 |  |
| 0 40 002 | Estimated error in surface soil moisture | % | 1 | 0 | 10 | % | 1 | 4 |  |
| 0 40 003 | Mean surface soil moisture | Numeric | 3 | 0 | 10 | Numeric | 3 | 4 |  |
| 0 40 004 | Rain fall detection | Numeric | 3 | 0 | 10 | Numeric | 3 | 4 |  |
| 0 40 005 | Soil moisture correction flag | Flag table | 0 | 0 | 8 | Flag table | 0 | 3 |  |
| 0 40 006 | Soil moisture processing flag | Flag table | 0 | 0 | 16 | Flag table | 0 | 6 |  |
| 0 40 007 | Soil moisture quality | % | 1 | 0 | 10 | % | 1 | 4 |  |
| 0 40 008 | Frozen land surface fraction | % | 1 | 0 | 10 | % | 1 | 4 |  |
| 0 40 009 | Inundation and wetland fraction | % | 1 | 0 | 10 | % | 1 | 4 |  |
| 0 40 010 | Topographic complexity | % | 1 | 0 | 10 | % | 1 | 4 |  |
| 0 40 011 | Interpolation flag | Flag table | 0 | 0 | 8 | Flag table | 0 | 3 |  |
| 0 40 012 | Radiometer data quality flag | Flag table | 0 | 0 | 8 | Flag table | 0 | 3 |  |
| 0 40 013 | Radiometer brightness temperature interpretation flag | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 40 014 | High-frequency fluctuations of the sea-surface topography correction | m | 4 | -3000 | 13 | m | 4 | 4 |  |
| 0 40 015 | Normalized differential vegetation index (NDVI) | Numeric | 2 | -100 | 8 | Numeric | 2 | 3 |  |
| 0 40 016 | Residual RMS in band | Numeric | 3 | 0 | 14 | Numeric | 3 | 5 |  |
| 0 40 017 | Non-normalized principal component score | Numeric | 0 | -1073741824 | 31 | Numeric | 0 | 10 |  |
| 0 40 018 | GIacAvgImagIIS - average of imager measurements | W m-2 sr-1 m | 6 | 0 | 24 | W m-2 sr-1 m | 6 | 8 |  |
| 0 40 019 | GIacVarImagIIS - variance of imager measurements | W m-2 sr-1 m | 6 | 0 | 24 | W m-2 sr-1 m | 6 | 8 |  |
| 0 40 020 | GqisFlagQualDetailed - quality flag for the system | Flag table | 0 | 0 | 17 | Flag table | 0 | 6 |  |
| 0 40 021 | Fraction of weighted AVHRR pixel in IASI FOV covered with snow/ice | % | 0 | 0 | 7 | % | 0 | 3 |  |
| 0 40 022 | Number of missing, bad or failed AVHRR pixels | Numeric | 0 | 0 | 7 | Numeric | 0 | 3 |  |
| 0 40 023 | Auxiliary altimeter state flags | Flag table | 0 | 0 | 5 | Flag table | 0 | 2 |  |
| 0 40 024 | Meteorological map availability | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 40 025 | Interpolation flag for mean diurnal tide | Code table | 0 | 0 | 2 | Code table | 0 | 1 |  |
| 0 40 026 | Score quantization factor | Numeric | 2 | 0 | 16 | Numeric | 2 | 5 |  |
| 0 40 027 | Sun glint angle | deg | 2 | -18000 | 16 | deg | 2 | 5 |  |
| 0 40 028 | GMI quality flag | Code table | 0 | 0 | 4 | Code table | 0 | 2 |  |
| 0 40 029 | Horizontal observation integration length | m | 0 | 0 | 26 | m | 0 | 8 |  |
| 0 40 030 | Horizontal line of sight wind | m/s | 2 | -32767 | 16 | m/s | 2 | 5 |  |
| 0 40 031 | Error estimate of horizontal line of sight wind | m/s | 2 | 0 | 15 | m/s | 2 | 5 |  |
| 0 40 032 | Derivative wind to pressure | m s-1 Pa-1 | 3 | -100000 | 18 | m s-1 Pa-1 | 3 | 6 |  |
| 0 40 033 | Derivative wind to temperature | m s-1 K-1 | 3 | -100000 | 18 | m s-1 K-1 | 3 | 6 |  |
| 0 40 034 | Derivative wind to backscatter ratio | m/s | 3 | -200000 | 19 | m/s | 3 | 6 |  |
| 0 40 035 | Satellite range | m | 0 | 380000 | 18 | m | 0 | 6 |  |
| 0 40 036 | Lidar L2b classification type | Code table | 0 | 0 | 4 | Code table | 0 | 2 |  |
| 0 40 037 | Backscatter ratio | Numeric | 3 | 500 | 20 | Numeric | 3 | 7 |  |
| 0 40 038 | Cloud particle size | m | 7 | 0 | 28 | m | 7 | 6 |  |
| 0 40 039 | Single look complex image intensity | Numeric | 0 | -25 | 5 | Numeric | 0 | 3 |  |
| 0 40 040 | Single look complex image skewness | Numeric | 2 | 1 | 13 | Numeric | 0 | 4 |  |
| 0 40 041 | Single look complex image kurtosis | Numeric | 2 | 1 | 13 | Numeric | 0 | 4 |  |
| 0 40 042 | Single look complex image variance | Numeric | 2 | 1 | 13 | Numeric | 0 | 4 |  |
| 0 40 043 | Satellite manoeuvre indicator | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 40 044 | Dust index | Numeric | 1 | 0 | 8 | Numeric | 1 | 3 |  |
| 0 40 045 | Cloud formation and height assignment | Flag table | 0 | 0 | 5 | Flag table | 0 | 2 |  |
| 0 40 046 | Cloudiness summary | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 40 047 | Validation flag for IASI or IASI-NG level 1 product | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 40 048 | Validation flag of AMSU-A level 1 data flow | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 40 049 | Cloud tests executed and results | Flag table | 0 | 0 | 16 | Flag table | 0 | 5 |  |
| 0 40 050 | Retrieval initialisation | Flag table | 0 | 0 | 8 | Flag table | 0 | 3 |  |
| 0 40 051 | Convergence of the iterative retrieval | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 40 052 | Indication of super-adiabatic and super-saturation in final retrieval | Flag table | 0 | 0 | 8 | Flag table | 0 | 3 |  |
| 0 40 053 | Number of iterations used for retrieval | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 |  |
| 0 40 054 | Potential processing and inputs errors | Flag table | 0 | 0 | 13 | Flag table | 0 | 4 |  |
| 0 40 055 | Diagnostics on the retrieval | Flag table | 0 | 0 | 21 | Flag table | 0 | 7 |  |
| 0 40 056 | General retrieval quality | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 40 057 | IASI level 2 retrieval flags | Flag table | 0 | 0 | 31 | Flag table | 0 | 10 |  |
| 0 40 058 | Number of vectors describing the characterization matrices | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 |  |
| 0 40 059 | Number of layers actually retrieved | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 |  |
| 0 40 060 | Number of profiles retrieved in scanline | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 |  |
| 0 40 061 | Air partial columns on each retrieved layer | mol cm-2 | 3 | 0 | 16 | mol cm-2 | 3 | 5 |  |
| 0 40 062 | A priori partial columns on each retrieved layer | mol cm-2 | 10 | 0 | 16 | mol cm-2 | 10 | 5 |  |
| 0 40 063 | Scaling vector multiplying the a priori vector in order to define the retrieved vector | Numeric | 5 | 0 | 26 | Numeric | 5 | 8 |  |
| 0 40 064 | Main eigenvalues of the sensitivity matrix | Numeric | 6 | 0 | 31 | Numeric | 6 | 10 |  |
| 0 40 065 | Main eigenvectors of the sensitivity matrix | Numeric | 6 | -1000000000 | 31 | Numeric | 6 | 10 |  |
| 0 40 066 | Quality indicator for atmospheric water vapour | Numeric | 1 | 0 | 8 | Numeric | 1 | 3 |  |
| 0 40 067 | Quality indicator for atmospheric temperature | Numeric | 1 | 0 | 8 | Numeric | 1 | 3 |  |
| 0 40 068 | General retrieval quality flag for SO2 | Code table | 0 | 0 | 4 | Code table | 0 | 2 |  |
| 0 40 069 | PWLR estimated retrieval error for surface air temperature | K | 4 | -1000000 | 21 | K | 4 | 7 | Note 114: Piece-wise linear regression cube is a first-guess optimal estimation method. |
| 0 40 070 | PWLR estimated retrieval error of surface dewpoint | K | 4 | -1000000 | 21 | K | 4 | 7 |  |
| 0 40 071 | Retrieval error covariance matrix for ozone in principal component domain | Numeric | 4 | -1000000 | 21 | Numeric | 4 | 7 |  |
| 0 40 072 | PWLR estimated retrieval quality indicator of atmospheric ozone | Numeric | 1 | 0 | 8 | Numeric | 1 | 3 |  |
| 0 40 073 | PWLR estimated retrieval error of surface skin temperature | K | 1 | 0 | 8 | K | 1 | 3 |  |
| 0 40 074 | General interferometry quality flags | Flag table | 0 | 0 | 16 |  |  |  |  |
| 0 40 075 | Retrieval error covariance matrix for humidity in principal component domain | Numeric | 0 | 0 | 21 | Numeric | 0 | 7 |  |
| 0 40 076 | Retrieval error covariance matrix for temperature in principal component domain | Numeric | 0 | 0 | 21 | Numeric | 0 | 7 |  |
| 0 40 077 | Ocean product quality flag | Flag table | 0 | 0 | 8 | Flag table | 0 | 3 |  |
| 0 40 080 | Cloud test results for IASI-NG level 2 | Flag table | 0 | 0 | 5 | Flag table | 0 | 2 |  |
| 0 40 081 | OLCI Level 1 classification and quality | Flag table | 0 | 0 | 31 |  |  |  |  |

Note 23: A precipitation value of -0.1 kg m-2 before scaling (-1 after scaling or in CREX) shall indicate a "trace" (non-measurable, less than 0.05 kg m-2).

Note 25: A previously defined significance may be cancelled by transmitting a "missing" from the appropriate code or flag table.

Note 27: All times are Universal Time Coordinated (UTC) unless otherwise noted.

Note 32: Bearing or azimuth shall only be used with respect to a stated location, and shall not redefine that location.

Note 62: Distance shall only be used with respect to a stated location and a bearing, azimuth or elevation; it shall not redefine that location.

Note 63: Downward radiation shall be assigned positive values.

Note 64: Downward w-components shall be assigned positive values where units are Pa s-1.

Note 66: East to west increments shall be assigned negative values.

Note 75: For CREX descriptors F = B, not 0.

Note 92: In order to distinguish unambiguously the cases of missing data and saturated pixels, n-bit image data should be encoded using a data width of n+1. Where such a descriptor is not already available in Class 30, operator descriptor 2 01 YYY should be used to modify the data width of the existing entry as required.

Note 105: Negative time periods or displacements shall be used to indicate time periods or displacements preceding the currently defined time.

Note 110: North to south increments shall be assigned negative values.

Note 115: Pixel data width can be changed with descriptor 2 01 YYY.

Note 121: Some indication of expected accuracy may be implied in conjunction with certain elements in this class.

Note 122: South latitude shall be assigned negative values.

Note 123: South to north v-components shall be assigned positive values.

Note 126: Surface wind direction measured at a station within 1° of the North Pole or within 1° of the South Pole shall be reported in such a way that the azimuth ring shall be aligned with its zero coinciding with the Greenwich 0° meridian.

Note 140: The pixel size on horizontal - 1 is given at location where map scale factor is unity.

Note 141: The pixel size on horizontal - 2 is given at location where map scale factor is unity.

Note 145: The significance of time periods or displacements may be indicated using the time significance code corresponding to table reference 0 08 021.

Note 149: The time location, when used with forecast values, shall indicate the time of the initial state for the forecast, or the beginning of the forecast period; when used with ensemble means of forecast values, the time location shall indicate the initial state or the beginning of the first forecast over which ensemble means are derived.

Note 155: This class may also contain elements relating to observational procedures.

Note 156: This class shall contain elements to describe the instrumentation used to obtain the meteorological elements reported.

Note 158: Time periods or displacements and time increments require an initial time location to be defined prior to their use, followed where appropriate by a time significance definition.

Note 160: Upward radiation shall be assigned negative values.

Note 161: Upward w-components shall be assigned positive values where units are m s-1.

Note 162: Useful ranges used above:1011 Bq to 1019 Bq for releases;10-2 Bq to 107 Bq and 10-2 mSv to 107 mSv for concentration and doses.

Note 163: Values of latitude and latitude increments are limited to the range -90 degrees to +90 degrees.

Note 164: Values of longitude are limited to the range -180 degrees to +180 degrees.

Note 165: Vertical elements and pressure shall be used to define values of these elements independent of the element or variable denoting the vertical coordinate.

Note 166: West longitude shall be assigned negative values.

Note 167: West to east u-components shall be assigned positive values.

Note 176: Where more than one time period or displacement is required to define complex time structures, they shall be defined in immediate succession, and the following ordering shall apply: ensemble period (if required), followed by forecast period (if required), followed by period for averaging or accumulation (if required).

Note 178: Where the expression "period specified" is entered under element name, an appropriate period shall be specified using descriptors from Class 04.

Note 179: Where values are accumulated or averaged (for example over a time period), the total number of values from which the accumulated or averaged values are obtained may be represented using reference 0 08 022.

Note 180: Wind reporting standards: Speed Direction No observation Missing Missing Calm 0 0 Normal observation > 0 1° - 360° Speed only > 0 Missing Direction only Missing 1°-360° "Light and variable" > 0 0

Note 181: Wind waves and waves reporting standards:Observation Speed Direction No observation Missing Missing Calm 0 0 Normal observation >0 1-360 Speed only >0 Missing Direction only Missing 1-360"Light and variable" >0 0