Category 08: Surface report sequences (sea)

3 08 001 (Buoy/platform - fixed)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 033 | Buoy/platform - fixed | Identification, type, date/time, position (high accuracy) |  |
| 3 02 011 | Low altitude station | Basic surface report |  |
| 0 22 042 | Sea/water temperature |  |  |

3 08 002 (Buoy/platform - fixed)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 034 | Buoy/platform - fixed | Identification, type, date/time, position (coarse accuracy) |  |
| 3 02 011 | Low altitude station | Basic surface report |  |
| 0 22 042 | Sea/water temperature |  |  |

3 08 003 (Buoy/platform - moving)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 035 | Buoy/platform - moving | Identification, movement, type, date/time, position (coarse accuracy) | Note 13: Descriptor 3 08 007 should be used instead of 3 08 003 to encode moving buoy/platform information. |
| 3 02 011 | Low altitude station | Basic surface report | Note 13: Descriptor 3 08 007 should be used instead of 3 08 003 to encode moving buoy/platform information. |
| 0 22 042 | Sea/water temperature |  | Note 13: Descriptor 3 08 007 should be used instead of 3 08 003 to encode moving buoy/platform information. |

3 08 004 (Ship)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 036 | Ship | Identification, movement, type, date/time, position (coarse accuracy) |  |
| 3 02 011 | Low altitude station | Basic surface report |  |
| 0 22 042 | Sea/water temperature |  |  |

3 08 005

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 08 004 | Ship | Basic ship report |  |
| 3 02 024 | Wind and swell waves |  |  |

3 08 006 (Buoy Section 1 optional parameters)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 10 004 | Pressure |  |  |
| 0 10 061 | 3-hour pressure change |  |  |
| 0 10 063 | Characteristic of pressure tendency |  |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 12 004 | Air temperature at 2 m |  |  |
| 0 13 003 | Relative humidity |  |  |
| 0 22 042 | Sea/water temperature |  |  |

3 08 007

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 055 | Identification and type of station, date/time, location (high accuracy), movement |  |  |
| 3 02 011 | Low altitude station | Basic surface report |  |
| 0 07 062 | Depth below sea/water surface |  |  |
| 0 22 042 | Sea/water temperature |  |  |

3 08 009 (Sequence for representation of synoptic reports from a sea station suitable for SHIP data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 093 | Ship identification, movement, date/time, horizontal and vertical coordinates |  |  |
| 3 02 001 | Pressure and 3-hour pressure change |  |  |
| 3 02 054 | Ship "instantaneous" data |  |  |
| 0 08 002 | Vertical significance (surface observations) |  |  |
| 3 02 055 | Icing and ice |  |  |
| 3 02 057 | Ship marine data |  |  |
| 3 02 060 | Ship "period" data |  |  |

3 08 010 (TRACKOB template)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 01 011 | Ship or mobile land station identifier |  |  |
| 1 13 000 | Delayed replication of 13 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 01 011 | Year, month, day |  |  |
| 3 01 012 | Hour, minute |  |  |
| 3 01 021 | Latitude/longitude (high accuracy) |  |  |
| 0 04 080 | Averaging period for following value |  |  |
| 0 22 049 | Sea-surface temperature |  |  |
| 0 04 080 | Averaging period for following value |  |  |
| 0 22 059 | Sea-surface salinity |  |  |
| 0 04 080 | Averaging period for following value |  |  |
| 0 22 005 | Direction of sea-surface current |  |  |
| 0 02 042 | Indicator for sea-surface current speed |  |  |
| 0 22 032 | Speed of sea-surface current |  |  |
| 0 02 042 | Indicator for sea-surface current speed | Cancel |  |
| 0 04 080 | Averaging period for following value | Cancel |  |

3 08 011 (Monthly values from an ocean weather station - CLIMAT SHIP)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 01 011 | Ship or mobile land station identifier | Ship's call sign |  |
| 0 02 001 | Type of station |  |  |
| 3 01 011 | Year, month, day |  | Note 59: The time identification refers to the beginning of the one-month period. |
| 3 01 012 | Hour, minute |  | Note 59: The time identification refers to the beginning of the one-month period. |
| 3 01 023 | Latitude/longitude (coarse accuracy) |  |  |
| 0 07 030 | Height of station ground above mean sea level |  | Note 25: If the height of the sensor was changed during the period specified, the value shall be that which existed for the greater part of the period. |
| 0 07 031 | Height of barometer above mean sea level |  | Note 25: If the height of the sensor was changed during the period specified, the value shall be that which existed for the greater part of the period. |
| 0 04 074 | Short time period or displacement | = UTC - LT | Note 59: The time identification refers to the beginning of the one-month period. |
| 0 04 023 | Time period or displacement | = Number of days in the month |  |
| 0 08 023 | First-order statistics | = 4 Mean value |  |
| 0 10 051 | Pressure reduced to mean sea level |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Temperature measurement | Note 25: If the height of the sensor was changed during the period specified, the value shall be that which existed for the greater part of the period. |
| 0 07 033 | Height of sensor above water surface | Temperature measurement | Note 25: If the height of the sensor was changed during the period specified, the value shall be that which existed for the greater part of the period. |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 13 004 | Vapour pressure |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 0 07 033 | Height of sensor above water surface | Set to missing (cancel) |  |
| 3 02 056 | Sea/water temperature | Sea-surface temperature, method of measurement, and depth below sea surface |  |
| 0 08 023 | First-order statistics | Set to missing |  |
| 0 04 003 | Day | = 1 | Note 27: In case of precipitation measurements, the one-month period begins at 06 UTC on the first day of the month and ends at 06 UTC on the first day of the following month. |
| 0 04 004 | Hour | = 6 | Note 27: In case of precipitation measurements, the one-month period begins at 06 UTC on the first day of the month and ends at 06 UTC on the first day of the following month. |
| 0 04 023 | Time period or displacement | = Number of days in the month | Note 27: In case of precipitation measurements, the one-month period begins at 06 UTC on the first day of the month and ends at 06 UTC on the first day of the following month. |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  | Note 25: If the height of the sensor was changed during the period specified, the value shall be that which existed for the greater part of the period. |
| 0 13 060 | Total accumulated precipitation |  |  |
| 0 13 051 | Frequency group, precipitation |  |  |
| 0 04 053 | Number of days with precipitation equal to or more than 1 mm |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |

3 08 012 (Monthly normals for an ocean weather station)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 04 001 | Year | Beginning of the reference period |  |
| 0 04 001 | Year | Ending of the reference period |  |
| 0 04 002 | Month |  |  |
| 0 04 003 | Day | = 1 | Note 59: The time identification refers to the beginning of the one-month period. |
| 0 04 004 | Hour | = 0 | Note 59: The time identification refers to the beginning of the one-month period. |
| 0 04 074 | Short time period or displacement | = UTC - LT | Note 59: The time identification refers to the beginning of the one-month period. |
| 0 04 022 | Time period or displacement | = 1 |  |
| 0 08 023 | First-order statistics | = 4 Mean value |  |
| 0 10 051 | Pressure reduced to mean sea level |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Temperature measurement | Note 25: If the height of the sensor was changed during the period specified, the value shall be that which existed for the greater part of the period. |
| 0 07 033 | Height of sensor above water surface | Temperature measurement | Note 25: If the height of the sensor was changed during the period specified, the value shall be that which existed for the greater part of the period. |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 13 004 | Vapour pressure |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 0 07 033 | Height of sensor above water surface | Set to missing (cancel) |  |
| 3 02 056 | Sea/water temperature | Sea-surface temperature, method of measurement, and depth below sea surface |  |
| 0 08 023 | First-order statistics | Set to missing |  |
| 0 04 001 | Year | Beginning of the reference period |  |
| 0 04 001 | Year | Ending of the reference period |  |
| 0 04 002 | Month |  |  |
| 0 04 003 | Day | = 1 | Note 27: In case of precipitation measurements, the one-month period begins at 06 UTC on the first day of the month and ends at 06 UTC on the first day of the following month. |
| 0 04 004 | Hour | = 6 | Note 27: In case of precipitation measurements, the one-month period begins at 06 UTC on the first day of the month and ends at 06 UTC on the first day of the following month. |
| 0 04 022 | Time period or displacement | = 1 |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Precipitation measurement | Note 25: If the height of the sensor was changed during the period specified, the value shall be that which existed for the greater part of the period. |
| 0 08 023 | First-order statistics | = 4 Mean value |  |
| 0 13 060 | Total accumulated precipitation |  |  |
| 0 04 053 | Number of days with precipitation equal to or more than 1 mm |  |  |
| 0 08 023 | First-order statistics | Set to missing |  |

3 08 013 (Representation of CLIMAT SHIP data of the actual month and for monthly normals)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 08 011 | Monthly values from an ocean weather station - CLIMAT SHIP |  |  |
| 3 08 012 | Monthly normals for an ocean weather station |  |  |

3 08 014 (Synoptic reports from sea stations suitable for VOS observation data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 01 018 | Encrypted ship’s call sign and encryption method |  |  |
| 0 03 001 | Surface station type |  |  |
| 3 01 093 | Ship identification, movement, date/time, horizontal and vertical coordinates |  |  |
| 2 08 032 | Change width of CCITT IA5 field |  |  |
| 0 01 079 | Unique identifier for the profile | 32 characters ¦ Unique ID for report |  |
| 2 08 000 | Change width of CCITT IA5 | Cancel |  |
| 3 02 062 | Ship “instantaneous” data |  |  |
| 3 02 063 | Ship “period” data |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 092 | VOSClim data elements |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 06 033 | Surface salinity |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 06 034 | Surface current |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 06 043 | Marine biogeochemical and radiation observations | E-SURFMAR S-AWS observations |  |

3 08 015 (Template for WAVEOB data expressed as frequency (Ia = 0 in FM-65 WAVEOB)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 01 003 | WMO Region number/geographical area | A1 - First digit of WMO number (e.g. 62024 => 6) |  |
| 0 01 020 | WMO Region sub-area | bw - Second digit of WMO number (e.g. 62024 => 2) |  |
| 0 01 005 | Buoy/platform identifier | nbnbnb - Last 3 digits of WMO number (e.g. 62024 => 024) |  |
| 0 01 011 | Ship or mobile land station identifier | D....D |  |
| 0 01 007 | Satellite identifier | I6I6I6 |  |
| 0 01 001 | WMO block number | II |  |
| 0 01 002 | WMO station number | iii - IIiii only apply to fixed sea stations |  |
| 0 02 044 | Indicator for method of calculating spectral wave data | Im - Code table 1744 (WAVEOB), 0 02 044 (BUFR) |  |
| 0 02 045 | Indicator for type of platform | Ip - Code table 1747 (WAVEOB), 0 02 045 (BUFR) |  |
| 3 01 011 | Year, month, day | YYMMJ - Date of observation |  |
| 3 01 012 | Hour, minute | GGgg - Time of observation |  |
| 3 01 021 | Latitude/longitude (high accuracy) | QcLaLaLaLa LoLoLoLoLo |  |
| 0 22 063 | Total water depth | 1hhhh |  |
| 0 22 076 | Direction from which dominant waves are coming | 9dddd - Section 0 |  |
| 0 22 077 | Directional spread of dominant wave | dsds - section 0 |  |
| 0 22 094 | Total number of wave bands | 111BTBT - Section 1 |  |
| 0 25 043 | Wave sampling interval (time) | SSSS - Section 1 |  |
| 0 22 078 | Duration of wave record | D’D’D’D’ - Section 1 |  |
| 1 05 002 | Replicate 5 descriptors 2 times | Replicate over sensor type |  |
| 0 02 046 | Wave measurement instrumentation | = 1 Heave sensor, = 2 Slope sensor |  |
| 0 22 070 | Significant wave height | 2HsHsHsHs or 6HseHseHseHse - Section 0 |  |
| 0 22 071 | Spectral peak wave period | 3PpPpPpPp or 7PspPspPspPsp - Section 0 |  |
| 0 22 073 | Maximum wave height | 4HmHmHmHm |  |
| 0 22 074 | Average wave period | 5PaPaPaPa or 8PsaPsaPsaPsa - Section 0 |  |
| 1 27 000 | Delayed replication of 27 descriptors | Replication over sensor type |  |
| 0 31 001 | Delayed descriptor replication factor | (0,1,2) normally 1 | Note 41: Normally 1, may be 2 if both heave and slope sensors are in use, or 0 if no spectral data. |
| 0 02 046 | Wave measurement instrumentation | = 1 Heave sensor, = 2 Slope sensor |  |
| 0 08 090 | Decimal scale of following significands | x - Scale to be applied to following element descriptors |  |
| 0 22 102 | Scaled maximum non-directional spectral wave density by frequency | CmCmCm - Section 2 or CsmCsmCsm - Section 3 |  |
| 0 08 090 | Decimal scale of following significands | Set to missing |  |
| 0 22 084 | Band containing maximum non-directional spectral wave density | nmnm - Section 2 or nsmnsm - Section 3 |  |
| 1 20 000 | Delayed replication of 20 descriptors | Replication over bands |  |
| 0 31 001 | Delayed descriptor replication factor | 111BTBT - Section 1 (number of band) |  |
| 0 22 080 | Waveband central frequency | 1f1f1f1f1x .... - Section 1 |  |
| 0 22 108 | Spectral wave density ratio | 1c1c1c2c2 ... - Section 2 or 1cs1cs1cs2cs2 ... - Section 3 |  |
| 0 22 086 | Mean direction from which waves are coming | 1da1da1 - Section 4 |  |
| 0 22 087 | Principal direction from which waves are coming | da2da2 - Section 4 |  |
| 0 22 088 | First normalized polar coordinate from Fourier coefficients | r1r1 - Section 4 |  |
| 0 22 089 | Second normalized polar coordinate from Fourier coefficients | r2r2 - Section 4 |  |
| 1 05 000 | Delayed replication of 5 descriptors |  | Note 39: Non-directional spectra (Ib = 0 in WAVEOB) or partial directional spectra (Ib = 1 in WAVEOB with one direction per wave number). Count = 0 (full directional spectra) or 1 (non-directional spectra or partial directional spectra). Partial directional spectra have only one direction per wave number band. |
| 0 31 001 | Delayed descriptor replication factor | n - Section 5 (number of directions counted); = 0 if Ib = 1 (directional) |  |
| 0 08 090 | Decimal scale of following significands | x - Scale to be applied to following element descriptors |  |
| 0 22 104 | Scaled non-directional spectral wave density by frequency | 1A1A1A1x ... - Section 5 |  |
| 0 08 090 | Decimal scale of following significands | Set to missing |  |
| 0 22 186 | Direction from which waves are coming | 1d1d1 - Section 5 | Note 37: Missing for non-directional spectra. |
| 0 22 187 | Directional spread of wave | dsds - Section 5 | Note 37: Missing for non-directional spectra. |
| 1 05 000 | Delayed replication of 5 descriptors |  | Note 22: Full directional spectra (Ib = 1 in WAVEOB with more than one direction per wave number band). The replication count is the number of directions per wave number band that should normally cover the full circle. |
| 0 31 001 | Delayed descriptor replication factor | n - Section 5 (number of directions counted); = 0 if Ib = 0 (non-directional) |  |
| 0 08 090 | Decimal scale of following significands | x - Scale to be applied to following element descriptors |  |
| 0 22 106 | Scaled directional spectral wave density by frequency | 1A1A1A1x ... - Section 5 |  |
| 0 08 090 | Decimal scale of following significands | Set to missing |  |
| 0 22 186 | Direction from which waves are coming | 1d1d1 - Section 5 |  |
| 0 22 187 | Directional spread of wave | dsds - Section 5 |  |

3 08 016 (Template for WAVEOB data expressed as the wave number (Ia = 1 in FM-65 WAVEOB)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 01 003 | WMO Region number/geographical area | A1 - First digit of WMO number (e.g. 62024 => 6) |  |
| 0 01 020 | WMO Region sub-area | bw - Second digit of WMO number (e.g. 62024 => 2) |  |
| 0 01 005 | Buoy/platform identifier | nbnbnb - Last 3 digits of WMO number (e.g. 62024 => 024) |  |
| 0 01 011 | Ship or mobile land station identifier | D....D |  |
| 0 01 007 | Satellite identifier | I6I6I6 |  |
| 0 01 001 | WMO block number | II |  |
| 0 01 002 | WMO station number | iii - IIiii only apply to fixed sea stations |  |
| 0 02 044 | Indicator for method of calculating spectral wave data | Im - Code table 1744 (WAVEOB), 0 02 044 (BUFR) |  |
| 0 02 045 | Indicator for type of platform | Ip - Code table 1747 (WAVEOB), 0 02 045 (BUFR) |  |
| 3 01 011 | Year, month, day | YYMMJ - Date of observation |  |
| 3 01 012 | Hour, minute | GGgg - Time of observation |  |
| 3 01 021 | Latitude/longitude (high accuracy) | QcLaLaLaLa LoLoLoLoLo |  |
| 0 22 063 | Total water depth | 1hhhh |  |
| 0 22 076 | Direction from which dominant waves are coming | 9dddd - Section 0 |  |
| 0 22 077 | Directional spread of dominant wave | dsds - Section 0 |  |
| 0 22 094 | Total number of wave bands | 111BTBT - Section 1 |  |
| 0 25 044 | Wave sampling interval (space) | SSSS - Section 1 |  |
| 0 22 079 | Length of wave record | D’D’D’D’ - Section 1 |  |
| 1 05 002 | Replicate 5 descriptors 2 times | Replication over sensor type |  |
| 0 02 046 | Wave measurement instrumentation | = 1 Heave sensor, = 2 Slope sensor |  |
| 0 22 070 | Significant wave height | 2HsHsHsHs or 6HseHseHseHse - Section 0 |  |
| 0 22 072 | Spectral peak wavelength | 3PpPpPpPp - Section 0 |  |
| 0 22 073 | Maximum wave height | 4HmHmHmHm |  |
| 0 22 075 | Average wavelength | 5PaPaPaPa - Section 0 |  |
| 1 27 000 | Delayed replication of 27 descriptors | Replication over sensor type |  |
| 0 31 001 | Delayed descriptor replication factor | (0,1,2) normally 1 | Note 41: Normally 1, may be 2 if both heave and slope sensors are in use, or 0 if no spectral data. |
| 0 02 046 | Wave measurement instrumentation | = 1 Heave sensor, = 2 Slope sensor |  |
| 0 08 090 | Decimal scale of following significands | x - Scale to be applied to following element descriptors |  |
| 0 22 103 | Scaled maximum non-directional spectral wave density by wave number | CmCmCm - Section 2 or CsmCsmCsm - Section 3 |  |
| 0 08 090 | Decimal scale of following significands | Set to missing |  |
| 0 22 084 | Band containing maximum non-directional spectral wave density | nmnm - Section 2 or nsmnsm - Section 3 |  |
| 1 20 000 | Delayed replication of 20 descriptors | Replication over band |  |
| 0 31 001 | Delayed descriptor replication factor | 111BTBT - Section 1 (number of band) |  |
| 0 22 081 | Waveband central wave number | 1f1f1f1f1x ... - Section 1 |  |
| 0 22 108 | Spectral wave density ratio | 1c1c1c2c2 ... - Section 2 or 1cs1cs1cs2cs2 ... - Section 3 |  |
| 0 22 086 | Mean direction from which waves are coming | 1da1da1 - Section 4 |  |
| 0 22 087 | Principal direction from which waves are coming | da2da2 - Section 4 |  |
| 0 22 088 | First normalized polar coordinate from Fourier coefficients | r1r1 - Section 4 |  |
| 0 22 089 | Second normalized polar coordinate from Fourier coefficients | r2r2 - Section 4 |  |
| 1 05 000 | Delayed replication of 5 descriptors |  | Note 39: Non-directional spectra (Ib = 0 in WAVEOB) or partial directional spectra (Ib = 1 in WAVEOB with one direction per wave number). Count = 0 (full directional spectra) or 1 (non-directional spectra or partial directional spectra). Partial directional spectra have only one direction per wave number band. |
| 0 31 001 | Delayed descriptor replication factor | n - Section 5 (number of directions counted); = 0 if Ib = 1 (directional) |  |
| 0 08 090 | Decimal scale of following significands | x - Scale to be applied to following element descriptors |  |
| 0 22 105 | Scaled non-directional spectral wave density by wave number | 1A1A1A1x ... - Section 5 |  |
| 0 08 090 | Decimal scale of following significands | Set to missing |  |
| 0 22 186 | Direction from which waves are coming | 1d1d1 - Section 5 | Note 37: Missing for non-directional spectra. |
| 0 22 187 | Directional spread of wave | dsds - Section 5 | Note 37: Missing for non-directional spectra. |
| 1 05 000 | Delayed replication of 5 descriptors |  | Note 22: Full directional spectra (Ib = 1 in WAVEOB with more than one direction per wave number band). The replication count is the number of directions per wave number band that should normally cover the full circle. |
| 0 31 001 | Delayed descriptor replication factor | n - Section 5 (number of directions counted); = 0 if Ib = 0 (non-directional) |  |
| 0 08 090 | Decimal scale of following significands | x - Scale to be applied to following element descriptors |  |
| 0 22 107 | Scaled directional spectral wave density by wave number | 1A1A1A1x ... - Section 5 |  |
| 0 08 090 | Decimal scale of following significands | Set to missing |  |
| 0 22 186 | Direction from which waves are coming | 1d1d1 - Section 5 |  |
| 0 22 187 | Directional spread of wave | dsds - Section 5 |  |

3 08 017 (Sequence for reporting observations from offshore platforms)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 056 | Sequence for platform identification, type, time and location of the observation report |  |  |
| 3 02 001 | Pressure and 3-hour pressure change |  |  |
| 3 02 052 | Ship temperature and humidity data |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 056 | Sea/water temperature | Optional |  |
| 3 02 064 | Ship or other marine platform wind data |  |  |
| 3 02 053 | Ship visibility data |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 004 | General cloud information | Optional |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 005 | Cloud layer | Optional |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 038 | Present and past weather | Optional |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 06 039 | Sequence for representation of basic wave measurements | Optional |  |

3 08 018 (Sequence for reporting of basic ship AWS observations)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 150 | WIGOS identifier |  |  |
| 3 01 093 | Ship identification, movement, date/time, horizontal and vertical coordinates |  |  |
| 3 02 001 | Pressure and 3-hour pressure change |  |  |
| 3 02 072 | Temperature and humidity data |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 056 | Sea/water temperature |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 064 | Ship or other marine platform wind data |  |  |

3 08 021 (Monthly values from an ocean weather station in compliance with regional or national reporting practices)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 01 011 | Ship or mobile land station identifier | Ship’s call sign |  |
| 0 02 001 | Type of station |  |  |
| 3 01 011 | Year, month, day |  |  |
| 3 01 012 | Hour, minute |  |  |
| 3 01 023 | Latitude/longitude (coarse accuracy) |  |  |
| 0 07 030 | Height of station ground above mean sea level |  |  |
| 0 07 031 | Height of barometer above mean sea level |  |  |
| 0 04 074 | Short time period or displacement |  |  |
| 0 04 023 | Time period or displacement | Number of days in the month |  |
| 0 08 023 | First-order statistics | = 4 Mean value |  |
| 0 10 051 | Pressure reduced to mean sea level |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Temperature measurement |  |
| 0 07 033 | Height of sensor above water surface | Temperature measurement |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 13 004 | Vapour pressure |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 0 07 033 | Height of sensor above water surface | Set to missing (cancel) |  |
| 3 02 056 | Sea/water temperature | Sea-surface temperature, method of measurement and depth below sea surface |  |
| 0 08 023 | First-order statistics | Set to missing (cancel) |  |
| 0 04 003 | Day | = 1 |  |
| 0 04 004 | Hour | = 0 |  |
| 0 04 074 | Short time period or displacement |  |  |
| 0 04 023 | Time period or displacement | Number of days in the month |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 13 060 | Total accumulated precipitation |  |  |
| 0 13 051 | Frequency group, precipitation |  |  |
| 0 04 053 | Number of days with precipitation equal to or more than 1 mm |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |

3 08 022 (Monthly normals for an ocean weather station in compliance with regional or national reporting practices)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 04 001 | Year | Beginning of the reference period |  |
| 0 04 001 | Year | Ending of the reference period |  |
| 0 04 002 | Month |  |  |
| 0 04 003 | Day | = 1 |  |
| 0 04 004 | Hour | = 0 |  |
| 0 04 074 | Short time period or displacement |  |  |
| 0 04 022 | Time period or displacement | = 1 |  |
| 0 08 023 | First-order statistics | = 4 Mean value |  |
| 0 10 051 | Pressure reduced to mean sea level |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Temperature measurement |  |
| 0 07 033 | Height of sensor above water surface | Temperature measurement |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 13 004 | Vapour pressure |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 0 07 033 | Height of sensor above water surface | Set to missing (cancel) |  |
| 3 02 056 | Sea/water temperature | Sea-surface temperature, method of measurement and depth below sea surface |  |
| 0 08 023 | First-order statistics | Set to missing (cancel) |  |
| 0 04 001 | Year | Beginning of the reference period |  |
| 0 04 001 | Year | Ending of the reference period |  |
| 0 04 002 | Month |  |  |
| 0 04 003 | Day | = 1 |  |
| 0 04 004 | Hour | = 0 |  |
| 0 04 074 | Short time period or displacement |  |  |
| 0 04 022 | Time period or displacement | = 1 |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Precipitation measurement |  |
| 0 08 023 | First-order statistics | = 4 Mean value |  |
| 0 13 060 | Total accumulated precipitation |  |  |
| 0 04 053 | Number of days with precipitation equal to or more than 1 mm |  |  |
| 0 08 023 | First-order statistics | Set to missing (cancel) |  |

3 08 023 (Sequence for representation of monthly values suitable for CLIMAT SHIP data in compliance with regional or national reporting practices)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 08 021 | Monthly values from an ocean weather station in compliance with regional or national reporting practices |  |  |
| 3 08 022 | Monthly normals for an ocean weather station in compliance with regional or national reporting practices |  |  |

Note 4: Additional descriptors are required to reduce the workload with respect to the station database, e.g. for identification of the federal state and identification of the highway.

Note 7: Compound BUFR messages may be described within the data description section, if required (e.g. 3 01 041, 3 04 001, 3 04 002, 3 04 003, 3 04 004, 3 04 005, 3 04 006).

Note 10: Decibel (dB) is a logarithmic measure of the relative power, or of the relative values of two flux densities, especially of sound intensities and radio and radar power densities. In radar meteorology, the logarithmic scale (dBZ) is used for measuring radar reflectivity factor (obtained from the American Meteorological Society Glossary of Meteorology).

Note 15: Descriptors 3 03 021 to 3 03 027 are not available in CREX.

Note 16: Each BUFR message may contain data for a number of locations; the BUFR compression technique involves negligible overheads for data items that are invariant.

Note 32: It could be argued that, as only additions are possible, only complete lines should be allowed; but it is conceivable that local areas will require changes as well as additions, so it is better and in any case clearer to provide descriptions for all the fields.

Note 33: It is better to use different Class 00 descriptors for the defining and defined elements, in the same way as different descriptors correspond to pressure considered as a coordinate and pressure measured at a given point; otherwise special rules would be needed to interpret such message. Entries 0 00 010 to 0 00 012 define F, X and Y for Tables B and D; entry 0 00 030 is a descriptor used as data and provides the F, X and Y values defining a sequence for Table D entries.

Note 34: Latitude displacement 0 05 015 in the sequence represents the latitude offset from the latitude of the launch site. Longitude displacement 0 06 015 in the sequence represents the longitude offset from the longitude of the launch site.

Note 35: Long time displacement 0 04 086 in the sequence represents the time offset from the launch time 3 01 013 (in seconds).

Note 43: Separation of single level satellite data into sets of BUFR messages helps compression and results inefficient data transmission and storage.

Note 49: The majority of stations has only one position on the road and one subsurface temperature sensor. Delayed Replications have been introduced to increase flexibility and volume efficiency.

Note 62: These descriptors should not be used in CREX for transmission.

Note 63: These entries include the facility to update the Table A code figure and data description.