Category 09: Vertical sounding sequences (conventional data)

3 09 001 (Vertical wind profile)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 037 | Land station for vertical soundings | Identification, etc. (land station, high accuracy position) |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 03 011 | Wind at height |  |  |

3 09 002 (Vertical wind profile)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 038 | Land station for vertical soundings | Identification, etc. (land station, coarse accuracy position) |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 03 011 | Wind at height |  |  |

3 09 003 (Vertical wind profile)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 037 | Land station for vertical soundings | Identification, etc. (land station, high accuracy position) |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 03 012 | Wind at pressure level |  |  |

3 09 004 (Vertical wind profile)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 038 | Land station for vertical soundings | Identification, etc. (land station, coarse accuracy position) |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 03 012 | Wind at pressure level |  |  |

3 09 005 (Vertical sounding with relative humidity)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 037 | Land station for vertical soundings | Identification, etc. (land station, high accuracy position) |  |
| 3 02 004 | General cloud information | Significant cloud layer |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 03 013 | Geopotential, temperature, humidity, wind at pressure level |  |  |

3 09 006 (Vertical sounding with relative humidity)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 038 | Land station for vertical soundings | Identification, etc. (land station, coarse accuracy position) |  |
| 3 02 004 | General cloud information | Significant cloud layer |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 03 013 | Geopotential, temperature, humidity, wind at pressure level |  |  |

3 09 007 (Vertical sounding with dewpoint data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 037 | Land station for vertical soundings | Identification, etc. (land station, high accuracy position) |  |
| 3 02 004 | General cloud information | Significant cloud layer |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 03 014 | Geopotential, temperature, dewpoint temperature, wind at pressure level |  |  |

3 09 008 (Vertical sounding with dewpoint data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 038 | Land station for vertical soundings | Identification, etc. (land station, coarse accuracy position) |  |
| 3 02 004 | General cloud information | Significant cloud layer |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 03 014 | Geopotential, temperature, dewpoint temperature, wind at pressure level |  |  |

3 09 011 (Vertical wind profile)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 039 | Ship for vertical soundings | Ship's identification, etc. |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 03 011 | Wind at height |  |  |

3 09 012 (Vertical wind profile)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 039 | Ship for vertical soundings | Ship's identification, etc. |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 03 012 | Wind at pressure level |  |  |

3 09 013 (Vertical sounding with relative humidity)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 039 | Ship for vertical soundings | Ship's identification, etc. |  |
| 3 02 004 | General cloud information | Significant cloud layer |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 03 013 | Geopotential, temperature, humidity, wind at pressure level |  |  |

3 09 014 (Vertical sounding with dewpoint data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 039 | Ship for vertical soundings | Ship's identification, etc. |  |
| 3 02 004 | General cloud information | Significant cloud layer |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 03 014 | Geopotential, temperature, dewpoint temperature, wind at pressure level |  |  |

3 09 015 (Vertical wind profile)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 040 | Ship for vertical soundings | Ship's identification, etc. |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 03 011 | Wind at height |  |  |

3 09 016 (Vertical wind profile)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 040 | Ship for vertical soundings | Ship's identification, etc. |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 03 012 | Wind at pressure level |  |  |

3 09 017 (Vertical sounding with relative humidity)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 040 | Ship for vertical soundings | Ship's identification, etc. |  |
| 3 02 004 | General cloud information | Significant cloud layer |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 03 013 | Geopotential, temperature, humidity, wind at pressure level |  |  |

3 09 018 (Vertical sounding with dewpoint data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 040 | Ship for vertical soundings | Ship's identification, etc. |  |
| 3 02 004 | General cloud information | Significant cloud layer |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 03 014 | Geopotential, temperature, dewpoint temperature, wind at pressure level |  |  |

3 09 019 (Wind profiler - wind data sounding)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 031 | Identification and type of station, date/time, location (high accuracy), height of station |  |  |
| 0 02 003 | Type of measuring equipment used |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 03 011 | Wind at height |  |  |

3 09 020 (Wind profiler - Cartesian coordinates)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 031 | Identification and type of station, date/time, location (high accuracy), height of station |  |  |
| 0 02 003 | Type of measuring equipment used |  |  |
| 1 04 000 | Delayed replication of 4 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 07 003 | Geopotential |  |  |
| 0 11 003 | u-component |  |  |
| 0 11 004 | v-component |  |  |
| 0 11 005 | w-component |  |  |

3 09 021 (Single wavelength wind profiler wind data (product data))

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 001 | WMO block and station numbers |  | Note 56: The sequences 3 09 021, 3 09 022, and 3 09 023 should not be used because they are outdated. The sequences 3 09 024, 3 09 025 and 3 09 026, respectively, should be used instead. |
| 0 05 001 | Latitude (high accuracy) |  |  |
| 0 06 001 | Longitude (high accuracy) |  |  |
| 0 07 030 | Height of station ground above mean sea level |  |  |
| 3 01 014 | Time period |  |  |
| 0 02 003 | Type of measuring equipment used |  |  |
| 0 02 121 | Mean frequency |  |  |
| 1 12 000 | Delayed replication of 12 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 07 007 | Height |  |  |
| 3 01 021 | Latitude/longitude (high accuracy) |  |  |
| 0 11 003 | u-component |  |  |
| 0 11 110 | Uncertainty in u-component |  |  |
| 0 11 004 | v-component |  |  |
| 0 11 111 | Uncertainty in v-component |  |  |
| 0 33 002 | Quality information |  |  |
| 0 11 006 | w-component |  |  |
| 0 11 112 | Uncertainty in w-component |  |  |
| 0 33 002 | Quality information |  |  |
| 0 10 071 | Vertical resolution |  |  |
| 0 27 079 | Horizontal width of sampled volume |  |  |

3 09 022 (RASS virtual temperature (product data))

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 001 | WMO block and station numbers |  | Note 56: The sequences 3 09 021, 3 09 022, and 3 09 023 should not be used because they are outdated. The sequences 3 09 024, 3 09 025 and 3 09 026, respectively, should be used instead. |
| 0 05 001 | Latitude (high accuracy) |  |  |
| 0 06 001 | Longitude (high accuracy) |  |  |
| 0 07 030 | Height of station ground above mean sea level |  |  |
| 3 01 014 | Time period |  |  |
| 0 02 003 | Type of measuring equipment used |  |  |
| 0 02 121 | Mean frequency |  |  |
| 1 10 000 | Delayed replication of 10 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 07 007 | Height |  |  |
| 3 01 021 | Latitude/longitude (high accuracy) |  |  |
| 0 12 007 | Virtual temperature |  |  |
| 0 12 008 | Uncertainty in virtual temperature |  |  |
| 0 33 002 | Quality information |  |  |
| 0 11 006 | w-component |  |  |
| 0 11 112 | Uncertainty in w-component |  |  |
| 0 33 002 | Quality information |  |  |
| 0 10 071 | Vertical resolution |  |  |
| 0 27 079 | Horizontal width of sampled volume |  |  |

3 09 023 (Single wavelength elastic backscatter lidar sequence)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 001 | WMO block and station numbers |  | Note 56: The sequences 3 09 021, 3 09 022, and 3 09 023 should not be used because they are outdated. The sequences 3 09 024, 3 09 025 and 3 09 026, respectively, should be used instead. |
| 0 05 001 | Latitude (high accuracy) |  |  |
| 0 06 001 | Longitude (high accuracy) |  |  |
| 0 07 030 | Height of station ground above mean sea level |  |  |
| 3 01 014 | Time period |  |  |
| 0 02 003 | Type of measuring equipment used |  |  |
| 3 02 004 | General cloud information |  |  |
| 3 02 005 | Cloud layer |  |  |
| 1 14 000 | Delayed replication of 14 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 07 007 | Height |  |  |
| 3 01 021 | Latitude/longitude (high accuracy) |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor | Number of wavelength measurements present in the data |  |
| 0 02 121 | Mean frequency |  |  |
| 0 15 063 | Attenuated backscatter |  |  |
| 0 15 064 | Uncertainty in attenuated backscatter |  |  |
| 0 15 065 | Particle backscatter coefficient |  |  |
| 0 15 066 | Uncertainty in particle backscatter coefficient |  |  |
| 0 15 067 | Particle extinction coefficient |  |  |
| 0 15 068 | Uncertainty in particle extinction coefficient |  |  |
| 0 15 069 | Particle lidar ratio |  |  |
| 0 15 070 | Uncertainty in lidar ratio |  |  |
| 0 15 071 | Particle depolarization ratio |  |  |
| 0 15 072 | Uncertainty in depolarization ratio |  |  |
| 0 33 002 | Quality information |  |  |
| 0 10 071 | Vertical resolution |  |  |
| 0 27 079 | Horizontal width of sampled volume |  |  |

3 09 024 (Single wavelength wind profiler wind data sequence (product data))

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 132 | Common header sequence |  |  |
| 2 01 151 | Change data width (by YYY-128 bit places) |  |  |
| 2 02 130 | Change scale (by YYY-128 places) |  |  |
| 0 02 121 | Mean frequency |  |  |
| 2 02 000 | Cancel scale change |  |  |
| 2 01 000 | Cancel data width change |  |  |
| 0 08 021 | Time significance | = 2 Time averaged |  |
| 0 04 025 | Time period or displacement |  | Note 60: The time period indicates the duration in minutes over which the measurements have been averaged. The end of the time period is marked by the end of scan (see above). |
| 1 09 000 | Delayed replication of 9 descriptors |  |  |
| 0 31 002 | Extended delayed descriptor replication factor |  |  |
| 0 07 007 | Height |  | Note 52: The Quality information descriptor 0 33 002 at this point relates to both u-component 0 11 003 and v-component 0 11 004. It is given for each respective gate of the profile measurement. |
| 3 01 021 | Latitude/longitude (high accuracy) |  | Note 47: The Height descriptor 0 07 007 and the Latitude/longitude descriptor 3 01 021 indicate the altitude above sea level and the horizontal position of each vertical bin of the profile measurement and not the location of the instrument itself (which is encoded in the preceding Common header sequence 3 01 132). |
| 0 11 003 | u-component |  |  |
| 0 11 004 | v-component |  |  |
| 0 33 002 | Quality information |  | Note 53: The Quality information descriptor 0 33 002 at this point relates to the w-component 0 11 006. It is given for each respective gate of the profile measurement. |
| 0 11 006 | w-component |  |  |
| 0 33 002 | Quality information |  |  |
| 0 10 071 | Vertical resolution |  |  |
| 0 27 079 | Horizontal width of sampled volume |  |  |

3 09 025 (RASS virtual temperature sequence (product data))

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 132 | Common Header Sequence |  |  |
| 2 01 151 | Change data width |  |  |
| 2 02 130 | Change scale |  |  |
| 0 02 121 | Mean frequency |  |  |
| 2 02 000 | Cancel scale change |  |  |
| 2 01 000 | Cancel data width change |  |  |
| 0 08 021 | Time significance | = 2 Time averaged |  |
| 0 04 025 | Time period or displacement |  | Note 60: The time period indicates the duration in minutes over which the measurements have been averaged. The end of the time period is marked by the end of scan (see above). |
| 1 06 000 | Delayed replication of 6 descriptors |  |  |
| 0 31 002 | Extended delayed descriptor replication factor |  |  |
| 0 07 007 | Height |  | Note 47: The Height descriptor 0 07 007 and the Latitude/longitude descriptor 3 01 021 indicate the altitude above sea level and the horizontal position of each vertical bin of the profile measurement and not the location of the instrument itself (which is encoded in the preceding Common header sequence 3 01 132). |
| 3 01 021 | Latitude/longitude (high accuracy) |  | Note 47: The Height descriptor 0 07 007 and the Latitude/longitude descriptor 3 01 021 indicate the altitude above sea level and the horizontal position of each vertical bin of the profile measurement and not the location of the instrument itself (which is encoded in the preceding Common header sequence 3 01 132). |
| 0 12 007 | Virtual temperature |  |  |
| 0 33 002 | Quality information |  | Note 55: The Quality information descriptor 0 33 002 relates to the Virtual temperature 0 12 007. It is given for each respective gate of the profile measurement. |
| 0 10 071 | Vertical resolution |  |  |
| 0 27 079 | Horizontal width of sampled volume |  |  |

3 09 026 (Single wavelength elastic backscatter lidar sequence)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 132 | Common header sequence |  |  |
| 3 02 004 | General cloud information |  |  |
| 3 02 005 | Cloud layer |  |  |
| 0 08 092 | Measurement uncertainty expression |  | Note 50: The Measurement uncertainty expression and significance descriptors 0 08 092 and 0 08 093 relate to all parameter uncertainties provided in the Backscatter data section |
| 0 08 093 | Measurement uncertainty significance |  | Note 47: The Height descriptor 0 07 007 and the Latitude/longitude descriptor 3 01 021 indicate the altitude above sea level and the horizontal position of each vertical bin of the profile measurement and not the location of the instrument itself (which is encoded in the preceding Common header sequence 3 01 132). |
| 0 08 021 | Time significance | = 2 Time averaged |  |
| 0 04 025 | Time period or displacement |  | Note 60: The time period indicates the duration in minutes over which the measurements have been averaged. The end of the time period is marked by the end of scan (see above). |
| 2 01 138 | Change data width |  |  |
| 2 02 126 | Change scale |  |  |
| 0 02 121 | Mean frequency |  |  |
| 2 02 000 | Cancel scale change |  |  |
| 2 01 000 | Cancel data width change |  |  |
| 1 15 000 | Delayed replication of 15 descriptors |  |  |
| 0 31 002 | Extended delayed descriptor replication factor |  |  |
| 0 07 007 | Height |  | Note 47: The Height descriptor 0 07 007 and the Latitude/longitude descriptor 3 01 021 indicate the altitude above sea level and the horizontal position of each vertical bin of the profile measurement and not the location of the instrument itself (which is encoded in the preceding Common header sequence 3 01 132). |
| 3 01 021 | Latitude/longitude (high accuracy) | = 2 Time averaged |  |
| 0 15 073 | Attenuated backscatter |  |  |
| 0 15 064 | Uncertainty in attenuated backscatter |  |  |
| 0 15 074 | Particle backscatter coefficient |  |  |
| 0 15 066 | Uncertainty in particle backscatter coefficient |  |  |
| 0 15 075 | Particle extinction coefficient |  |  |
| 0 15 068 | Uncertainty in particle extinction coefficient |  |  |
| 0 15 076 | Particle lidar ratio |  |  |
| 0 15 077 | Uncertainty in lidar ratio |  |  |
| 0 15 078 | Particle depolarization ratio |  |  |
| 0 15 072 | Uncertainty in depolarization ratio |  |  |
| 0 33 002 | Quality information |  | Note 54: The Quality information descriptor 0 33 002 relates to all parameters provided in the Backscatter data section and is given for each respective vertical bin of the profile measurement. |
| 0 10 071 | Vertical resolution |  |  |
| 0 27 079 | Horizontal width of sampled volume |  |  |

3 09 027 (Multi wavelength ground-based lidar sequence)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 132 | Common header sequence |  |  |
| 0 08 043 | Atmospheric chemical or physical constituent type |  |  |
| 0 25 061 | Software version number |  |  |
| 3 02 004 | General cloud information |  |  |
| 3 02 005 | Cloud layer |  |  |
| 0 08 092 | Measurement uncertainty expression |  | (see Note 18) |
| 0 08 093 | Measurement uncertainty significance |  | Note 47: The Height descriptor 0 07 007 and the Latitude/longitude descriptor 3 01 021 indicate the altitude above sea level and the horizontal position of each vertical bin of the profile measurement and not the location of the instrument itself (which is encoded in the preceding Common header sequence 3 01 132). |
| 0 08 021 | Time significance | = 2 Time averaged |  |
| 0 04 025 | Time period or displacement |  | Note 60: The time period indicates the duration in minutes over which the measurements have been averaged. The end of the time period is marked by the end of scan (see above). |
| 1 18 000 | Delayed replication of 18 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 02 090 | Instrument wavelength |  |  |
| 1 15 000 | Delayed replication of 15 descriptors |  |  |
| 0 31 002 | Extended delayed descriptor replication factor |  |  |
| 0 07 007 | Height |  | Note 47: The Height descriptor 0 07 007 and the Latitude/longitude descriptor 3 01 021 indicate the altitude above sea level and the horizontal position of each vertical bin of the profile measurement and not the location of the instrument itself (which is encoded in the preceding Common header sequence 3 01 132). |
| 3 01 021 | Latitude/longitude (high accuracy) | = 2 Time averaged |  |
| 0 15 073 | Attenuated backscatter |  |  |
| 0 15 064 | Uncertainty in attenuated backscatter |  |  |
| 0 15 074 | Particle backscatter coefficient |  |  |
| 0 15 066 | Uncertainty in particle backscatter coefficient |  |  |
| 0 15 075 | Particle extinction coefficient |  |  |
| 0 15 068 | Uncertainty in particle extinction coefficient |  |  |
| 0 15 076 | Particle lidar ratio |  |  |
| 0 15 077 | Uncertainty in lidar ratio |  |  |
| 0 15 078 | Particle depolarization ratio |  |  |
| 0 15 072 | Uncertainty in depolarization ratio |  |  |
| 0 33 002 | Quality information |  | Note 54: The Quality information descriptor 0 33 002 relates to all parameters provided in the Backscatter data section and is given for each respective vertical bin of the profile measurement. |
| 0 10 071 | Vertical resolution |  |  |
| 0 27 079 | Horizontal width of sampled volume |  |  |

3 09 030 (Ozone sonde flight data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 15 004 | Ozone sounding correction factor (CF) |  | Note 44: Sequence 3 09 030 is deprecated because of incorrect usage of descriptor 0 04 015; sequence 3 09 031 should be used instead. |
| 0 15 005 | Ozone p |  | Note 44: Sequence 3 09 030 is deprecated because of incorrect usage of descriptor 0 04 015; sequence 3 09 031 should be used instead. |
| 1 04 000 | Delayed replication of 4 descriptors |  | Note 44: Sequence 3 09 030 is deprecated because of incorrect usage of descriptor 0 04 015; sequence 3 09 031 should be used instead. |
| 0 31 001 | Delayed descriptor replication factor |  | Note 44: Sequence 3 09 030 is deprecated because of incorrect usage of descriptor 0 04 015; sequence 3 09 031 should be used instead. |
| 0 04 015 | Time increment | Since launch time, if needed, in minutes | Note 44: Sequence 3 09 030 is deprecated because of incorrect usage of descriptor 0 04 015; sequence 3 09 031 should be used instead. |
| 0 08 006 | Ozone vertical sounding significance |  | Note 44: Sequence 3 09 030 is deprecated because of incorrect usage of descriptor 0 04 015; sequence 3 09 031 should be used instead. |
| 0 07 004 | Pressure |  | Note 44: Sequence 3 09 030 is deprecated because of incorrect usage of descriptor 0 04 015; sequence 3 09 031 should be used instead. |
| 0 15 003 | Measured ozone partial pressure (sounding) |  | Note 44: Sequence 3 09 030 is deprecated because of incorrect usage of descriptor 0 04 015; sequence 3 09 031 should be used instead. |

3 09 031 (Ozone sonde flight data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 15 004 | Ozone sounding correction factor (CF) |  |  |
| 0 15 005 | Ozone p |  |  |
| 1 04 000 | Delayed replication of 4 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 04 025 | Time period or displacement | Since launch time in minutes |  |
| 0 08 006 | Ozone vertical sounding significance |  |  |
| 0 07 004 | Pressure |  |  |
| 0 15 003 | Measured ozone partial pressure (sounding) |  |  |

3 09 040 (Ozone sounding not coupled to a ground-based spectrophotometer)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 075 | Sounding identification |  | Note 67: This sequence is deprecated because it includes deprecated sequence 3 09 030; sequence 3 09 045,3 09 046, 3 09 047, 3 09 048 and 3 09 049 should be used instead of 3 09 040, 3 09 041, 3 09 042, 3 09 043 and 3 09 044, respectively. |
| 3 01 076 | Ozone sounding instrumentation |  | Note 67: This sequence is deprecated because it includes deprecated sequence 3 09 030; sequence 3 09 045,3 09 046, 3 09 047, 3 09 048 and 3 09 049 should be used instead of 3 09 040, 3 09 041, 3 09 042, 3 09 043 and 3 09 044, respectively. |
| 3 09 030 | Ozone sonde flight data |  | Note 67: This sequence is deprecated because it includes deprecated sequence 3 09 030; sequence 3 09 045,3 09 046, 3 09 047, 3 09 048 and 3 09 049 should be used instead of 3 09 040, 3 09 041, 3 09 042, 3 09 043 and 3 09 044, respectively. |

3 09 041 (Ozone sounding coupled to measurements from a Brewer ground-based spectrophotometer; the total ozone obtained from the Brewer is a single value)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 07 041 | Total ozone measurement from a Brewer ground-based spectrophotometer obtained from a single observation | Description of the ground-based part | Note 67: This sequence is deprecated because it includes deprecated sequence 3 09 030; sequence 3 09 045,3 09 046, 3 09 047, 3 09 048 and 3 09 049 should be used instead of 3 09 040, 3 09 041, 3 09 042, 3 09 043 and 3 09 044, respectively. |
| 3 01 075 | Sounding identification | Identification of the ozone sounding part | Note 67: This sequence is deprecated because it includes deprecated sequence 3 09 030; sequence 3 09 045,3 09 046, 3 09 047, 3 09 048 and 3 09 049 should be used instead of 3 09 040, 3 09 041, 3 09 042, 3 09 043 and 3 09 044, respectively. |
| 3 01 076 | Ozone sounding instrumentation |  | Note 67: This sequence is deprecated because it includes deprecated sequence 3 09 030; sequence 3 09 045,3 09 046, 3 09 047, 3 09 048 and 3 09 049 should be used instead of 3 09 040, 3 09 041, 3 09 042, 3 09 043 and 3 09 044, respectively. |
| 3 09 030 | Ozone sonde flight data |  | Note 67: This sequence is deprecated because it includes deprecated sequence 3 09 030; sequence 3 09 045,3 09 046, 3 09 047, 3 09 048 and 3 09 049 should be used instead of 3 09 040, 3 09 041, 3 09 042, 3 09 043 and 3 09 044, respectively. |

3 09 042 (Ozone sounding coupled to measurements from a Brewer ground-based spectrophotometer; the total ozone obtained from the Brewer is an averaged value)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 07 042 | Total ozone measurement from a Brewer ground-based spectrophotometer obtained from averaged observations | Description of the ground-based part | Note 67: This sequence is deprecated because it includes deprecated sequence 3 09 030; sequence 3 09 045,3 09 046, 3 09 047, 3 09 048 and 3 09 049 should be used instead of 3 09 040, 3 09 041, 3 09 042, 3 09 043 and 3 09 044, respectively. |
| 3 01 075 | Sounding identification | Identification of the ozone sounding part | Note 67: This sequence is deprecated because it includes deprecated sequence 3 09 030; sequence 3 09 045,3 09 046, 3 09 047, 3 09 048 and 3 09 049 should be used instead of 3 09 040, 3 09 041, 3 09 042, 3 09 043 and 3 09 044, respectively. |
| 3 01 076 | Ozone sounding instrumentation |  | Note 67: This sequence is deprecated because it includes deprecated sequence 3 09 030; sequence 3 09 045,3 09 046, 3 09 047, 3 09 048 and 3 09 049 should be used instead of 3 09 040, 3 09 041, 3 09 042, 3 09 043 and 3 09 044, respectively. |
| 3 09 030 | Ozone sonde flight data |  | Note 67: This sequence is deprecated because it includes deprecated sequence 3 09 030; sequence 3 09 045,3 09 046, 3 09 047, 3 09 048 and 3 09 049 should be used instead of 3 09 040, 3 09 041, 3 09 042, 3 09 043 and 3 09 044, respectively. |

3 09 043 (Ozone sounding coupled to measurements from a Dobson ground-based spectrophotometer; the total ozone obtained from the Dobson is a single value)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 07 043 | Total ozone measurement from a Dobson ground-based spectrophotometer obtained from a single observation | Description of the ground-based part | Note 67: This sequence is deprecated because it includes deprecated sequence 3 09 030; sequence 3 09 045,3 09 046, 3 09 047, 3 09 048 and 3 09 049 should be used instead of 3 09 040, 3 09 041, 3 09 042, 3 09 043 and 3 09 044, respectively. |
| 3 01 075 | Sounding identification | Identification of the ozone sounding part | Note 67: This sequence is deprecated because it includes deprecated sequence 3 09 030; sequence 3 09 045,3 09 046, 3 09 047, 3 09 048 and 3 09 049 should be used instead of 3 09 040, 3 09 041, 3 09 042, 3 09 043 and 3 09 044, respectively. |
| 3 01 076 | Ozone sounding instrumentation |  | Note 67: This sequence is deprecated because it includes deprecated sequence 3 09 030; sequence 3 09 045,3 09 046, 3 09 047, 3 09 048 and 3 09 049 should be used instead of 3 09 040, 3 09 041, 3 09 042, 3 09 043 and 3 09 044, respectively. |
| 3 09 030 | Ozone sonde flight data |  | Note 67: This sequence is deprecated because it includes deprecated sequence 3 09 030; sequence 3 09 045,3 09 046, 3 09 047, 3 09 048 and 3 09 049 should be used instead of 3 09 040, 3 09 041, 3 09 042, 3 09 043 and 3 09 044, respectively. |

3 09 044 (Ozone sounding coupled to measurements from a Dobson ground-based spectrophotometer; the total ozone obtained from the Dobson is an averaged value)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 07 044 | Total ozone measurement from a Dobson ground-based spectrophotometer obtained from averaged observations | Description of the ground-based part | Note 67: This sequence is deprecated because it includes deprecated sequence 3 09 030; sequence 3 09 045,3 09 046, 3 09 047, 3 09 048 and 3 09 049 should be used instead of 3 09 040, 3 09 041, 3 09 042, 3 09 043 and 3 09 044, respectively. |
| 3 01 075 | Sounding identification | Identification of the ozone sounding part | Note 67: This sequence is deprecated because it includes deprecated sequence 3 09 030; sequence 3 09 045,3 09 046, 3 09 047, 3 09 048 and 3 09 049 should be used instead of 3 09 040, 3 09 041, 3 09 042, 3 09 043 and 3 09 044, respectively. |
| 3 01 076 | Ozone sounding instrumentation |  | Note 67: This sequence is deprecated because it includes deprecated sequence 3 09 030; sequence 3 09 045,3 09 046, 3 09 047, 3 09 048 and 3 09 049 should be used instead of 3 09 040, 3 09 041, 3 09 042, 3 09 043 and 3 09 044, respectively. |
| 3 09 030 | Ozone sonde flight data |  | Note 67: This sequence is deprecated because it includes deprecated sequence 3 09 030; sequence 3 09 045,3 09 046, 3 09 047, 3 09 048 and 3 09 049 should be used instead of 3 09 040, 3 09 041, 3 09 042, 3 09 043 and 3 09 044, respectively. |

3 09 045 (Ozone sounding not coupled to a ground-based spectrophotometer)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 075 | Sounding identification |  |  |
| 3 01 076 | Ozone sounding instrumentation |  |  |
| 3 09 031 | Ozone sonde flight data |  |  |

3 09 046 (Ozone sounding coupled to measurements from a Brewer ground-based spectrophotometer; the total ozone obtained from the Brewer is a single value)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 07 041 | Total ozone measurement from a Brewer ground-based spectrophotometer obtained from a single observation | Description of the ground-based part |  |
| 3 01 075 | Sounding identification | Identification of the ozone sounding part |  |
| 3 01 076 | Ozone sounding instrumentation |  |  |
| 3 09 031 | Ozone sonde flight data |  |  |

3 09 047 (Ozone sounding coupled to measurements from a Brewer ground-based spectrophotometer; the total ozone obtained from the Brewer is an averaged value)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 07 042 | Total ozone measurement from a Brewer ground-based spectrophotometer obtained from averaged observations | Description of the ground-based part |  |
| 3 01 075 | Sounding identification | Identification of the ozone sounding part |  |
| 3 01 076 | Ozone sounding instrumentation |  |  |
| 3 09 031 | Ozone sonde flight data |  |  |

3 09 048 (Ozone sounding coupled to measurements from a Dobson ground-based spectrophotometer; the total ozone obtained from the Dobson is a single value)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 07 043 | Total ozone measurement from a Dobson ground-based spectrophotometer obtained from a single observation | Description of the ground-based part |  |
| 3 01 075 | Sounding identification | Identification of the ozone sounding part |  |
| 3 01 076 | Ozone sounding instrumentation |  |  |
| 3 09 031 | Ozone sonde flight data |  |  |

3 09 049 (Ozone sounding coupled to measurements from a Dobson ground-based spectrophotometer; the total ozone obtained from the Dobson is an averaged value)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 07 044 | Total ozone measurement from a Dobson ground-based spectrophotometer obtained from averaged observations | Description of the ground-based part |  |
| 3 01 075 | Sounding identification | Identification of the ozone sounding part |  |
| 3 01 076 | Ozone sounding instrumentation |  |  |
| 3 09 031 | Ozone sonde flight data |  |  |

3 09 050 (Sequence for representation of PILOT, PILOT SHIP and PILOT MOBIL observation type data with pressure as the vertical coordinate)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 110 | Identification of launch site and instrumentation for wind measurements |  |  |
| 3 01 113 | Date/time of launch |  |  |
| 3 01 114 | Horizontal and vertical coordinates of launch site |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 002 | Extended delayed descriptor replication factor |  |  |
| 3 03 050 | Wind data at a pressure level with radiosonde position |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 03 051 | Wind shear data at a pressure level with radiosonde position |  |  |

3 09 051 (Sequence for representation of PILOT, PILOT SHIP and PILOT MOBIL observation type data with height as the vertical coordinate)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 110 | Identification of launch site and instrumentation for wind measurements |  |  |
| 3 01 113 | Date/time of launch |  |  |
| 3 01 114 | Horizontal and vertical coordinates of launch site |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 002 | Extended delayed descriptor replication factor |  |  |
| 3 03 052 | Wind data at a height level with radiosonde position |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 03 053 | Wind shear data at a height level with radiosonde position |  |  |

3 09 052 (Sequence for representation of TEMP, TEMP SHIP and TEMP MOBIL observation type data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 111 | Identification of launch site and instrumentation for P, T, U and wind measurements |  |  |
| 3 01 113 | Date/time of launch |  |  |
| 3 01 114 | Horizontal and vertical coordinates of launch site |  |  |
| 3 02 049 | Cloud information reported with vertical soundings |  |  |
| 0 22 043 | Sea/water temperature |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 002 | Extended delayed descriptor replication factor |  |  |
| 3 03 054 | Temperature, dewpoint and wind data at a pressure level with radiosonde position |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 03 051 | Wind shear data at a pressure level with radiosonde position |  |  |

3 09 053 (Sequence for representation of TEMP DROP observation type data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 112 | Identification of launch point and instrumentation of dropsonde |  |  |
| 3 01 113 | Date/time of launch |  |  |
| 3 01 114 | Horizontal and vertical coordinates of launch site |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 002 | Extended delayed descriptor replication factor |  |  |
| 3 03 054 | Temperature, dewpoint and wind data at a pressure level with radiosonde position |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 03 051 | Wind shear data at a pressure level with radiosonde position |  |  |

3 09 054 (Sequence for representation of CLIMAT TEMP and CLIMAT TEMP SHIP data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 001 | WMO block and station numbers | Identification of launch site |  |
| 0 01 011 | Ship or mobile land station identifier | Ship's call sign |  |
| 3 01 011 | Year, month, day |  |  |
| 3 01 012 | Hour, minute |  |  |
| 3 01 021 | Latitude/longitude (high accuracy) |  |  |
| 0 07 030 | Height of station ground above mean sea level |  |  |
| 0 07 031 | Height of barometer above mean sea level |  |  |
| 0 07 007 | Height | Release of sonde above mean sea level |  |
| 0 04 023 | Time period or displacement | Number of days in the month |  |
| 0 04 059 | Times of observation used to compute the reported mean values |  |  |
| 1 15 000 | Delayed replication of 15 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 08 001 | Vertical sounding significance |  |  |
| 0 08 023 | First-order statistics | = 4 Mean value |  |
| 0 07 004 | Pressure |  |  |
| 0 10 009 | Geopotential height |  |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 12 103 | Dewpoint temperature |  |  |
| 0 08 023 | First-order statistics | = 32 Vector mean |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 08 023 | First-order statistics | Set to missing |  |
| 0 11 019 | Steadiness of wind |  |  |
| 0 08 050 | Qualifier for number of missing values in calculation of statistic | = 2 Temperature |  |
| 0 08 020 | Total number of missing entities (with respect to accumulation or average) | Days |  |
| 0 08 050 | Qualifier for number of missing values in calculation of statistic | = 9 Wind |  |
| 0 08 020 | Total number of missing entities (with respect to accumulation or average) | Days |  |

3 09 055 (Template for the representation of high resolution radiosonde data with geopotential height as the vertical coordinate)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 111 | Identification of launch site and instrumentation for P, T, U and wind measurements |  |  |
| 0 25 061 | Software identification and version number |  |  |
| 0 01 081 | Radiosonde serial number |  |  |
| 0 01 082 | Radiosonde ascension number |  |  |
| 0 02 067 | Radiosonde operating frequency |  |  |
| 0 02 095 | Type of pressure sensor |  |  |
| 0 02 096 | Type of temperature sensor |  |  |
| 0 02 097 | Type of humidity sensor |  |  |
| 0 02 081 | Type of balloon |  |  |
| 0 02 082 | Weight of balloon |  |  |
| 0 02 084 | Type of gas used in balloon |  |  |
| 0 02 191 | Geopotential height calculation |  |  |
| 3 01 113 | Date/time of launch |  | Note 71: Time of launch 3 01 113 in the sequence shall be reported with the highest possible accuracy available. If the launch time is not available with second accuracy, the entry for seconds shall be put to zero. |
| 3 01 114 | Horizontal and vertical coordinates of launch site |  |  |
| 0 10 004 | Pressure |  |  |
| 3 02 032 | Temperature and humidity data |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 02 002 | Type of instrumentation for wind measurement |  |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 0 20 003 | Present weather |  |  |
| 3 02 049 | Cloud information reported with vertical soundings |  |  |
| 0 22 043 | Sea/water temperature |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 002 | Extended delayed descriptor replication factor |  |  |
| 3 03 055 | Temperature, dewpoint, relative humidity and wind data at a height level with radiosonde position |  | Note 36: Long time displacement 0 04 086 in the sequence represents the time offset from the launch time 3 01 113 (in seconds). ¦ Note 34: Latitude displacement 0 05 015 in the sequence represents the latitude offset from the latitude of the launch site. Longitude displacement 0 06 015 in the sequence represents the longitude offset from the longitude of the launch site. ¦ Note 26: If the radiosonde is equipped with a relative humidity sensor, 0 13 009 in the sequence shall be reported as mandatory and dewpoint temperature may be included as a derived value. If the radiosonde is equipped with a dewpoint temperature sensor, 0 12 103 in the sequence shall be reported and 0 13 009 shall be set to a missing value. |

3 09 056 (Sequence for representation of radiosonde descent data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 150 | WIGOS identifier |  |  |
| 3 01 111 | Identification of launch site and instrumentation for P, T, U and wind measurements |  |  |
| 3 01 128 | Additional information on radiosonde ascent | Valid also for decent |  |
| 3 01 113 | Date/time of launch |  | Note 9: Date/time of launch indicates date/time of start of descent measurement. |
| 0 08 091 | Coordinates significance | = 2 Start of observation |  |
| 3 01 021 | Latitude/longitude (high accuracy) |  |  |
| 0 07 007 | Height | Begin of descending of radiosonde above mean sea level |  |
| 0 08 091 | Coordinates significance | Set to missing (cancel) |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 002 | Extended delayed descriptor replication factor |  |  |
| 3 03 056 | Temperature, dewpoint and wind data at a pressure level with radiosonde position and higher precision of pressure and geopotential height |  | Note 30: In this sequence for representation of radiosonde descent data, indication of standard levels using the extended vertical sounding significance (0 08 042) is not mandatory. ¦ Note 8: Data represented by this sequence should be sorted in descending order with respect to pressure. |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 03 051 | Wind shear data at a pressure level with radiosonde position |  |  |

3 09 057 (Sequence for representation of TEMP, TEMP SHIP and TEMP MOBIL observation type data with higher precision of pressure and geopotential height)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 150 | WIGOS identifier |  |  |
| 3 01 111 | Identification of launch site and instrumentation for P, T, U and wind measurements |  |  |
| 3 01 128 | Additional information on radiosonde ascent |  |  |
| 3 01 113 | Date/time of launch |  |  |
| 3 01 114 | Horizontal and vertical coordinates of launch site |  |  |
| 3 02 049 | Cloud information reported with vertical soundings |  |  |
| 0 22 043 | Sea/water temperature |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 002 | Extended delayed descriptor replication factor |  |  |
| 3 03 056 | Temperature, dewpoint and wind data at a pressure level with radiosonde position and higher precision of pressure and geopotential height |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 03 051 | Wind shear data at a pressure level with radiosonde position |  |  |

3 09 060 (Radiosonde complete registration and surface observation)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 123 | Radiosonde full header information |  |  |
| 3 01 121 | Radiosonde launch point location |  |  |
| 3 02 050 | Radiosonde surface observation |  |  |
| 3 03 040 | Radiosonde duration of flight and termination information |  |  |

3 09 061 (Raw PTU)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 120 | Radiosonde abbreviated header and launch information |  |  |
| 0 08 041 | Data significance | = 6 Flight level observation |  |
| 3 01 122 | Date/time (to hundredths of second) |  |  |
| 2 01 131 | Change data width |  |  |
| 2 02 129 | Change scale |  |  |
| 0 25 069 | Flight level pressure corrections |  |  |
| 0 07 004 | Pressure |  |  |
| 2 02 000 | Change scale | Cancel |  |
| 2 01 000 | Change data width | Cancel |  |
| 0 33 007 | Per cent confidence | Pressure |  |
| 0 33 035 | Manual/automatic quality control | Pressure |  |
| 0 33 015 | Data quality check indicator | Pressure |  |
| 0 13 009 | Relative humidity |  |  |
| 0 33 007 | Per cent confidence | Relative humidity |  |
| 0 33 035 | Manual/automatic quality control | Relative humidity |  |
| 0 33 015 | Data quality check indicator | Relative humidity |  |
| 0 02 013 | Solar and infrared radiation correction |  |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 33 007 | Per cent confidence | Temperature |  |
| 0 33 035 | Manual/automatic quality control | Temperature |  |
| 0 33 015 | Data quality check indicator | Temperature |  |

3 09 062 (Raw GPS unsmoothed wind)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 120 | Radiosonde abbreviated header and launch information |  |  |
| 0 08 041 | Data significance | = 6 Flight level observation |  |
| 3 01 122 | Date/time (to hundredths of second) |  |  |
| 0 05 001 | Latitude (high accuracy) |  |  |
| 0 33 035 | Manual/automatic quality control | Latitude |  |
| 0 33 015 | Data quality check indicator | Latitude |  |
| 0 06 001 | Longitude (high accuracy) |  |  |
| 0 33 035 | Manual/automatic quality control | Longitude |  |
| 0 33 015 | Data quality check indicator | Longitude |  |
| 0 07 007 | Height |  |  |
| 0 33 035 | Manual/automatic quality control | Height |  |
| 0 33 015 | Data quality check indicator | Height |  |
| 0 11 003 | u-component |  |  |
| 0 33 035 | Manual/automatic quality control | u-component |  |
| 0 33 015 | Data quality check indicator | u-component |  |
| 0 11 004 | v-component |  |  |
| 0 33 035 | Manual/automatic quality control | v-component |  |
| 0 33 015 | Data quality check indicator | v-component |  |
| 0 33 007 | Per cent confidence | Raw GPS unsmoothed wind |  |

3 09 063 (Raw GPS smoothed wind)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 120 | Radiosonde abbreviated header and launch information |  |  |
| 0 08 041 | Data significance | = 6 Flight level observation |  |
| 3 01 122 | Date/time (to hundredths of second) |  |  |
| 0 05 001 | Latitude (high accuracy) |  |  |
| 0 33 035 | Manual/automatic quality control | Latitude |  |
| 0 33 015 | Data quality check indicator | Latitude |  |
| 0 06 001 | Longitude (high accuracy) |  |  |
| 0 33 035 | Manual/automatic quality control | Longitude |  |
| 0 33 015 | Data quality check indicator | Longitude |  |
| 0 07 007 | Height |  |  |
| 0 33 035 | Manual/automatic quality control | Height |  |
| 0 33 015 | Data quality check indicator | Height |  |
| 0 11 003 | u-component |  |  |
| 0 33 035 | Manual/automatic quality control | u-component |  |
| 0 33 015 | Data quality check indicator | u-component |  |
| 0 11 004 | v-component |  |  |
| 0 33 035 | Manual/automatic quality control | v-component |  |
| 0 33 015 | Data quality check indicator | v-component |  |
| 0 33 007 | Per cent confidence | Raw GPS smoothed wind |  |

3 09 064 (Processed PTU)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 120 | Radiosonde abbreviated header and launch information |  |  |
| 0 08 041 | Data significance | = 6 Flight level observation |  |
| 3 01 122 | Date/time (to hundredths of second) |  |  |
| 2 01 131 | Change data width |  |  |
| 2 02 129 | Change scale |  |  |
| 1 04 002 | Replicate 4 descriptors 2 times |  |  |
| 0 25 069 | Flight level pressure corrections |  |  |
| 0 07 004 | Pressure |  |  |
| 0 33 035 | Manual/automatic quality control | Pressure |  |
| 0 33 015 | Data quality check indicator | Pressure |  |
| 0 13 003 | Relative humidity |  |  |
| 0 33 035 | Manual/automatic quality control | Relative humidity |  |
| 0 33 015 | Data quality check indicator | Relative humidity |  |
| 2 02 000 | Change scale | Cancel |  |
| 2 01 000 | Change data width | Cancel |  |
| 1 04 002 | Replicate 4 descriptors 2 times |  |  |
| 0 02 013 | Solar and infrared radiation correction |  |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 33 035 | Manual/automatic quality control | Temperature |  |
| 0 33 015 | Data quality check indicator | Temperature |  |
| 0 12 103 | Dewpoint temperature |  |  |
| 0 33 035 | Manual/automatic quality control | Dewpoint temperature |  |
| 0 33 015 | Data quality check indicator | Dewpoint temperature |  |
| 0 10 009 | Geopotential height |  |  |
| 0 33 035 | Manual/automatic quality control | Geopotential height |  |
| 0 33 015 | Data quality check indicator | Geopotential height |  |

3 09 065 (Processed GPS)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 120 | Radiosonde abbreviated header and launch information |  |  |
| 0 08 041 | Data significance | = 6 Flight level observation |  |
| 3 01 122 | Date/time (to hundredths of second) |  |  |
| 0 05 001 | Latitude (high accuracy) |  |  |
| 0 33 035 | Manual/automatic quality control | Latitude |  |
| 0 33 015 | Data quality check indicator | Latitude |  |
| 0 06 001 | Longitude (high accuracy) |  |  |
| 0 33 035 | Manual/automatic quality control | Longitude |  |
| 0 33 015 | Data quality check indicator | Longitude |  |
| 0 07 007 | Height |  |  |
| 0 33 035 | Manual/automatic quality control | Height |  |
| 0 33 015 | Data quality check indicator | Height |  |
| 0 11 003 | u-component |  |  |
| 0 33 035 | Manual/automatic quality control | u-component |  |
| 0 33 015 | Data quality check indicator | u-component |  |
| 0 11 004 | v-component |  |  |
| 0 33 035 | Manual/automatic quality control | v-component |  |
| 0 33 015 | Data quality check indicator | v-component |  |

3 09 066 (Standard and significant levels)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 120 | Radiosonde abbreviated header and launch information |  |  |
| 0 08 041 | Data significance | = 6 Flight level observation |  |
| 3 01 122 | Date/time (to hundredths of second) |  |  |
| 0 08 040 | Flight level significance |  |  |
| 2 01 131 | Change data width |  |  |
| 2 02 129 | Change scale |  |  |
| 0 25 069 | Flight level pressure corrections |  |  |
| 0 07 004 | Pressure |  |  |
| 0 13 003 | Relative humidity |  |  |
| 2 02 000 | Change scale | Cancel |  |
| 2 01 000 | Change data width | Cancel |  |
| 0 02 013 | Solar and infrared radiation correction |  |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 12 103 | Dewpoint temperature |  |  |
| 0 10 009 | Geopotential height |  |  |
| 0 10 007 | Height |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 11 001 | Wind direction |  |  |

3 09 070 (Vertical profile for numerical weather prediction data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 01 035 | Originating centre |  |  |
| 0 01 032 | Generating application |  |  |
| 0 01 015 | Station or site name |  |  |
| 0 01 063 | ICAO location indicator |  |  |
| 3 01 001 | WMO block and station numbers |  |  |
| 3 01 011 | Year, month, day |  |  |
| 3 01 012 | Hour, minute | Reference time of the forecast (T-zero) |  |
| 3 01 021 | Latitude/longitude (high accuracy) |  |  |
| 2 07 001 | Increase scale, reference value and data width | Increase scale factor by 1; reference value and data width are recalculated in accordance with the Table C specification of operator 2 07 YYY |  |
| 0 10 001 | Height of land surface | Station elevation (non coordinate) | Note 70: This value is the official or best estimate of the actual elevation of the station. It is provided for comparison with the model’s virtual terrain elevation. The two can be substantially different in rugged terrain. The scale factor is increased to make the value directly comparable with 0 07 030 below. |
| 2 07 000 | Increase scale, reference value and data width | Cancel |  |
| 0 08 086 | Vertical significance for NWP | Bit 9 set to 1 Virtual station height |  |
| 0 07 030 | Height of station ground above mean sea level | Elevation of model terrain at the latitude/longitude of station. As qualified by 0 08 086, this value is both station and model specific. |  |
| 0 25 031 | NWP-generated vertical profile thinning method |  | Note 29: In this instance, the term "thinning" refers to a method that may be applied to select a subset of levels from a model that may have many native vertical levels. Selecting only a subset reduces the size of the pseudo-sounding, at the possible cost of information loss and extra processing. |
| 0 08 021 | Time significance | = 4 Forecast, = 16 Analysis, = 27 First guess |  |
| 0 04 014 | Time increment | Validity time of the forecast expressed as a Delta T from reference time. In the case of an analysis or 00 hour forecast, the value is set to zero |  |
| 0 10 004 | Pressure |  |  |
| 0 10 051 | Pressure reduced to mean sea level |  |  |
| 0 10 009 | Geopotential height |  |  |
| 0 20 010 | Cloud cover (total) |  |  |
| 0 13 095 | Total column water vapour |  |  |
| 1 28 000 | Delayed replication of 28 descriptors |  |  |
| 0 31 002 | Extended delayed descriptor replication factor | The number of levels used in the vertical profile is determined by this replication. The number of levels is discretionary and comprises all agl levels and pressure levels |  |
| 1 13 000 | Delayed replication of 13 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor | = 1 Vertical coordinate is pressure, = 0 Otherwise |  |
| 0 08 086 | Vertical significance for NWP | Bit 1 set to 0 and other bits as appropriate |  |
| 0 07 004 | Pressure |  | Note 40: Non-surface levels on the model’s native vertical coordinate are transposed to pressure coordinate. This makes the levels more readily intelligible for human interpretation and easier to use by generic display applications. The levels may correspond exactly to native model levels, or be interpolated between model levels to pressure levels chosen by the generating centre. |
| 0 11 001 | Wind direction | Degrees true |  |
| 0 11 002 | Wind speed | m/s |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 12 102 | Wet-bulb temperature |  |  |
| 0 12 103 | Dewpoint temperature |  |  |
| 0 10 009 | Geopotential height |  |  |
| 1 03 000 | Delayed replication of 3 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor | = 1 Optional enhanced model data is to be included |  |
| 0 11 021 | Relative vorticity |  |  |
| 0 11 022 | Divergence |  |  |
| 0 11 005 | w-component | Vertical motion |  |
| 1 04 000 | Delayed replication of 4 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor | = 1 Vertical coordinate is 10 metres above ground level, = 0 Otherwise |  |
| 0 08 086 | Vertical significance for NWP | Bit 1 set to 1, bit 8 set to 1 |  |
| 0 07 006 | Height above station | = 10 m |  |
| 0 11 001 | Wind direction | Degrees true |  |
| 0 11 002 | Wind speed | m/s |  |
| 1 05 000 | Delayed replication of 5 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor | = 1 Vertical coordinate is 2 metres above ground level, = 0 Otherwise |  |
| 0 08 086 | Vertical significance for NWP |  |  |
| 0 07 006 | Height above station | = 2 m |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 12 102 | Wet-bulb temperature |  |  |
| 0 12 103 | Dewpoint temperature |  |  |

3 09 071 (Sequence for representation of PILOT in the area of ASECNA)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 001 | WMO block and station numbers |  |  |
| 0 02 014 | Tracking technique/status of system used |  |  |
| 0 02 003 | Type of measuring equipment used |  |  |
| 3 01 113 | Date/time of launch |  |  |
| 3 01 114 | Horizontal and vertical coordinates of launch site |  |  |
| 3 01 023 | Latitude/longitude (coarse accuracy) |  |  |
| 0 07 030 | Height of station ground above mean sea level |  |  |
| 0 07 007 | Height | Release of balloon |  |
| 1 03 000 | Delayed replication of 3 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 07 009 | Geopotential height |  |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |

3 09 072 (Ground-based microwave radiometer brightness temperature observations (Lv 1))

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 132 | Common header sequence |  |  |
| 1 07 000 | Delayed replication of 7 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 03 032 | Channel centre frequency |  |  |
| 0 03 033 | Channel bandwidth |  |  |
| 0 02 106 | 3-dB beam width |  |  |
| 1 02 000 | Delayed replication of 2 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 03 032 | Channel centre frequency |  |  |
| 0 12 190 | Brightness temperature error covariance |  |  |
| 1 13 000 | Delayed replication of 13 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor | Number of microwave radiometer measurements |  |
| 0 08 021 | Time significance | =29 End of scan or time of ending |  |
| 3 01 011 | Year, month, day |  |  |
| 3 01 013 | Hour, minute, second |  |  |
| 0 08 021 | Time significance | = 2 Time averaged |  |
| 0 04 026 | Time period or displacement |  |  |
| 0 02 134 | Antenna beam azimuth |  |  |
| 0 02 135 | Antenna elevation |  |  |
| 0 20 143 | Liquid cloud |  |  |
| 1 03 000 | Delayed replication of 3 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 03 032 | Channel centre frequency |  |  |
| 0 33 021 | Quality of following value |  |  |
| 0 12 063 | Brightness temperature |  |  |
| 1 14 000 | Delayed replication of 14 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor | Number of infrared radiometer measurements |  |
| 0 08 021 | Time significance | = 29 End of scan or time of ending |  |
| 3 01 011 | Year, month, day |  |  |
| 3 01 013 | Hour, minute, second |  |  |
| 0 08 021 | Time significance | = 2 Time averaged |  |
| 0 04 026 | Time period or displacement |  |  |
| 0 02 134 | Antenna beam azimuth |  |  |
| 0 02 135 | Antenna elevation |  |  |
| 1 05 000 | Delayed replication of 5 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 03 034 | Channel wavelength |  |  |
| 0 03 031 | Channel bandwidth, full width half maximum (FWHM) |  |  |
| 0 02 106 | 3dB beam width |  |  |
| 0 33 021 | Quality of following value |  |  |
| 0 12 063 | Brightness temperature |  |  |

3 09 073 (Ground-based microwave radiometer retrieval products (Lv2))

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 132 | Common header sequence |  |  |
| 0 08 021 | Time significance | = 2 Time averaged |  |
| 0 04 026 | Time period or displacement |  |  |
| 0 13 095 | Total column water vapour |  |  |
| 0 13 109 | Ice/liquid water path | Ground-based MWR are only sensitive to liquid water |  |
| 0 20 013 | Height of base of cloud |  |  |
| 0 08 092 | Measurement uncertainty expression | = 0 Standard uncertainty |  |
| 0 08 093 | Measurement uncertainty significance |  |  |
| 0 13 095 | Total column water vapour |  |  |
| 0 13 109 | Ice/liquid water path |  |  |
| 0 20 013 | Height of base of cloud |  |  |
| 0 08 092 | Measurement uncertainty expression | Set to missing (cancel) |  |
| 0 08 093 | Measurement uncertainty significance | Set to missing (cancel) |  |
| 3 03 100 | Forecast indices |  |  |
| 1 17 000 | Delayed replication of 17 descriptors |  |  |
| 0 31 002 | Extended delayed replication factor |  |  |
| 0 07 007 | Height |  |  |
| 3 01 021 | Latitude/longitude (high accuracy) |  |  |
| 0 33 021 | Quality of following value |  |  |
| 0 12 001 | Temperature |  |  |
| 0 33 021 | Quality of following value |  |  |
| 0 13 002 | Mixing ratio |  |  |
| 0 33 021 | Quality of following value |  |  |
| 0 13 003 | Relative humidity |  |  |
| 0 08 092 | Measurement uncertainty expression | = 0 Standard uncertainty |  |
| 0 08 093 | Measurement uncertainty significance |  |  |
| 0 12 001 | Temperature |  |  |
| 0 13 002 | Mixing ratio |  |  |
| 0 13 003 | Relative humidity |  |  |
| 0 08 092 | Measurement uncertainty expression | Set to missing (cancel) |  |
| 0 08 093 | Measurement uncertainty significance | Set to missing (cancel) |  |
| 0 10 071 | Vertical resolution |  |  |
| 0 27 079 | Horizontal width of sampled volume |  |  |

Note 4: Additional descriptors are required to reduce the workload with respect to the station database, e.g. for identification of the federal state and identification of the highway.

Note 7: Compound BUFR messages may be described within the data description section, if required (e.g. 3 01 041, 3 04 001, 3 04 002, 3 04 003, 3 04 004, 3 04 005, 3 04 006).

Note 10: Decibel (dB) is a logarithmic measure of the relative power, or of the relative values of two flux densities, especially of sound intensities and radio and radar power densities. In radar meteorology, the logarithmic scale (dBZ) is used for measuring radar reflectivity factor (obtained from the American Meteorological Society Glossary of Meteorology).

Note 15: Descriptors 3 03 021 to 3 03 027 are not available in CREX.

Note 16: Each BUFR message may contain data for a number of locations; the BUFR compression technique involves negligible overheads for data items that are invariant.

Note 32: It could be argued that, as only additions are possible, only complete lines should be allowed; but it is conceivable that local areas will require changes as well as additions, so it is better and in any case clearer to provide descriptions for all the fields.

Note 33: It is better to use different Class 00 descriptors for the defining and defined elements, in the same way as different descriptors correspond to pressure considered as a coordinate and pressure measured at a given point; otherwise special rules would be needed to interpret such message. Entries 0 00 010 to 0 00 012 define F, X and Y for Tables B and D; entry 0 00 030 is a descriptor used as data and provides the F, X and Y values defining a sequence for Table D entries.

Note 34: Latitude displacement 0 05 015 in the sequence represents the latitude offset from the latitude of the launch site. Longitude displacement 0 06 015 in the sequence represents the longitude offset from the longitude of the launch site.

Note 35: Long time displacement 0 04 086 in the sequence represents the time offset from the launch time 3 01 013 (in seconds).

Note 43: Separation of single level satellite data into sets of BUFR messages helps compression and results inefficient data transmission and storage.

Note 49: The majority of stations has only one position on the road and one subsurface temperature sensor. Delayed Replications have been introduced to increase flexibility and volume efficiency.

Note 62: These descriptors should not be used in CREX for transmission.

Note 63: These entries include the facility to update the Table A code figure and data description.