Class 11: Wind and turbulence

| FXY | ELEMENT NAME | BUFR UNIT | SCALE | REF VALUE | BITS | CREX UNIT | SCALE | CHARS | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 11 001 | Wind direction | degree true | 0 | 0 | 9 | degree true | 0 | 3 |  |
| 0 11 002 | Wind speed | m/s | 1 | 0 | 12 | m/s | 1 | 4 |  |
| 0 11 003 | u-component | m/s | 1 | -4096 | 13 | m/s | 1 | 4 |  |
| 0 11 004 | v-component | m/s | 1 | -4096 | 13 | m/s | 1 | 4 |  |
| 0 11 005 | w-component | Pa/s | 1 | -512 | 10 | Pa/s | 1 | 4 |  |
| 0 11 006 | w-component | m/s | 2 | -4096 | 13 | m/s | 2 | 4 |  |
| 0 11 007 | Relative wind direction (in degrees off bow) | deg | 0 | 0 | 9 | deg | 0 | 3 |  |
| 0 11 008 | Relative wind speed | m/s | 1 | 0 | 12 | m/s | 1 | 4 |  |
| 0 11 010 | Wind direction associated with wind speed which follows | degree true | 0 | 0 | 9 | degree true | 0 | 3 |  |
| 0 11 011 | Wind direction at 10 m | degree true | 0 | 0 | 9 | degree true | 0 | 3 |  |
| 0 11 012 | Wind speed at 10 m | m/s | 1 | 0 | 12 | m/s | 1 | 4 |  |
| 0 11 013 | Wind direction at 5 m | degree true | 0 | 0 | 9 | degree true | 0 | 3 |  |
| 0 11 014 | Wind speed at 5 m | m/s | 1 | 0 | 12 | m/s | 1 | 4 |  |
| 0 11 016 | Extreme counterclockwise wind direction of a variable wind | degree true | 0 | 0 | 9 | degree true | 0 | 3 |  |
| 0 11 017 | Extreme clockwise wind direction of a variable wind | degree true | 0 | 0 | 9 | degree true | 0 | 3 |  |
| 0 11 019 | Steadiness of wind | % | 0 | 0 | 7 | % | 0 | 3 | Note 147: The steadiness factor (descriptor 0 11 019) is the ratio of speed of the monthly mean vector wind to the speed of the monthly mean scalar wind expressed as a percentage. It is reported to the nearest one percent. |
| 0 11 021 | Relative vorticity | /s | 9 | -65536 | 17 | /s | 9 | 6 |  |
| 0 11 022 | Divergence | /s | 9 | -65536 | 17 | /s | 9 | 6 |  |
| 0 11 023 | Velocity potential | m2/s | -2 | -65536 | 17 | m2/s | -2 | 6 |  |
| 0 11 024 | Average u-component after the vortex removal | m/s | 1 | -4096 | 13 | m/s | 0 | 0 |  |
| 0 11 025 | Average v-component after the vortex removal | m/s | 1 | -4096 | 13 | m/s | 0 | 0 |  |
| 0 11 030 | Extended degree of turbulence | Code table | 0 | 0 | 6 | Code table | 0 | 2 |  |
| 0 11 031 | Degree of turbulence | Code table | 0 | 0 | 4 | Code table | 0 | 2 |  |
| 0 11 032 | Height of base of turbulence | m | -1 | -40 | 16 | m | -1 | 5 |  |
| 0 11 033 | Height of top of turbulence | m | -1 | -40 | 16 | m | -1 | 5 |  |
| 0 11 034 | Vertical gust velocity | m/s | 1 | -1024 | 11 | m/s | 1 | 4 |  |
| 0 11 035 | Vertical gust acceleration | m s-2 | 2 | -8192 | 14 | m s-2 | 2 | 5 |  |
| 0 11 036 | Maximum derived equivalent vertical gust speed | m/s | 1 | 0 | 10 | m/s | 1 | 4 |  |
| 0 11 037 | Turbulence index | Code table | 0 | 0 | 6 | Code table | 0 | 2 |  |
| 0 11 038 | Time of occurrence of peak eddy dissipation rate | Code table | 0 | 0 | 5 | Code table | 0 | 2 |  |
| 0 11 039 | Extended time of occurrence of peak eddy dissipation rate | Code table | 0 | 0 | 6 | Code table | 0 | 2 |  |
| 0 11 040 | Maximum wind speed (mean wind) | m/s | 1 | 0 | 12 | m/s | 1 | 4 |  |
| 0 11 041 | Maximum wind gust speed | m/s | 1 | 0 | 12 | m/s | 1 | 4 |  |
| 0 11 042 | Maximum wind speed (10-minute mean wind) | m/s | 1 | 0 | 12 | m/s | 1 | 4 |  |
| 0 11 043 | Maximum wind gust direction | degree true | 0 | 0 | 9 | degree true | 0 | 3 |  |
| 0 11 044 | Mean wind direction for surface - 1 500 m (5 000 feet) | degree true | 0 | 0 | 9 | degree true | 0 | 3 |  |
| 0 11 045 | Mean wind speed for surface - 1 500 m (5 000 feet) | m/s | 1 | 0 | 12 | m/s | 1 | 4 |  |
| 0 11 046 | Maximum instantaneous wind speed | m/s | 1 | 0 | 12 | m/s | 1 | 4 |  |
| 0 11 047 | Maximum instantaneous wind speed over 10 minutes | m/s | 1 | 0 | 12 | m/s | 1 | 4 |  |
| 0 11 049 | Standard deviation of wind direction | degree true | 0 | 0 | 9 | degree true | 0 | 3 |  |
| 0 11 050 | Standard deviation of horizontal wind speed | m/s | 1 | 0 | 12 | m/s | 1 | 4 |  |
| 0 11 051 | Standard deviation of vertical wind speed | m/s | 1 | 0 | 8 | m/s | 1 | 3 |  |
| 0 11 052 | Formal uncertainty in wind speed | m/s | 2 | 0 | 13 | m/s | 2 | 5 |  |
| 0 11 053 | Formal uncertainty in wind direction | degree true | 2 | 0 | 15 | degree true | 2 | 5 |  |
| 0 11 054 | Mean wind direction for 1 500 - 3 000 m | degree true | 0 | 0 | 9 | degree true | 0 | 3 |  |
| 0 11 055 | Mean wind speed for 1 500 - 3 000 m | m/s | 1 | 0 | 12 | m/s | 1 | 4 |  |
| 0 11 061 | Absolute wind shear in 1 km layer below | m/s | 1 | 0 | 12 | m/s | 1 | 4 |  |
| 0 11 062 | Absolute wind shear in 1 km layer above | m/s | 1 | 0 | 12 | m/s | 1 | 4 |  |
| 0 11 070 | Designator of the runway affected by wind shear (including ALL) | CCITT IA5 | 0 | 0 | 32 | Character | 0 | 4 |  |
| 0 11 071 | Turbulent vertical momentum flux | m2 s-2 | 3 | -128 | 14 | m2 s-2 | 3 | 5 |  |
| 0 11 072 | Turbulent vertical buoyancy flux | K m s-1 | 3 | -128 | 11 | K m s-1 | 3 | 4 |  |
| 0 11 073 | Turbulent kinetic energy | m2 s-2 | 2 | -1024 | 13 | m2 s-2 | 2 | 4 |  |
| 0 11 074 | Dissipation energy | m2 s-2 | 2 | -1024 | 10 | m2 s-2 | 2 | 4 |  |
| 0 11 075 | Mean turbulence intensity (eddy dissipation rate) | m2/3 s-1 | 2 | 0 | 8 | m2/3 s-1 | 2 | 3 |  |
| 0 11 076 | Peak turbulence intensity (eddy dissipation rate) | m2/3 s-1 | 2 | 0 | 8 | m2/3 s-1 | 2 | 3 |  |
| 0 11 077 | Reporting interval or averaging time for eddy dissipation rate | s | 0 | 0 | 12 | s | 0 | 4 |  |
| 0 11 080 | Number of views used for wind retrieval | Numeric | 0 | 0 | 3 |  |  |  |  |
| 0 11 081 | Model wind direction at 10 m | degree true | 2 | 0 | 16 | degree true | 2 | 5 |  |
| 0 11 082 | Model wind speed at 10 m | m/s | 2 | 0 | 14 | m/s | 2 | 4 |  |
| 0 11 083 | Wind speed | km/h | 0 | 0 | 9 | km/h | 0 | 3 |  |
| 0 11 084 | Wind speed | kt | 0 | 0 | 8 | kt | 0 | 3 |  |
| 0 11 085 | Maximum wind gust speed | km/h | 0 | 0 | 9 | km/h | 0 | 3 |  |
| 0 11 086 | Maximum wind gust speed | kt | 0 | 0 | 8 | kt | 0 | 3 |  |
| 0 11 095 | u-component of the model wind vector | m/s | 1 | -4096 | 13 | m/s | 1 | 4 |  |
| 0 11 096 | v-component of the model wind vector | m/s | 1 | -4096 | 13 | m/s | 1 | 4 |  |
| 0 11 097 | Wind speed from altimeter | m/s | 2 | 0 | 12 | m/s | 2 | 4 |  |
| 0 11 098 | Wind speed from radiometer | m/s | 2 | 0 | 12 | m/s | 2 | 4 |  |
| 0 11 100 | Aircraft true airspeed | m/s | 1 | 0 | 12 | m/s | 1 | 4 |  |
| 0 11 101 | Aircraft ground speed u-component | m/s | 1 | -4096 | 13 | m/s | 1 | 4 |  |
| 0 11 102 | Aircraft ground speed v-component | m/s | 1 | -4096 | 13 | m/s | 1 | 4 |  |
| 0 11 103 | Aircraft ground speed w-component | m/s | 1 | -512 | 10 | m/s | 1 | 3 |  |
| 0 11 104 | True heading of aircraft, ship or other mobile platform | degree true | 0 | 0 | 9 | degree true | 0 | 3 |  |
| 0 11 105 | EDR algorithm version | Numeric | 0 | 0 | 6 | Numeric | 0 | 2 |  |
| 0 11 106 | Running minimum confidence | Numeric | 1 | 0 | 4 | Numeric | 1 | 2 |  |
| 0 11 107 | Maximum number bad inputs | Numeric | 0 | 0 | 5 | Numeric | 0 | 2 |  |
| 0 11 108 | Peak location | Numeric | 1 | 0 | 4 | Numeric | 1 | 2 |  |
| 0 11 109 | Number of good EDR | Numeric | 0 | 0 | 4 | Numeric | 0 | 2 |  |
| 0 11 110 | Uncertainty in u-component | m/s | 1 | -4096 | 13 | m/s | 1 | 4 |  |
| 0 11 111 | Uncertainty in v-component | m/s | 1 | -4096 | 13 | m/s | 1 | 4 |  |
| 0 11 112 | Uncertainty in w-component | m/s | 2 | -4096 | 13 | m/s | 2 | 4 |  |
| 0 11 113 | Tracking correlation of vector | Numeric | 3 | -1000 | 12 | Numeric | 3 | 4 |  |

Note 64: Downward w-components shall be assigned positive values where units are Pa s-1.

Note 123: South to north v-components shall be assigned positive values.

Note 126: Surface wind direction measured at a station within 1° of the North Pole or within 1° of the South Pole shall be reported in such a way that the azimuth ring shall be aligned with its zero coinciding with the Greenwich 0° meridian.

Note 161: Upward w-components shall be assigned positive values where units are m s-1.

Note 167: West to east u-components shall be assigned positive values.

Note 180: Wind reporting standards: Speed Direction No observation Missing Missing Calm 0 0 Normal observation > 0 1° - 360° Speed only > 0 Missing Direction only Missing 1°-360° "Light and variable" > 0 0