Class 33: Quality information

| FXY | ELEMENT NAME | BUFR UNIT | SCALE | REF VALUE | BITS | CREX UNIT | SCALE | CHARS | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 33 002 | Quality information | Code table | 0 | 0 | 2 | Code table | 0 | 1 |  |
| 0 33 003 | Quality information | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 33 005 | Quality information (AWS data) | Flag table | 0 | 0 | 30 | Flag table | 0 | 10 |  |
| 0 33 006 | Internal measurement status information (AWS) | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 33 007 | Per cent confidence | % | 0 | 0 | 7 | % | 0 | 3 |  |
| 0 33 015 | Data quality-check indicator | Code table | 0 | 0 | 6 | Code table | 0 | 2 |  |
| 0 33 020 | Quality control indication of following value | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 33 021 | Quality of following value | Code table | 0 | 0 | 2 | Code table | 0 | 1 |  |
| 0 33 022 | Quality of buoy satellite transmission | Code table | 0 | 0 | 2 | Code table | 0 | 1 |  |
| 0 33 023 | Quality of buoy location | Code table | 0 | 0 | 2 | Code table | 0 | 1 |  |
| 0 33 024 | Station elevation quality mark (for mobile stations) | Code table | 0 | 0 | 4 | Code table | 0 | 2 |  |
| 0 33 025 | ACARS interpolated values indicator | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 33 026 | Moisture quality | Code table | 0 | 0 | 6 | Code table | 0 | 2 |  |
| 0 33 027 | Location quality class (range of radius of 66 % confidence) | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 33 028 | Snapshot overall quality | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 33 030 | Scan line status flags for ATOVS | Flag table | 0 | 0 | 24 | Flag table | 0 | 8 |  |
| 0 33 031 | Scan line quality flags for ATOVS | Flag table | 0 | 0 | 24 | Flag table | 0 | 8 |  |
| 0 33 032 | Channel quality flags for ATOVS | Flag table | 0 | 0 | 24 | Flag table | 0 | 8 |  |
| 0 33 033 | Field of view quality flags for ATOVS | Flag table | 0 | 0 | 24 | Flag table | 0 | 8 |  |
| 0 33 035 | Manual/automatic quality control | Code table | 0 | 0 | 4 | Code table | 0 | 2 |  |
| 0 33 036 | Nominal confidence threshold | % | 0 | 0 | 7 | % | 0 | 3 |  |
| 0 33 037 | Wind correlation error | Flag table | 0 | 0 | 20 | Flag table | 0 | 7 |  |
| 0 33 038 | Quality flags for ground-based GNSS data | Flag table | 0 | 0 | 10 | Flag table | 0 | 4 |  |
| 0 33 039 | Quality flags for radio occultation data | Flag table | 0 | 0 | 16 | Flag table | 0 | 6 |  |
| 0 33 040 | Confidence interval | % | 0 | 0 | 7 | % | 0 | 3 |  |
| 0 33 041 | Attribute of following value | Code table | 0 | 0 | 2 | Code table | 0 | 1 |  |
| 0 33 042 | Type of limit represented by following value | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 33 043 | AST confidence | Flag table | 0 | 0 | 8 | Flag table | 0 | 3 |  |
| 0 33 044 | ASAR quality information | Flag table | 0 | 0 | 15 | Flag table | 0 | 5 |  |
| 0 33 045 | Probability of following event | % | 0 | 0 | 7 | % | 0 | 3 | Note 168: When defining an event for use with descriptor 0 33 045 or 0 33 046, descriptor 0 33 042 may be employed in order to indicate that the following value is actually abound for a range of values. ¦ Note 173: When using descriptor 0 33 045 or 0 33 046, operator 2 41 000 shall be used in order to define the following event to which the reported probability value applies. |
| 0 33 046 | Conditional probability of following event with respect to specified conditioning event | % | 0 | 0 | 7 | % | 0 | 3 | Note 168: When defining an event for use with descriptor 0 33 045 or 0 33 046, descriptor 0 33 042 may be employed in order to indicate that the following value is actually abound for a range of values. ¦ Note 173: When using descriptor 0 33 045 or 0 33 046, operator 2 41 000 shall be used in order to define the following event to which the reported probability value applies. ¦ Note 174: When using descriptor 0 33 046, operator 2 42 000 shall precede the occurrence of this descriptor in order to define the event upon which the reported probability value is conditioned. |
| 0 33 047 | Measurement confidence data | Flag table | 0 | 0 | 31 | Flag table | 0 | 11 |  |
| 0 33 048 | Confidence measure of SAR inversion | Code table | 0 | 0 | 2 | Code table | 0 | 1 |  |
| 0 33 049 | Confidence measure of wind retrieval | Code table | 0 | 0 | 2 | Code table | 0 | 1 |  |
| 0 33 050 | Global GTSPP quality flag | Code table | 0 | 0 | 4 | Code table | 0 | 2 |  |
| 0 33 052 | S band ocean retracking quality | Flag table | 0 | 0 | 21 | Flag table | 0 | 7 |  |
| 0 33 053 | Ku band ocean retracking quality | Flag table | 0 | 0 | 21 | Flag table | 0 | 7 |  |
| 0 33 054 | Overall quality of retrieved atmospheric profile | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 33 055 | Wind vector quality flag | Flag table | 0 | 0 | 24 | Flag table | 0 | 8 |  |
| 0 33 056 | Sigma-0 quality flag | Flag table | 0 | 0 | 24 | Flag table | 0 | 8 |  |
| 0 33 060 | GqisFlagQual - individual IASI-System quality flag | Code table | 0 | 0 | 2 | Code table | 0 | 1 |  |
| 0 33 061 | GqisQualIndex - indicator for instrument noise performance (contributions from spectral and radiometric calibration) | % | 0 | 0 | 7 | % | 0 | 3 |  |
| 0 33 062 | GqisQualIndexLoc - indicator for geometric quality index | % | 0 | 0 | 7 | % | 0 | 3 |  |
| 0 33 063 | GqisQualIndexRad - indicator for instrument noise performance (contributions from radiometric calibration) | % | 0 | 0 | 7 | % | 0 | 3 |  |
| 0 33 064 | GqisQualIndexSpect - indicator for instrument noise performance (contributions from spectral calibration) | % | 0 | 0 | 7 | % | 0 | 3 |  |
| 0 33 065 | GqisSysTecSondQual - output of system TEC (Technical Expertise Centre) quality function | Numeric | 0 | 0 | 24 | Numeric | 0 | 8 |  |
| 0 33 066 | AMV quality flag | Flag table | 0 | 0 | 24 | Flag table | 0 | 8 |  |
| 0 33 070 | Total ozone quality | Code table | 0 | 0 | 4 | Code table | 0 | 2 |  |
| 0 33 071 | Profile ozone quality | Code table | 0 | 0 | 4 | Code table | 0 | 2 |  |
| 0 33 072 | Ozone error | Code table | 0 | 0 | 5 | Code table | 0 | 2 |  |
| 0 33 075 | Scan-level quality flags | Flag table | 0 | 0 | 13 | Flag table | 0 | 5 |  |
| 0 33 076 | Calibration quality flags | Flag table | 0 | 0 | 9 | Flag table | 0 | 3 |  |
| 0 33 077 | Field-of-view quality flags | Flag table | 0 | 0 | 19 | Flag table | 0 | 7 |  |
| 0 33 078 | Geolocation quality | Code table | 0 | 0 | 4 | Code table | 0 | 2 |  |
| 0 33 079 | Granule level quality flags | Flag table | 0 | 0 | 16 | Flag table | 0 | 6 |  |
| 0 33 080 | Scan level quality flags | Flag table | 0 | 0 | 20 | Flag table | 0 | 7 |  |
| 0 33 081 | Channel data quality flags | Flag table | 0 | 0 | 12 | Flag table | 0 | 4 |  |
| 0 33 082 | Geolocation quality flags | Flag table | 0 | 0 | 16 | Flag table | 0 | 6 |  |
| 0 33 083 | Radiance data quality flags | Flag table | 0 | 0 | 16 | Flag table | 0 | 6 |  |
| 0 33 084 | Pixel level quality flags | Flag table | 0 | 0 | 16 | Flag table | 0 | 6 |  |
| 0 33 085 | Aerosol optical thickness quality flags | Flag table | 0 | 0 | 18 | Flag table | 0 | 6 |  |
| 0 33 086 | Quality of pixel level retrieval | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 33 087 | Extent of satellite within South Atlantic anomaly (based on climatological data) | Code table | 0 | 0 | 4 | Code table | 0 | 1 |  |
| 0 33 088 | Ozone total column quality flag | Flag table | 0 | 0 | 18 | Flag table | 0 | 6 |  |
| 0 33 089 | Noise equivalent delta temperature (NEdT) quality indicators for warm target calibration | K | 2 | 0 | 12 | K | 2 | 4 |  |
| 0 33 090 | NEdT quality indicators for cold target calibration | K | 2 | 0 | 12 | K | 2 | 4 |  |
| 0 33 091 | NEdT quality indicators for overall calibration | K | 2 | 0 | 12 | K | 2 | 4 |  |
| 0 33 092 | Band specific ocean quality flag | Flag table | 0 | 0 | 9 | Flag table | 0 | 3 |  |
| 0 33 093 | Extended quality flags for ground-based GNSS data | Flag table | 0 | 0 | 31 | Flag table | 0 | 31 |  |
| 0 33 094 | Calibration quality control flags | Flag table | 0 | 0 | 24 | Flag table | 0 | 24 |  |
| 0 33 095 | MWI temperatures flag | Flag table | 0 | 0 | 8 | Flag table | 0 | 3 |  |
| 0 33 096 | MWI calibration flag | Flag table | 0 | 0 | 16 | Flag table | 0 | 6 |  |
| 0 33 097 | MWI scan quality flag | Flag table | 0 | 0 | 16 | Flag table | 0 | 6 |  |
| 0 33 098 | MWI data quality flag | Flag table | 0 | 0 | 9 | Flag table | 0 | 3 |  |
| 0 33 099 | MWI navigation status flag | Flag table | 0 | 0 | 16 | Flag table | 0 | 6 |  |
| 0 33 100 | ICI temperatures flag | Flag table | 0 | 0 | 8 | Flag table | 0 | 3 |  |
| 0 33 101 | ICI calibration flag | Flag table | 0 | 0 | 16 | Flag table | 0 | 6 |  |
| 0 33 102 | ICI scan quality flag | Flag table | 0 | 0 | 16 | Flag table | 0 | 6 |  |
| 0 33 103 | ICI data quality flag | Flag table | 0 | 0 | 9 | Flag table | 0 | 3 |  |
| 0 33 104 | ICI navigation status flag | Flag table | 0 | 0 | 16 | Flag table | 0 | 6 |  |
| 0 33 105 | ICI processing flags | Flag table | 0 | 0 | 16 | Flag table | 0 | 6 |  |
| 0 33 106 | MWI processing flags | Flag table | 0 | 0 | 16 | Flag table | 0 | 6 |  |
| 0 33 107 | MWS scan line processing flags | Flag table | 0 | 0 | 16 | Flag table | 0 | 6 |  |
| 0 33 108 | MWS navigation status flags | Flag table | 0 | 0 | 16 | Flag table | 0 | 6 |  |
| 0 33 109 | MWS overall quality flags | Flag table | 0 | 0 | 16 | Flag table | 0 | 6 |  |
| 0 33 110 | MWS channel processing flags | Flag table | 0 | 0 | 9 | Flag table | 0 | 3 |  |
| 0 33 111 | SCA quality flags | Flag table | 0 | 0 | 32 | Flag table | 0 | 11 |  |
| 0 33 112 | Faraday rotation angle (one way) | deg | 2 | -18000 | 16 | deg | 2 | 6 |  |
| 0 33 113 | IASI-NG general quality flags | Flag table | 0 | 0 | 16 | Flag table | 0 | 6 |  |
| 0 33 114 | Out of bounds check | Flag table | 0 | 0 | 17 | Flag table | 0 | 6 |  |
| 0 33 115 | Aerosol optical depth quality information | Code table | 0 | 0 | 5 | Code table | 0 | 2 |  |
| 0 33 116 | First guess in bounds check | Flag table | 0 | 0 | 17 | Flag table | 0 | 6 |  |
| 0 33 117 | In bounds check | Flag table | 0 | 0 | 17 | Flag table | 0 | 6 |  |
| 0 33 118 | Relative error percentage | % | 2 | -10000 | 15 | % | 2 | 5 |  |
| 0 33 119 | Forecast error percentage | % | 0 | -100 | 8 | % | 0 | 3 |  |