Class 01: Identification

| FXY | ELEMENT NAME | BUFR UNIT | SCALE | REF VALUE | BITS | CREX UNIT | SCALE | CHARS | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 001001 | WMO block number | Numeric | 0 | 0 | 7 | Numeric | 0 | 2 |  |
| 001002 | WMO station number | Numeric | 0 | 0 | 10 | Numeric | 0 | 3 |  |
| 001003 | WMO Region number/geographical area | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 001004 | WMO Region sub-area | Numeric | 0 | 0 | 3 | Numeric | 0 | 1 | Note 42: Range of value for parameter 0 04 015 limited from -99 to 99; CREX common sequence D 06 019 being the original sequence with 2 characters only for the corresponding descriptor. |
| 001005 | Buoy/platform identifier | Numeric | 0 | 0 | 17 | Numeric | 0 | 5 |  |
| 001006 | Aircraft flight number | CCITT IA5 | 0 | 0 | 64 | Character | 0 | 8 |  |
| 001007 | Satellite identifier | Code table | 0 | 0 | 10 | Code table | 0 | 4 |  |
| 001008 | Aircraft registration number or other identification | CCITT IA5 | 0 | 0 | 64 | Character | 0 | 8 |  |
| 001009 | Type of commercial aircraft | CCITT IA5 | 0 | 0 | 64 | Character | 0 | 8 |  |
| 001010 | Stationary buoy platform identifier; e.g. C-MAN buoys | CCITT IA5 | 0 | 0 | 64 | Character | 0 | 8 |  |
| 001011 | Ship or mobile land station identifier | CCITT IA5 | 0 | 0 | 72 | Character | 0 | 9 |  |
| 001012 | Direction of motion of moving observing platform | degree true | 0 | 0 | 9 | degree true | 0 | 3 | Note 125: Stationary position of ship shall be reported by 0 01 012 set to 0 and 0 01 013 set to 0. Course of ship unknown (Ds = 9) shall be reported by 0 01 012 set to 509. ¦ Note 60: The time period indicates the duration in minutes over which the measurements have been averaged. The end of the time period is marked by the end of scan (see above). |
| 001013 | Speed of motion of moving observing platform | m/s | 0 | 0 | 10 | m/s | 0 | 3 | Note 60: The time period indicates the duration in minutes over which the measurements have been averaged. The end of the time period is marked by the end of scan (see above). ¦ Note 125: Stationary position of ship shall be reported by 0 01 012 set to 0 and 0 01 013 set to 0. Course of ship unknown (Ds = 9) shall be reported by 0 01 012 set to 509. |
| 001014 | Platform drift speed (high precision) | m/s | 2 | 0 | 10 | m/s | 2 | 4 |  |
| 001015 | Station or site name | CCITT IA5 | 0 | 0 | 160 | Character | 0 | 20 |  |
| 001016 | Satellite sub-identifier | Numeric | 0 | 0 | 16 | Numeric | 0 | 5 |  |
| 001018 | Short station or site name | CCITT IA5 | 0 | 0 | 40 | Character | 0 | 5 |  |
| 001019 | Long station or site name | CCITT IA5 | 0 | 0 | 256 | Character | 0 | 32 |  |
| 001020 | WMO Region sub-area | Numeric | 0 | 0 | 4 | Numeric | 0 | 2 |  |
| 001021 | Synoptic feature identifier | Numeric | 0 | 0 | 14 | Numeric | 0 | 4 |  |
| 001022 | Name of feature | CCITT IA5 | 0 | 0 | 224 | Character | 0 | 28 | Note 72: Time of launch shall be reported with the highest possible accuracy available. If the launch time is not available with second accuracy, the entry for seconds shall be set to zero. |
| 001023 | Observation sequence number | Numeric | 0 | 0 | 9 | Numeric | 0 | 3 |  |
| 001024 | Wind speed source | Code table | 0 | 0 | 5 | Code table | 0 | 2 |  |
| 001025 | Storm identifier | CCITT IA5 | 0 | 0 | 24 | Character | 0 | 3 | Note 148: The storm identifier (descriptor 0 01 025) has the following meaning: the first two characters shall be a numeric sequence number assigned by the originator of the message; the third character is a letter indicating the ocean basin where the storm is located, as follows:W NW Pacific OceanE NE Pacific Ocean to 140°WC NE Pacific Ocean 140°W - 180°WL N Atlantic Ocean, including Caribbean and Gulf of Mexico A N Arabian Sea B Bay of Bengal S S Indian Ocean P S Pacific Ocean F RSMC Nadi's zone in South Pacific U Australia O South China Sea T East China Sea There is no requirement that differing observers coordinate sequence numbers even though they both may be reporting the same storm. |
| 001026 | WMO storm name | CCITT IA5 | 0 | 0 | 64 | Character | 0 | 8 | Note 43: Separation of single level satellite data into sets of BUFR messages helps compression and results inefficient data transmission and storage. |
| 001027 | WMO long storm name | CCITT IA5 | 0 | 0 | 80 | Character | 0 | 10 | Note 182: WMO long storm name (descriptor 0 01 027): the storm name "Nameless" shall be used in those cases where an identifiable tropical disturbance has not reached tropical storm strength and has not been assigned an official name. |
| 001028 | Aerosol optical depth (AOD) source | Code table | 0 | 0 | 5 | Code table | 0 | 2 |  |
| 001029 | SSI source | Code table | 0 | 0 | 5 | Code table | 0 | 2 |  |
| 001030 | Numerical model identifier | CCITT IA5 | 0 | 0 | 128 | Character | 0 | 16 | Note 153: The value of this feature could be a string of characters, which contain the name of the model and other useful elements such as the model mesh. |
| 001031 | Identification of originating/generating centre | Code table | 0 | 0 | 16 | Code table | 0 | 5 | Note 44: Sequence 3 09 030 is deprecated because of incorrect usage of descriptor 0 04 015; sequence 3 09 031 should be used instead. |
| 001032 | Generating application | Code table defined by originating/generating centre | 0 | 0 | 8 | Code table | 0 | 3 | Note 175: Where a centre other than the originating centre generates quality information, replacement or substitute values, and/or statistical information, the centre may be indicated by using 0 01 033. ¦ Note 21: Front direction (towards which the front is moving) must always be given as it is needed for plotting purposes. A front direction with a front speed of zero would indicate a slow front. A value in the code table exists to represent a quasi-stationary front. ¦ Note 38: n/a |
| 001033 | Identification of originating/generating centre | Common Code table C-1 | 0 | 0 | 8 | Common Code table C-1 | 0 | 3 |  |
| 001034 | Identification of originating/generating sub-centre | Common Code table C-12 | 0 | 0 | 8 | Common Code table C-12 | 0 | 3 |  |
| 001035 | Originating centre | Common Code table C-11 | 0 | 0 | 16 | Common Code table C-11 | 0 | 5 |  |
| 001036 | Agency in charge of operating the observing platform | Code table | 0 | 0 | 20 | Code table | 0 | 7 |  |
| 001037 | SIGMET sequence identifier | CCITT IA5 | 0 | 0 | 24 | Character | 0 | 3 |  |
| 001038 | Source of sea ice fraction | Code table | 0 | 0 | 5 | Code table | 0 | 2 |  |
| 001039 | Graphical Area Forecast (GFA) sequence identifier | CCITT IA5 | 0 | 0 | 40 | Character | 0 | 5 |  |
| 001040 | Processing centre ID code | CCITT IA5 | 0 | 0 | 48 | Character | 0 | 6 |  |
| 001041 | Absolute platform velocity - first component | m/s | 5 | -1073741824 | 31 | m/s | 5 | 10 | Note 134: The components of absolute platform velocity (0 01 041, 0 01 042, 0 01 043) are defined as follows:- First component: From the Earth's centre to 0 degree longitude at the Equator: velocity of the platform along this line relative to the Earth's centre.- Second component: From the Earth's centre to 90 degrees East longitude at the Equator: velocity of the platform along this line relative to the Earth's centre.- Third component: From the Earth's centre to the north pole: velocity of the platform along this line relative to the Earth's centre. ¦ Note 154: The values for descriptors 0 01 041, 0 01 042 and 0 01 043 have been chosen to be suitable for polar orbiting satellites in approximately Sun-synchronous orbits. Geostationary orbits would require greater data widths for distance and slightly less for speed. ¦ Note 95: Left handed x, y and z axes have been chosen for descriptors 0 01 041, 0 01 042 and 0 01 043. |
| 001042 | Absolute platform velocity - second component | m/s | 5 | -1073741824 | 31 | m/s | 5 | 10 | Note 134: The components of absolute platform velocity (0 01 041, 0 01 042, 0 01 043) are defined as follows:- First component: From the Earth's centre to 0 degree longitude at the Equator: velocity of the platform along this line relative to the Earth's centre.- Second component: From the Earth's centre to 90 degrees East longitude at the Equator: velocity of the platform along this line relative to the Earth's centre.- Third component: From the Earth's centre to the north pole: velocity of the platform along this line relative to the Earth's centre. ¦ Note 154: The values for descriptors 0 01 041, 0 01 042 and 0 01 043 have been chosen to be suitable for polar orbiting satellites in approximately Sun-synchronous orbits. Geostationary orbits would require greater data widths for distance and slightly less for speed. ¦ Note 95: Left handed x, y and z axes have been chosen for descriptors 0 01 041, 0 01 042 and 0 01 043. |
| 001043 | Absolute platform velocity - third component | m/s | 5 | -1073741824 | 31 | m/s | 5 | 10 | Note 134: The components of absolute platform velocity (0 01 041, 0 01 042, 0 01 043) are defined as follows:- First component: From the Earth's centre to 0 degree longitude at the Equator: velocity of the platform along this line relative to the Earth's centre.- Second component: From the Earth's centre to 90 degrees East longitude at the Equator: velocity of the platform along this line relative to the Earth's centre.- Third component: From the Earth's centre to the north pole: velocity of the platform along this line relative to the Earth's centre. ¦ Note 154: The values for descriptors 0 01 041, 0 01 042 and 0 01 043 have been chosen to be suitable for polar orbiting satellites in approximately Sun-synchronous orbits. Geostationary orbits would require greater data widths for distance and slightly less for speed. ¦ Note 95: Left handed x, y and z axes have been chosen for descriptors 0 01 041, 0 01 042 and 0 01 043. |
| 001044 | Standard generating application | Code table | 0 | 0 | 8 | Code table | 0 | 3 |  |
| 001050 | Platform transmitter ID number | Numeric | 0 | 0 | 17 | Numeric | 0 | 6 |  |
| 001051 | Platform transmitter ID number | CCITT IA5 | 0 | 0 | 96 | Character | 0 | 12 |  |
| 001052 | Platform transmitter ID | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 001053 | Tsunameter report sequence number triggered by a tsunami event | Numeric | 0 | 0 | 7 | Numeric | 0 | 2 |  |
| 001060 | Aircraft reporting point (Beacon identifier) | CCITT IA5 | 0 | 0 | 64 | Character | 0 | 8 |  |
| 001062 | Short ICAO location indicator | CCITT IA5 | 0 | 0 | 32 | Character | 0 | 4 |  |
| 001063 | ICAO location indicator | CCITT IA5 | 0 | 0 | 64 | Character | 0 | 8 |  |
| 001064 | Runway designator | CCITT IA5 | 0 | 0 | 32 | Character | 0 | 4 |  |
| 001065 | ICAO region identifier | CCITT IA5 | 0 | 0 | 256 | Character | 0 | 32 |  |
| 001075 | Tide station identification | CCITT IA5 | 0 | 0 | 40 | Character | 0 | 5 |  |
| 001079 | Unique identifier for the profile | CCITT IA5 | 0 | 0 | 64 | Character | 0 | 8 |  |
| 001080 | Ship line number according to SOOP | CCITT IA5 | 0 | 0 | 32 | Character | 0 | 4 |  |
| 001081 | Radiosonde serial number | CCITT IA5 | 0 | 0 | 160 | Character | 0 | 20 |  |
| 001082 | Radiosonde ascension number | Numeric | 0 | 0 | 14 | Numeric | 0 | 4 | Note 45: Some types of stations do not have the ability to identify the surface condition accurately. They can only report conditions such as "not dry" or "glazed". The code table for road surface conditions has been adjusted accordingly. |
| 001083 | Radiosonde release number | Numeric | 0 | 0 | 3 | Numeric | 0 | 1 | Note 45: Some types of stations do not have the ability to identify the surface condition accurately. They can only report conditions such as "not dry" or "glazed". The code table for road surface conditions has been adjusted accordingly. |
| 001085 | Observing platform manufacturer's model | CCITT IA5 | 0 | 0 | 160 | Character | 0 | 20 |  |
| 001086 | Observing platform manufacturer's serial number | CCITT IA5 | 0 | 0 | 256 | Character | 0 | 32 |  |
| 001087 | WMO marine observing platform extended identifier | Numeric | 0 | 0 | 23 | Numeric | 0 | 7 |  |
| 001090 | Technique for making up initial perturbations | Code table | 0 | 0 | 8 | Code table | 0 | 3 |  |
| 001091 | Ensemble member number | Numeric | 0 | 0 | 10 | Numeric | 0 | 4 |  |
| 001092 | Type of ensemble forecast | Code table | 0 | 0 | 8 | Code table | 0 | 3 |  |
| 001093 | Balloon lot number | CCITT IA5 | 0 | 0 | 96 | Character | 0 | 12 |  |
| 001094 | WBAN number | Numeric | 0 | 0 | 17 | Numeric | 0 | 5 |  |
| 001095 | Observer identification | CCITT IA5 | 0 | 0 | 32 | Character | 0 | 4 |  |
| 001096 | Station acquisition | CCITT IA5 | 0 | 0 | 160 | Character | 0 | 20 |  |
| 001099 | Unique product definition | CCITT IA5 | 0 | 0 | 248 | Character | 0 | 31 |  |
| 001101 | State identifier | Code table | 0 | 0 | 10 | Code table | 0 | 3 |  |
| 001102 | National station number | Numeric | 0 | 0 | 30 | Numeric | 0 | 9 |  |
| 001103 | IMO Number. Unique Lloyd's register | Numeric | 0 | 0 | 24 | Numeric | 0 | 7 |  |
| 001104 | State/federal state identifier | CCITT IA5 | 0 | 0 | 32 | Character | 0 | 4 |  |
| 001105 | Highway designator | CCITT IA5 | 0 | 0 | 40 | Character | 0 | 5 |  |
| 001106 | Location along highway as indicated by position markers | m | -2 | 0 | 14 | m | -2 | 5 |  |
| 001110 | Aircraft tail number | CCITT IA5 | 0 | 0 | 48 | Character | 0 | 6 |  |
| 001111 | Origination airport | CCITT IA5 | 0 | 0 | 24 | Character | 0 | 3 |  |
| 001112 | Destination airport | CCITT IA5 | 0 | 0 | 24 | Character | 0 | 3 |  |
| 001113 | Template version number defined by originating centre | Numeric | 1 | 0 | 9 | Numeric | 1 | 3 |  |
| 001114 | Encrypted ship or mobile land station identifier (base64 encoding) | CCITT IA5 | 0 | 0 | 352 | Character | 0 | 44 |  |
| 001115 | Identifier of the cruise or mission under which the data were collected | CCITT IA5 | 0 | 0 | 160 | Character | 0 | 20 |  |
| 001124 | Grid point identifier | Numeric | 0 | 0 | 24 | Numeric | 0 | 8 |  |
| 001125 | WIGOS identifier series | Numeric | 0 | 0 | 4 | Numeric | 0 | 2 |  |
| 001126 | WIGOS issuer of identifier | Numeric | 0 | 0 | 16 | Numeric | 0 | 5 |  |
| 001127 | WIGOS issue number | Numeric | 0 | 0 | 16 | Numeric | 0 | 5 |  |
| 001128 | WIGOS local identifier (character) | CCITT IA5 | 0 | 0 | 128 | Character | 0 | 16 |  |
| 001130 | Storm identifier (long) | CCITT IA5 | 0 | 0 | 32 |  |  |  |  |
| 001144 | Snapshot identifier | Numeric | 0 | 0 | 31 | Numeric | 0 | 10 |  |
| 001145 | Light source identifier | Numeric | 0 | -8 | 20 | Numeric | 0 | 7 | Note 185: Positive value of descriptor 0 01 145 corresponds to Hipparcos identifier; 0 is Sun, -2 is Venus, -4 is Mars, -5 is Jupiter, -6 is Saturn, -7 is Earth's moon, and -8 is Bright limb. |
| 001150 | Coordinate reference system | Code table | 0 | 0 | 16 | Code table | 0 | 5 |  |
| 001151 | Fixed mean sea-level reference datum | Code table | 0 | 0 | 12 | Code table | 0 | 4 |  |
| 001152 | Semi-major axis of rotation ellipsoid | m | 2 | 0 | 31 | m | 2 | 11 |  |
| 001153 | Semi-minor axis of rotation ellipsoid | m | 2 | 0 | 31 | m | 2 | 11 |  |
| 001154 | Sensor identifier | Numeric | 0 | 0 | 12 | Numeric | 0 | 4 |  |
| 001155 | Retrieval identifier | Code table | 0 | 0 | 8 | Code table | 0 | 3 |  |
| 001156 | Balloon flight number | CCITT IA5 | 0 | 0 | 64 |  |  |  |  |
| 001157 | Balloon registration number or other identification | CCITT IA5 | 0 | 0 | 80 |  |  |  |  |
| 001158 | Type of balloon envelope | Code table | 0 | 0 | 4 |  |  |  |  |
| 001159 | Typical size of balloon envelope | m | 1 | 0 | 12 |  |  |  | Note 195: In descriptor 0 01 159, report the diameter of spherical balloons and report the larger dimension (length, height) of other shapes. |