Category 02: Meteorological sequences common to surface data

| FXY1 | TITLE | SUBTITLE | FXY2 | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| 3 02 001 | (Pressure and 3-hour pressure change) |  | 0 10 004 | Pressure | Station level |  |
|  |  |  | 0 10 051 | Pressure reduced to mean sea level |  |  |
|  |  |  | 0 10 061 | 3-hour pressure change |  |  |
|  |  |  | 0 10 063 | Characteristic of pressure tendency |  |  |
| 3 02 002 | (High altitude station) |  | 0 10 004 | Pressure | Station level |  |
|  |  |  | 0 07 004 | Pressure | Pressure level |  |
|  |  |  | 0 10 003 | Geopotential | Pressure level |  |
|  |  |  | 0 10 061 | 3-hour pressure change |  |  |
|  |  |  | 0 10 063 | Characteristic of pressure tendency |  |  |
| 3 02 003 | (Wind, temperature, humidity, visibility, weather) |  | 0 11 011 | Wind direction at 10 m |  |  |
|  |  |  | 0 11 012 | Wind speed at 10 m |  |  |
|  |  |  | 0 12 004 | Air temperature at 2 m |  |  |
|  |  |  | 0 12 006 | Dewpoint temperature at 2 m |  |  |
|  |  |  | 0 13 003 | Relative humidity |  |  |
|  |  |  | 0 20 001 | Horizontal visibility |  |  |
|  |  |  | 0 20 003 | Present weather |  |  |
|  |  |  | 0 20 004 | Past weather (1) |  |  |
|  |  |  | 0 20 005 | Past weather (2) |  |  |
| 3 02 004 | (General cloud information) |  | 0 20 010 | Cloud cover (total) |  |  |
|  |  |  | 0 08 002 | Vertical significance (surface observations) |  |  |
|  |  |  | 0 20 011 | Cloud amount |  |  |
|  |  |  | 0 20 013 | Height of base of cloud |  |  |
|  |  |  | 0 20 012 | Cloud type |  |  |
|  |  |  | 0 20 012 | Cloud type |  |  |
|  |  |  | 0 20 012 | Cloud type |  |  |
| 3 02 005 | (Cloud layer) |  | 0 08 002 | Vertical significance (surface observations) |  |  |
|  |  |  | 0 20 011 | Cloud amount |  |  |
|  |  |  | 0 20 012 | Cloud type |  |  |
|  |  |  | 0 20 013 | Height of base of cloud |  |  |
| 3 02 006 | (Pressure and 24-hour pressure change) |  | 0 10 004 | Pressure | Station level |  |
|  |  |  | 0 10 051 | Pressure reduced to mean sea level |  |  |
|  |  |  | 0 10 062 | 24-hour pressure change |  |  |
|  |  |  | 0 10 063 | Characteristic of pressure tendency |  |  |
| 3 02 011 | (Low altitude station) |  | 3 02 001 | Pressure and 3-hour pressure change |  |  |
|  |  |  | 3 02 003 | Wind, temperature, humidity, visibility, weather |  |  |
|  |  |  | 3 02 004 | General cloud information | Significant cloud layer |  |
| 3 02 012 | (High altitude station) |  | 3 02 002 | High altitude station | Pressure and pressure change |  |
|  |  |  | 3 02 003 | Wind, temperature, humidity, visibility, weather |  |  |
|  |  |  | 3 02 004 | General cloud information | Significant cloud layer |  |
| 3 02 013 | (Basic surface report) |  | 3 02 006 | Pressure and 24-hour pressure change |  |  |
|  |  |  | 3 02 003 | Wind, temperature, humidity, visibility, weather |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 3 02 005 | Cloud layer |  |  |
| 3 02 021 | (Waves) |  | 0 22 001 | Direction of waves |  |  |
|  |  |  | 0 22 011 | Period of waves |  |  |
|  |  |  | 0 22 021 | Height of waves |  |  |
| 3 02 022 | (Wind waves) |  | 0 22 002 | Direction of wind waves |  |  |
|  |  |  | 0 22 012 | Period of wind waves |  |  |
|  |  |  | 0 22 022 | Height of wind waves |  |  |
| 3 02 023 | (Swell waves) |  | 0 22 003 | Direction of swell waves |  |  |
|  |  |  | 0 22 013 | Period of swell waves |  |  |
|  |  |  | 0 22 023 | Height of swell waves |  |  |
| 3 02 024 | (Wind and swell waves) |  | 3 02 022 | Wind waves |  |  |
|  |  |  | 1 01 002 | Replicate 1 descriptor 2 times |  |  |
|  |  |  | 3 02 023 | Swell waves | 2 systems of swell |  |
| 3 02 031 | (Pressure information) |  | 3 02 001 | Pressure and 3-hour pressure change |  |  |
|  |  |  | 0 10 062 | 24-hour pressure change |  |  |
|  |  |  | 0 07 004 | Pressure | Standard level |  |
|  |  |  | 0 10 009 | Geopotential height |  |  |
| 3 02 032 | (Temperature and humidity data) |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Temperature and humidity measurement |  |
|  |  |  | 0 12 101 | Temperature/air temperature | Scale: 2 |  |
|  |  |  | 0 12 103 | Dewpoint temperature | Scale: 2 |  |
|  |  |  | 0 13 003 | Relative humidity |  |  |
| 3 02 033 | (Visibility data) |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Visibility measurement |  |
|  |  |  | 0 20 001 | Horizontal visibility |  |  |
| 3 02 034 | (Precipitation past 24 hours) |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Precipitation measurement |  |
|  |  |  | 0 13 023 | Total precipitation past 24 hours |  |  |
| 3 02 035 | (Basic synoptic "instantaneous" data) |  | 3 02 032 | Temperature and humidity data |  |  |
|  |  |  | 3 02 033 | Visibility data |  |  |
|  |  |  | 3 02 034 | Precipitation past 24 hours |  |  |
|  |  |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
|  |  |  | 3 02 004 | General cloud information |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 3 02 005 | Cloud layer | Individual cloud layer or mass |  |
| 3 02 036 | (Clouds with bases below station level) |  | 1 05 000 | Delayed replication of 5 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 08 002 | Vertical significance (surface observations) |  |  |
|  |  |  | 0 20 011 | Cloud amount |  |  |
|  |  |  | 0 20 012 | Cloud type |  |  |
|  |  |  | 0 20 014 | Height of top of cloud |  |  |
|  |  |  | 0 20 017 | Cloud top description |  |  |
| 3 02 037 | (State of ground, snow depth, ground minimum temperature) |  | 0 20 062 | State of the ground (with or without snow) |  |  |
|  |  |  | 0 13 013 | Total snow depth |  |  |
|  |  |  | 0 12 113 | Ground minimum temperature, past 12 hours | Scale: 2 |  |
| 3 02 038 | (Present and past weather) |  | 0 20 003 | Present weather |  |  |
|  |  |  | 0 04 024 | Time period or displacement | Hours |  |
|  |  |  | 0 20 004 | Past weather (1) |  |  |
|  |  |  | 0 20 005 | Past weather (2) |  |  |
| 3 02 039 | (Sunshine data (from 1 hour and 24 hour period)) |  | 0 04 024 | Time period or displacement | Hours |  |
|  |  |  | 0 14 031 | Total sunshine |  |  |
| 3 02 040 | (Precipitation measurement) |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Precipitation measurement |  |
|  |  |  | 1 02 002 | Replicate 2 descriptors 2 times |  |  |
|  |  |  | 0 04 024 | Time period or displacement | Hours |  |
|  |  |  | 0 13 011 | Total precipitation/total water equivalent |  |  |
| 3 02 041 | (Extreme temperature data) |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Temperature measurement |  |
|  |  |  | 0 04 024 | Time period or displacement |  |  |
|  |  |  | 0 04 024 | Time period or displacement |  | Note 78: Within RA IV, the maximum temperature at 1200 UTC is reported for the previous calendar day (i.e. the ending time of the period is not equal to the nominal time of the report). To construct the required time range, descriptor 0 04 024 has to be included two times. If the period ends at the nominal time of the report, value of the second 0 04 024 shall be set to 0 ¦ Note 77: Within RA III, the maximum daytime temperature and the minimum night-time temperature is reported (i.e. the ending time of the period may not be equal to the nominal time of the report). To construct the required time range, descriptor 0 04 024 has to be included two times. If the period ends at the nominal time of the report,value of the second 0 04 024 shall be set to 0. |
|  |  |  | 0 12 111 | Maximum temperature, at height and over period specified | Scale: 2 |  |
|  |  |  | 0 04 024 | Time period or displacement |  |  |
|  |  |  | 0 04 024 | Time period or displacement |  | Note 77: Within RA III, the maximum daytime temperature and the minimum night-time temperature is reported (i.e. the ending time of the period may not be equal to the nominal time of the report). To construct the required time range, descriptor 0 04 024 has to be included two times. If the period ends at the nominal time of the report,value of the second 0 04 024 shall be set to 0. |
|  |  |  | 0 12 112 | Minimum temperature, at height and over period specified | Scale: 2 |  |
| 3 02 042 | (Wind data) |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Wind measurement |  |
|  |  |  | 0 02 002 | Type of instrumentation for wind measurement |  |  |
|  |  |  | 0 08 021 | Time significance | = 2 Time averaged |  |
|  |  |  | 0 04 025 | Time period or displacement | = -10 minutes, or number of minutes after a significant change of wind |  |
|  |  |  | 0 11 001 | Wind direction |  |  |
|  |  |  | 0 11 002 | Wind speed |  |  |
|  |  |  | 0 08 021 | Time significance | Set to missing |  |
|  |  |  | 1 03 002 | Replicate 3 descriptors 2 times |  |  |
|  |  |  | 0 04 025 | Time period or displacement | Minutes |  |
|  |  |  | 0 11 043 | Maximum wind gust direction |  |  |
|  |  |  | 0 11 041 | Maximum wind gust speed |  |  |
| 3 02 043 | (Basic synoptic "period" data) |  | 3 02 038 | Present and past weather |  |  |
|  |  |  | 1 01 002 | Replicate 1 descriptor 2 times |  |  |
|  |  |  | 3 02 039 | Sunshine data (from 1 hour and 24 hour period) |  |  |
|  |  |  | 3 02 040 | Precipitation measurement |  |  |
|  |  |  | 3 02 041 | Extreme temperature data |  |  |
|  |  |  | 3 02 042 | Wind data |  |  |
|  |  |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 3 02 044 | (Evaporation data) |  | 0 04 024 | Time period or displacement | Hours |  |
|  |  |  | 0 02 004 | Type of instrumentation for evaporation measurement or type of crop for which evapotranspiration is reported |  |  |
|  |  |  | 0 13 033 | Evaporation/evapotranspiration |  |  |
| 3 02 045 | (Radiation data (from 1 hour and 24 hour period)) |  | 0 04 024 | Time period or displacement | Hours |  |
|  |  |  | 0 14 002 | Long-wave radiation, integrated over period specified |  |  |
|  |  |  | 0 14 004 | Short-wave radiation, integrated over period specified |  |  |
|  |  |  | 0 14 016 | Net radiation, integrated over period specified |  |  |
|  |  |  | 0 14 028 | Global solar radiation (high accuracy), integrated over period specified |  |  |
|  |  |  | 0 14 029 | Diffuse solar radiation (high accuracy), integrated over period specified |  |  |
|  |  |  | 0 14 030 | Direct solar radiation (high accuracy), integrated over period specified |  |  |
| 3 02 046 | (Temperature change) |  | 0 04 024 | Time period or displacement |  |  |
|  |  |  | 0 04 024 | Time period or displacement |  | Note 73: To construct the required time range, descriptor 0 04 024 has to be included two times. |
|  |  |  | 0 12 049 | Temperature change over specified period |  |  |
| 3 02 047 | (Direction of cloud drift) |  | 1 02 003 | Replicate 2 descriptors 3 times |  |  |
|  |  |  | 0 08 002 | Vertical significance (surface observations) |  |  |
|  |  |  | 0 20 054 | True direction from which a phenomenon or clouds are moving or in which they are observed |  |  |
| 3 02 048 | (Direction and elevation of cloud) |  | 0 05 021 | Bearing or azimuth |  |  |
|  |  |  | 0 07 021 | Elevation | Elevation angle |  |
|  |  |  | 0 20 012 | Cloud type |  |  |
|  |  |  | 0 05 021 | Bearing or azimuth | Set to missing (cancel) |  |
|  |  |  | 0 07 021 | Elevation | Set to missing (cancel) |  |
| 3 02 049 | (Cloud information reported with vertical soundings) |  | 0 08 002 | Vertical significance (surface observations) |  |  |
|  |  |  | 0 20 011 | Cloud amount | Low or middle clouds Nh |  |
|  |  |  | 0 20 013 | Height of base of cloud | h |  |
|  |  |  | 0 20 012 | Cloud type | Low clouds CL |  |
|  |  |  | 0 20 012 | Cloud type | Middle clouds CM |  |
|  |  |  | 0 20 012 | Cloud type | High clouds CH |  |
|  |  |  | 0 08 002 | Vertical significance (surface observations) | Set to missing |  |
| 3 02 050 | (Radiosonde surface observation) |  | 0 08 041 | Data significance | = 5 Surface observation displacement from launch point |  |
|  |  |  | 0 05 021 | Bearing or azimuth |  |  |
|  |  |  | 0 07 005 | Height increment |  |  |
|  |  |  | 2 02 130 | Change scale |  |  |
|  |  |  | 0 06 021 | Distance |  |  |
|  |  |  | 2 02 000 | Change scale | Cancel |  |
|  |  |  | 0 08 041 | Data significance | = 4 Surface observation |  |
|  |  |  | 2 01 131 | Change data width |  |  |
|  |  |  | 2 02 129 | Change scale |  |  |
|  |  |  | 0 02 115 | Type of surface observing equipment |  |  |
|  |  |  | 0 10 004 | Pressure |  |  |
|  |  |  | 0 02 115 | Type of surface observing equipment |  |  |
|  |  |  | 0 13 003 | Relative humidity |  |  |
|  |  |  | 2 02 000 | Change scale | Cancel |  |
|  |  |  | 2 01 000 | Change data width | Cancel |  |
|  |  |  | 0 02 115 | Type of surface observing equipment |  |  |
|  |  |  | 0 11 001 | Wind direction |  |  |
|  |  |  | 0 11 002 | Wind speed |  |  |
|  |  |  | 0 02 115 | Type of surface observing equipment |  |  |
|  |  |  | 1 02 002 | Replicate 2 descriptors 2 times |  |  |
|  |  |  | 0 12 101 | Temperature/air temperature |  |  |
|  |  |  | 0 04 024 | Time period or displacement | Hours |  |
|  |  |  | 0 02 115 | Type of surface observing equipment |  |  |
|  |  |  | 0 12 103 | Dewpoint temperature |  |  |
|  |  |  | 0 12 102 | Wet-bulb temperature |  |  |
|  |  |  | 1 01 003 | Replicate 1 descriptor 3 times |  |  |
|  |  |  | 0 20 012 | Cloud type |  |  |
|  |  |  | 0 20 011 | Cloud amount |  |  |
|  |  |  | 0 20 013 | Height of base of cloud |  |  |
|  |  |  | 1 01 002 | Replicate 1 descriptor 2 times |  |  |
|  |  |  | 0 20 003 | Present weather |  |  |
| 3 02 051 |  |  | 0 10 004 | Pressure |  |  |
|  |  |  | 0 10 051 | Pressure reduced to mean sea level |  |  |
|  |  |  | 0 07 004 | Pressure | Vertical location |  |
|  |  |  | 0 10 003 | Geopotential |  |  |
|  |  |  | 0 12 004 | Air temperature at 2 m |  |  |
|  |  |  | 0 12 051 | Standard deviation temperature |  |  |
|  |  |  | 0 12 016 | Maximum temperature at 2 m, past 24 hours |  |  |
|  |  |  | 0 12 017 | Minimum temperature at 2 m, past 24 hours |  |  |
|  |  |  | 0 13 004 | Vapour pressure |  |  |
|  |  |  | 1 02 004 | Replicate 2 descriptors 4 times |  |  |
|  |  |  | 0 08 051 | Qualifier for number of missing values in calculation of statistic |  |  |
|  |  |  | 0 08 020 | Total number of missing entities (with respect to accumulation or average) |  |  |
| 3 02 052 | (Ship temperature and humidity data) |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Temperature and humidity measurement |  |
|  |  |  | 0 07 033 | Height of sensor above water surface | Temperature and humidity measurement |  |
|  |  |  | 0 12 101 | Temperature/air temperature | Scale: 2 |  |
|  |  |  | 0 02 039 | Method of wet-bulb temperature measurement |  |  |
|  |  |  | 0 12 102 | Wet-bulb temperature | Scale: 2 |  |
|  |  |  | 0 12 103 | Dewpoint temperature | Scale: 2 |  |
|  |  |  | 0 13 003 | Relative humidity |  |  |
| 3 02 053 | (Ship visibility data) |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Visibility measurement |  |
|  |  |  | 0 07 033 | Height of sensor above water surface | Visibility measurement |  |
|  |  |  | 0 20 001 | Horizontal visibility |  |  |
| 3 02 054 | (Ship "instantaneous" data) |  | 3 02 052 | Ship temperature and humidity data |  |  |
|  |  |  | 3 02 053 | Ship visibility data |  |  |
|  |  |  | 0 07 033 | Height of sensor above water surface | Set to missing (cancel) |  |
|  |  |  | 3 02 034 | Precipitation past 24 hours |  |  |
|  |  |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
|  |  |  | 3 02 004 | General cloud information |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 3 02 005 | Cloud layer |  |  |
| 3 02 055 | (Icing and ice) |  | 0 20 031 | Ice deposit (thickness) |  |  |
|  |  |  | 0 20 032 | Rate of ice accretion (estimated) |  |  |
|  |  |  | 0 20 033 | Cause of ice accretion |  |  |
|  |  |  | 0 20 034 | Sea ice concentration |  |  |
|  |  |  | 0 20 035 | Amount and type of ice |  |  |
|  |  |  | 0 20 036 | Ice situation |  |  |
|  |  |  | 0 20 037 | Ice development |  |  |
|  |  |  | 0 20 038 | Bearing of ice edge |  |  |
| 3 02 056 | (Sea/water temperature) |  | 0 02 038 | Method of water temperature and/or salinity measurement |  |  |
|  |  |  | 0 07 063 | Depth below sea/water surface (cm) | Sea-surface temperature measurement |  |
|  |  |  | 0 22 043 | Sea/water temperature |  |  |
|  |  |  | 0 07 063 | Depth below sea/water surface (cm) | Set to missing (cancel) |  |
| 3 02 057 | (Ship marine data) |  | 3 02 056 | Sea/water temperature | Sea-surface temperature, method of measurement, and depth below sea surface |  |
|  |  |  | 3 02 021 | Waves |  |  |
|  |  |  | 3 02 024 | Wind and swell waves |  |  |
| 3 02 058 | (Ship extreme temperature data) |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Temperature measurement |  |
|  |  |  | 0 07 033 | Height of sensor above water surface | Temperature measurement |  |
|  |  |  | 0 04 024 | Time period or displacement |  |  |
|  |  |  | 0 04 024 | Time period or displacement |  | Note 78: Within RA IV, the maximum temperature at 1200 UTC is reported for the previous calendar day (i.e. the ending time of the period is not equal to the nominal time of the report). To construct the required time range, descriptor 0 04 024 has to be included two times. If the period ends at the nominal time of the report, value of the second 0 04 024 shall be set to 0 ¦ Note 77: Within RA III, the maximum daytime temperature and the minimum night-time temperature is reported (i.e. the ending time of the period may not be equal to the nominal time of the report). To construct the required time range, descriptor 0 04 024 has to be included two times. If the period ends at the nominal time of the report,value of the second 0 04 024 shall be set to 0. |
|  |  |  | 0 12 111 | Maximum temperature, at height and over period specified | Scale: 2 |  |
|  |  |  | 0 04 024 | Time period or displacement |  |  |
|  |  |  | 0 04 024 | Time period or displacement |  | Note 77: Within RA III, the maximum daytime temperature and the minimum night-time temperature is reported (i.e. the ending time of the period may not be equal to the nominal time of the report). To construct the required time range, descriptor 0 04 024 has to be included two times. If the period ends at the nominal time of the report,value of the second 0 04 024 shall be set to 0. |
|  |  |  | 0 12 112 | Minimum temperature, at height and over period specified | Scale: 2 |  |
| 3 02 059 | (Ship wind data) |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Wind measurement |  |
|  |  |  | 0 07 033 | Height of sensor above water surface | Wind measurement |  |
|  |  |  | 0 02 002 | Type of instrumentation for wind measurement |  |  |
|  |  |  | 0 08 021 | Time significance | = 2 Time averaged |  |
|  |  |  | 0 04 025 | Time period or displacement | = -10 minutes, or number of minutes after a significant change of wind |  |
|  |  |  | 0 11 001 | Wind direction |  |  |
|  |  |  | 0 11 002 | Wind speed |  |  |
|  |  |  | 0 08 021 | Time significance | Set to missing |  |
|  |  |  | 1 03 002 | Replicate 3 descriptors 2 times |  |  |
|  |  |  | 0 04 025 | Time period or displacement | Minutes |  |
|  |  |  | 0 11 043 | Maximum wind gust direction |  |  |
|  |  |  | 0 11 041 | Maximum wind gust speed |  |  |
| 3 02 060 | (Ship "period" data) |  | 3 02 038 | Present and past weather |  |  |
|  |  |  | 3 02 040 | Precipitation measurement |  |  |
|  |  |  | 3 02 058 | Ship extreme temperature data |  |  |
|  |  |  | 3 02 059 | Ship wind data |  |  |
| 3 02 062 | (Ship "instantaneous" data) |  | 0 25 188 | Method for reducing pressure to sea level |  |  |
|  |  |  | 3 02 001 | Pressure and 3-hour pressure change |  |  |
|  |  |  | 3 02 093 | Extended ship temperature and humidity data |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 3 02 053 | Ship visibility data |  |  |
|  |  |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
|  |  |  | 0 07 033 | Height of sensor above water surface | Set to missing (cancel) |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 3 02 004 | General cloud information |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 3 02 005 | Cloud layer |  |  |
|  |  |  | 0 08 002 | Vertical significance (surface observations) | Set to missing (cancel) |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 3 02 055 | Icing and ice |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 3 02 056 | Sea/water temperature |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 3 02 021 | Waves |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 3 02 024 | Wind and swell waves |  |  |
| 3 02 063 | (Ship "period" data) |  | 3 02 038 | Present and past weather |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 3 02 040 | Precipitation measurement |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 3 02 034 | Precipitation past 24 hours |  |  |
|  |  |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 3 02 058 | Ship extreme temperature data |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 3 02 064 | Ship or other marine platform wind data |  |  |
| 3 02 064 | (Ship or other marine platform wind data) |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
|  |  |  | 0 07 033 | Height of sensor above water surface |  |  |
|  |  |  | 0 02 002 | Type of instrumentation for wind measurement |  |  |
|  |  |  | 0 08 021 | Time significance | = 2 Time averaged |  |
|  |  |  | 0 04 025 | Time period or displacement |  |  |
|  |  |  | 0 11 001 | Wind direction |  |  |
|  |  |  | 0 11 002 | Wind speed |  |  |
|  |  |  | 0 08 021 | Time significance | Set to missing (cancel) |  |
|  |  |  | 1 03 000 | Delayed replication of 3 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 04 025 | Time period or displacement |  |  |
|  |  |  | 0 11 043 | Maximum wind gust direction |  |  |
|  |  |  | 0 11 041 | Maximum wind gust speed |  |  |
| 3 02 065 | (CFOSAT (Chinese-French Oceanography Satellite) : SWIM off-nadir data) |  | 3 01 129 | Observing satellite and instruments |  |  |
|  |  |  | 1 04 000 | Delayed replication of 4 descriptors |  |  |
|  |  |  | 0 31 002 | Extended delayed descriptor replication factor |  |  |
|  |  |  | 0 05 074 | Index in range direction |  |  |
|  |  |  | 2 02 133 | Change scale |  |  |
|  |  |  | 0 06 029 | Wave number |  |  |
|  |  |  | 2 02 000 | Cancel change scale |  |  |
|  |  |  | 1 02 024 | Replicate 2 descriptors 24 times |  |  |
|  |  |  | 0 05 075 | Index in azimuthal direction |  |  |
|  |  |  | 0 25 082 | Azimuth angle |  |  |
|  |  |  | 0 05 092 | Side track index or along lateral track row number |  |  |
|  |  |  | 0 01 124 | Grid point identifier |  |  |
|  |  |  | 0 05 001 | Latitude (high accuracy) |  |  |
|  |  |  | 0 06 001 | Longitude (high accuracy) |  |  |
|  |  |  | 3 01 130 | High precision timestamp |  |  |
|  |  |  | 1 10 000 | Delayed replication of 10 descriptors |  |  |
|  |  |  | 0 31 002 | Extended delayed descriptor replication factor |  |  |
|  |  |  | 0 05 074 | Index in range direction |  |  |
|  |  |  | 1 08 024 | Replicate 8 descriptors 24 times |  |  |
|  |  |  | 0 05 075 | Index in azimuthal direction |  |  |
|  |  |  | 0 08 085 | Beam identifier |  |  |
|  |  |  | 0 22 161 | Wave spectra |  |  |
|  |  |  | 0 08 085 | Beam identifier |  |  |
|  |  |  | 1 02 000 | Delayed replication of 2 descriptors |  |  |
|  |  |  | 0 31 002 | Extended delayed descriptor replication factor |  |  |
|  |  |  | 0 02 134 | Antenna beam azimuth |  |  |
|  |  |  | 0 22 161 | Wave spectra |  |  |
| 3 02 066 | (Dangerous weather phenomena) |  | 0 20 023 | Other weather phenomena |  |  |
|  |  |  | 0 20 024 | Intensity of phenomena |  |  |
|  |  |  | 0 20 027 | Phenomena occurrence |  |  |
|  |  |  | 0 20 054 | True direction from which a phenomenon or clouds are moving or in which they are observed |  |  |
|  |  |  | 0 20 023 | Other weather phenomena |  |  |
|  |  |  | 0 20 027 | Phenomena occurrence |  |  |
|  |  |  | 0 20 054 | True direction from which a phenomenon or clouds are moving or in which they are observed |  |  |
|  |  |  | 0 20 025 | Obscuration |  |  |
|  |  |  | 0 20 026 | Character of obscuration |  |  |
|  |  |  | 0 20 027 | Phenomena occurrence |  |  |
|  |  |  | 0 20 040 | Evolution of drift snow |  |  |
|  |  |  | 0 20 066 | Maximum diameter of hailstones |  |  |
|  |  |  | 0 20 027 | Phenomena occurrence |  |  |
|  |  |  | 0 20 021 | Type of precipitation |  |  |
|  |  |  | 0 20 067 | Diameter of deposit |  |  |
|  |  |  | 0 20 027 | Phenomena occurrence |  |  |
| 3 02 067 | (Additional synoptical parameters) |  | 0 01 023 | Observation sequence number |  |  |
|  |  | Additional “instantaneous” data | 0 04 025 | Time period or displacement | = 0 minutes |  |
|  |  | Additional “instantaneous” data | 0 02 177 | Method of snow depth measurement | = 0 Manual observation, = 1 Ultrasonic method, = 2 Video camera method, = 3 Laser method, 4-13 Reserved, = 14 Others, = 15 Missing value |  |
|  |  | Additional present weather | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  | Additional present weather | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  | Additional present weather | 0 20 003 | Present weather | 960ww, 961ww |  |
|  |  | Visibility in different directions | 1 03 000 | Delayed replication of 3 descriptors |  |  |
|  |  | Visibility in different directions | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  | Visibility in different directions | 0 05 021 | Bearing or azimuth | 981VV-988VV |  |
|  |  | Visibility in different directions | 0 20 001 | Horizontal visibility | VV |  |
|  |  | Visibility in different directions | 0 05 021 | Bearing or azimuth | Set to missing (cancel) |  |
|  |  | Sea data (observations from a coastal station) | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  | Sea data (observations from a coastal station) | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Sea data (observations from a coastal station) | 3 02 056 | Sea/water temperature | Sea-surface temperature, method of measurement, and depth below sea surface |  |
|  |  | Sea data (observations from a coastal station) | 1 03 000 | Delayed replication of 3 descriptors |  |  |
|  |  | Sea data (observations from a coastal station) | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Sea data (observations from a coastal station) | 0 33 041 | Attribute of following value |  |  |
|  |  | Sea data (observations from a coastal station) | 0 20 058 | Visibility seawards from a coastal station | 980VsVs |  |
|  |  | Sea data (observations from a coastal station) | 0 22 061 | State of the sea | 924SVs |  |
|  |  | Sea data (observations from a coastal station) | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  | Sea data (observations from a coastal station) | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Sea data (observations from a coastal station) | 3 02 022 | Wind waves |  |  |
|  |  | Sea data (observations from a coastal station) | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  | Sea data (observations from a coastal station) | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  | Sea data (observations from a coastal station) | 3 02 023 | Swell waves |  |  |
|  |  | Clouds | 1 04 000 | Delayed replication of 4 descriptors |  |  |
|  |  | Clouds | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  | Clouds | 0 20 054 | True direction from which a phenomenon or clouds are moving or in which they are observed | Da, Dp |  |
|  |  | Clouds | 0 20 137 | Evolution of clouds | 940Cn3 |  |
|  |  | Clouds | 0 20 012 | Cloud type | 941CDp, 943CLDp |  |
|  |  | Clouds | 0 20 090 | Special clouds | 993CsDa |  |
|  |  | Clouds | 1 03 000 | Delayed replication of 3 descriptors |  |  |
|  |  | Clouds | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  | Clouds | 0 20 054 | True direction from which a phenomenon or clouds are moving or in which they are observed |  |  |
|  |  | Clouds | 0 20 137 | Evolution of clouds |  |  |
|  |  | Clouds | 0 20 136 | Supplementary cloud type | 948C0Da, 949CaDa, 950Nmn3, 951Nvn4 |  |
|  |  | Additional "period" data | 0 04 025 | Time period or displacement | Reference period of fresh fallen snow |  |
|  |  | Additional "period" data | 0 13 012 | Depth of fresh snow |  |  |
|  |  | Additional wind data | 0 04 025 | Time period or displacement | = -60 minutes |  |
|  |  | Additional wind data | 0 11 042 | Maximum wind speed (10-minute mean wind) | 912ff 902tt 912ff .. mandatory ff >= 18 |  |
|  |  | Additional wind data | 1 04 000 | Delayed replication of 4 descriptors |  |  |
|  |  | Additional wind data | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  | Additional wind data | 0 08 021 | Time significance | = 30 Time of occurrence, = 17 Start of phenomenon |  |
|  |  | Additional wind data | 0 04 025 | Time period or displacement | = -xx ¦ 902tt |  |
|  |  | Additional wind data | 0 11 042 | Maximum wind speed (10-minute mean wind) | 912ff |  |
|  |  | Additional wind data | 0 08 021 | Time significance | Set to missing (cancel) |  |
|  |  | Significant change in wind speed and/or direction | 1 15 000 | Delayed replication of 15 descriptors |  |  |
|  |  | Significant change in wind speed and/or direction | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  | Significant change in wind speed and/or direction | 0 08 021 | Time significance | = 30 Time of occurrence, = 17 Start of phenomenon |  |
|  |  | Significant change in wind speed and/or direction | 0 04 015 | Time increment | = -xx1 |  |
|  |  | Significant change in wind speed and/or direction | 0 08 021 | Time significance | = 2 Time averaged |  |
|  |  | Significant change in wind speed and/or direction | 0 04 025 | Time period or displacement | = -10 minutes, or number of minutes after a significant change of wind |  |
|  |  | Significant change in wind speed and/or direction | 0 11 001 | Wind direction | 915dd |  |
|  |  | Significant change in wind speed and/or direction | 0 11 002 | Wind speed | 913ff |  |
|  |  | Significant change in wind speed and/or direction | 0 08 021 | Time significance | = 22 Time of occurrence of wind shift |  |
|  |  | Significant change in wind speed and/or direction | 0 04 015 | Time increment | = +xx2 |  |
|  |  | Significant change in wind speed and/or direction | 0 08 021 | Time significance | = 2 Time averaged |  |
|  |  | Significant change in wind speed and/or direction | 0 04 025 | Time period or displacement | = -10 minutes, or number of minutes after a significant change of wind |  |
|  |  | Significant change in wind speed and/or direction | 0 11 001 | Wind direction | 915dd |  |
|  |  | Significant change in wind speed and/or direction | 0 11 002 | Wind speed | 913ff |  |
|  |  | Significant change in wind speed and/or direction | 0 08 021 | Time significance | Set to missing (cancel) |  |
|  |  | Significant change in wind speed and/or direction | 0 04 025 | Time period or displacement | = 0 minutes |  |
|  |  | Significant change in wind speed and/or direction | 0 04 015 | Time increment | = +(xx1-xx2) ¦ Non negative to reset the time to the actual time |  |
|  |  | Additional weather | 1 03 000 | Delayed replication of 3 descriptors |  |  |
|  |  | Additional weather | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  | Additional weather | 0 04 025 | Time period or displacement | = -xx, i.e. from |  |
|  |  | Additional weather | 0 04 025 | Time period or displacement | = -xx, i.e. to |  |
|  |  | Additional weather | 0 20 003 | Present weather | 962ww, 963w1w1, 964ww, 965w1w1, 966ww, 967w1w1 |  |
|  |  | Additional 9SPSPspsp groups | 1 10 000 | Delayed replication of 10 descriptors |  |  |
|  |  | Additional 9SPSPspsp groups | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  | Additional 9SPSPspsp groups | 0 04 025 | Time period or displacement | = -xx, i.e. from |  |
|  |  | Additional 9SPSPspsp groups | 0 04 025 | Time period or displacement | = -xx, i.e. to |  |
|  |  | Additional 9SPSPspsp groups | 0 05 021 | Bearing or azimuth | Da, Dp |  |
|  |  | Additional 9SPSPspsp groups | 0 05 021 | Bearing or azimuth | Da, Dp |  |
|  |  | Additional 9SPSPspsp groups | 0 20 054 | True direction from which a phenomenon or clouds are moving or in which they are observed | Da, Dp |  |
|  |  | Additional 9SPSPspsp groups | 0 20 024 | Intensity of phenomena | = 1 Light, = 2 Moderate, = 3 Heavy, = 4 Violent, = 5 Severe |  |
|  |  | Additional 9SPSPspsp groups | 0 20 025 | Obscuration |  |  |
|  |  | Additional 9SPSPspsp groups | 0 20 026 | Character of obscuration |  |  |
|  |  | Additional 9SPSPspsp groups | 0 20 027 | Phenomena occurrence |  |  |
|  |  | Additional 9SPSPspsp groups | 0 20 063 | Special phenomena |  |  |
| 3 02 069 | (Visibility data) |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
|  |  |  | 0 07 033 | Height of sensor above water surface |  |  |
|  |  |  | 0 33 041 | Attribute of following value |  |  |
|  |  |  | 0 20 001 | Horizontal visibility |  |  |
| 3 02 070 | (Wind data) |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
|  |  |  | 0 07 033 | Height of sensor above water surface |  |  |
|  |  |  | 0 11 001 | Wind direction |  |  |
|  |  |  | 0 11 002 | Wind speed |  |  |
|  |  |  | 0 11 043 | Maximum wind gust direction |  |  |
|  |  |  | 0 11 041 | Maximum wind gust speed |  |  |
|  |  |  | 0 11 016 | Extreme counterclockwise wind direction of a variable wind |  |  |
|  |  |  | 0 11 017 | Extreme clockwise wind direction of a variable wind |  |  |
| 3 02 071 | (Wind data from one-hour period) |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
|  |  |  | 0 07 033 | Height of sensor above water surface |  |  |
|  |  |  | 0 08 021 | Time significance | = 2 Time averaged |  |
|  |  |  | 0 04 025 | Time period or displacement | = -10 minutes, or number of minutes after a significant change of wind, if any |  |
|  |  |  | 0 11 001 | Wind direction |  |  |
|  |  |  | 0 11 002 | Wind speed |  |  |
|  |  |  | 0 08 021 | Time significance | Set to missing |  |
|  |  |  | 1 03 002 | Replicate 3 descriptors 2 times |  |  |
|  |  |  | 0 04 025 | Time period or displacement | = -10 minutes in the first replication, = -60 minutes in the second replication |  |
|  |  |  | 0 11 043 | Maximum wind gust direction |  |  |
|  |  |  | 0 11 041 | Maximum wind gust speed |  |  |
|  |  |  | 0 04 025 | Time period or displacement | = -10 minutes |  |
|  |  |  | 0 11 016 | Extreme counterclockwise wind direction of a variable wind |  |  |
|  |  |  | 0 11 017 | Extreme clockwise wind direction of a variable wind |  |  |
| 3 02 072 | (Temperature and humidity data) |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
|  |  |  | 0 07 033 | Height of sensor above water surface |  |  |
|  |  |  | 0 12 101 | Temperature/air temperature | Scale: 2 |  |
|  |  |  | 0 12 103 | Dewpoint temperature | Scale: 2 |  |
|  |  |  | 0 13 003 | Relative humidity |  |  |
| 3 02 073 | (Cloud data) |  | 0 20 010 | Cloud cover (total) |  |  |
|  |  |  | 1 05 004 | Replicate 5 descriptors 4 times |  |  |
|  |  |  | 0 08 002 | Vertical significance (surface observations) |  |  |
|  |  |  | 0 20 011 | Cloud amount |  |  |
|  |  |  | 0 20 012 | Cloud type |  |  |
|  |  |  | 0 33 041 | Attribute of following value |  |  |
|  |  |  | 0 20 013 | Height of base of cloud |  |  |
| 3 02 074 | (Present and past weather) |  | 0 20 003 | Present weather |  |  |
|  |  |  | 0 04 025 | Time period or displacement |  |  |
|  |  |  | 0 20 004 | Past weather (1) |  |  |
|  |  |  | 0 20 005 | Past weather (2) |  |  |
| 3 02 075 | (Intensity of precipitation, size of precipitation element) |  | 0 08 021 | Time significance | = 2 Time averaged |  |
|  |  |  | 0 04 025 | Time period or displacement | = -10 minutes |  |
|  |  |  | 0 13 055 | Intensity of precipitation |  |  |
|  |  |  | 0 13 058 | Size of precipitating element |  |  |
|  |  |  | 0 08 021 | Time significance | Set to missing |  |
| 3 02 076 | (Precipitation, obscuration and other phenomena) |  | 0 20 021 | Type of precipitation |  |  |
|  |  |  | 0 20 022 | Character of precipitation |  |  |
|  |  |  | 0 26 020 | Duration of precipitation |  |  |
|  |  |  | 0 20 023 | Other weather phenomena |  |  |
|  |  |  | 0 20 024 | Intensity of phenomena |  |  |
|  |  |  | 0 20 025 | Obscuration |  |  |
|  |  |  | 0 20 026 | Character of obscuration |  |  |
| 3 02 077 | (Extreme temperature data) |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
|  |  |  | 0 07 033 | Height of sensor above water surface |  |  |
|  |  |  | 0 04 025 | Time period or displacement |  |  |
|  |  |  | 0 12 111 | Maximum temperature, at height and over period specified | Scale: 2 |  |
|  |  |  | 0 12 112 | Minimum temperature, at height and over period specified | Scale: 2 |  |
|  |  |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Ground temperature |  |
|  |  |  | 0 04 025 | Time period or displacement |  |  |
|  |  |  | 0 12 112 | Minimum temperature, at height and over period specified | Scale: 2 ¦ Ground temperature |  |
| 3 02 078 | (State of ground and snow depth measurement) |  | 0 02 176 | Method of state of ground measurement |  |  |
|  |  |  | 0 20 062 | State of the ground (with or without snow) |  |  |
|  |  |  | 0 02 177 | Method of snow depth measurement |  |  |
|  |  |  | 0 13 013 | Total snow depth |  |  |
| 3 02 079 | (Precipitation measurement) |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
|  |  |  | 0 02 175 | Method of precipitation measurement |  |  |
|  |  |  | 0 02 178 | Method of liquid content measurement of precipitation |  |  |
|  |  |  | 0 04 025 | Time period or displacement |  |  |
|  |  |  | 0 13 011 | Total precipitation/total water equivalent |  |  |
| 3 02 080 | (Evaporation measurement) |  | 0 02 185 | Method of evaporation measurement |  |  |
|  |  |  | 0 04 025 | Time period or displacement |  |  |
|  |  |  | 0 13 033 | Evaporation/evapotranspiration |  |  |
| 3 02 081 | (Total sunshine data) |  | 0 04 025 | Time period or displacement |  |  |
|  |  |  | 0 14 031 | Total sunshine |  |  |
| 3 02 082 | (Radiation data) |  | 0 04 025 | Time period or displacement |  |  |
|  |  |  | 0 14 002 | Long-wave radiation, integrated over period specified |  |  |
|  |  |  | 0 14 004 | Short-wave radiation, integrated over period specified |  |  |
|  |  |  | 0 14 016 | Net radiation, integrated over period specified |  |  |
|  |  |  | 0 14 028 | Global solar radiation (high accuracy), integrated over period specified |  |  |
|  |  |  | 0 14 029 | Diffuse solar radiation (high accuracy), integrated over period specified |  |  |
|  |  |  | 0 14 030 | Direct solar radiation (high accuracy), integrated over period specified |  |  |
| 3 02 083 | (First-order statistics of P, W, T, U data) |  | 0 04 025 | Time period or displacement |  |  |
|  |  |  | 0 08 023 | First-order statistics |  |  |
|  |  |  | 0 10 004 | Pressure |  |  |
|  |  |  | 0 11 001 | Wind direction |  |  |
|  |  |  | 0 11 002 | Wind speed |  |  |
|  |  |  | 0 12 101 | Temperature/air temperature | Scale: 2 |  |
|  |  |  | 0 13 003 | Relative humidity |  |  |
|  |  |  | 0 08 023 | First-order statistics | Set to missing |  |
| 3 02 084 | ("Instantaneous" data of sequence 3 07 096) |  | 3 02 031 | Pressure information |  |  |
|  |  |  | 3 02 072 | Temperature and humidity data |  |  |
|  |  |  | 1 03 000 | Delayed replication of 3 descriptors |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 1 01 005 | Replicate 1 descriptor 5 times |  |  |
|  |  |  | 3 07 063 | Depth below land surface and soil temperature |  |  |
|  |  |  | 0 07 061 | Depth below land surface | Set to missing (cancel) |  |
|  |  | Visibility data | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  | Visibility data | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Visibility data | 3 02 069 | Visibility data |  |  |
|  |  | Visibility data | 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
|  |  | Visibility data | 0 07 033 | Height of sensor above water surface | Set to missing (cancel) |  |
|  |  | Marine data | 1 05 000 | Delayed replication of 5 descriptors |  |  |
|  |  | Marine data | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Marine data | 0 20 031 | Ice deposit (thickness) |  |  |
|  |  | Marine data | 0 20 032 | Rate of ice accretion (estimated) |  |  |
|  |  | Marine data | 0 02 038 | Method of water temperature and/or salinity measurement |  |  |
|  |  | Marine data | 0 22 043 | Sea/water temperature | Scale: 2 |  |
|  |  | Marine data | 3 02 021 | Waves |  |  |
|  |  | State of ground and snow depth measurement | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  | State of ground and snow depth measurement | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | State of ground and snow depth measurement | 3 02 078 | State of ground and snow depth measurement |  |  |
|  |  | State of ground and snow depth measurement | 0 12 113 | Ground minimum temperature, past 12 hours | Scale: 2 |  |
|  |  | Cloud data | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  | Cloud data | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Cloud data | 3 02 004 | General cloud information |  |  |
|  |  | Cloud data | 1 05 000 | Delayed replication of 5 descriptors |  |  |
|  |  | Cloud data | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  | Cloud data | 0 08 002 | Vertical significance (surface observations) |  |  |
|  |  | Cloud data | 0 20 011 | Cloud amount |  |  |
|  |  | Cloud data | 0 20 012 | Cloud type |  |  |
|  |  | Cloud data | 0 33 041 | Attribute of following value |  |  |
|  |  | Cloud data | 0 20 013 | Height of base of cloud |  |  |
|  |  | Cloud data | 3 02 036 | Clouds with bases below station level |  |  |
|  |  | Direction of cloud drift 6DLDMDH | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  | Direction of cloud drift 6DLDMDH | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Direction of cloud drift 6DLDMDH | 3 02 047 | Direction of cloud drift |  |  |
|  |  | Direction of cloud drift 6DLDMDH | 0 08 002 | Vertical significance (surface observations) | Set to missing (cancel) |  |
|  |  | Direction and elevation of cloud 57CDaec | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  | Direction and elevation of cloud 57CDaec | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Direction and elevation of cloud 57CDaec | 3 02 048 | Direction and elevation of cloud |  |  |
| 3 02 085 | ("Period" data of sequence 3 07 096) | Present and past weather data | 1 05 000 | Delayed replication of 5 descriptors |  |  |
|  |  | Present and past weather data | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Present and past weather data | 0 20 003 | Present weather |  |  |
|  |  | Present and past weather data | 1 03 002 | Replicate 3 descriptors 2 times |  |  |
|  |  | Present and past weather data | 0 04 024 | Time period or displacement | = -1 hour in the first replication, = -x hours in the second replication, x corresponding to the time period of W1W2 in the SYNOP report |  |
|  |  | Present and past weather data | 0 20 004 | Past weather (1) |  |  |
|  |  | Present and past weather data | 0 20 005 | Past weather (2) |  |  |
|  |  | Intensity of precipitation, size of precipitation element | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  | Intensity of precipitation, size of precipitation element | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Intensity of precipitation, size of precipitation element | 3 02 175 | Intensity of precipitation, size of precipitation element |  |  |
|  |  | Precipitation, obscuration and other phenomena | 1 02 000 | Delayed replication of 2 descriptors |  |  |
|  |  | Precipitation, obscuration and other phenomena | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Precipitation, obscuration and other phenomena | 0 04 025 | Time period or displacement | = -10 minutes |  |
|  |  | Precipitation, obscuration and other phenomena | 3 02 076 | Precipitation, obscuration and other phenomena |  |  |
|  |  | Lightning data | 1 02 000 | Delayed replication of 2 descriptors |  |  |
|  |  | Lightning data | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Lightning data | 0 04 025 | Time period or displacement | = -10 minutes |  |
|  |  | Lightning data | 0 13 059 | Number of flashes (thunderstorm) |  |  |
|  |  | Wind data | 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
|  |  | Wind data | 0 07 033 | Height of sensor above water surface |  |  |
|  |  | Wind data | 0 08 021 | Time significance | = 2 Time averaged |  |
|  |  | Wind data | 0 04 025 | Time period or displacement | = -10 minutes, or number of minutes after a significant change of wind |  |
|  |  | Wind data | 0 11 001 | Wind direction |  |  |
|  |  | Wind data | 0 11 002 | Wind speed |  |  |
|  |  | Wind data | 0 08 021 | Time significance | Set to missing |  |
|  |  | Wind data | 1 03 003 | Replicate 3 descriptors 3 times |  |  |
|  |  | Wind data | 0 04 025 | Time period or displacement | = -10 minutes in the first replication, = -60 minutes in the second replication, = -60x3 or 60x6 minutes in the third replication |  |
|  |  | Wind data | 0 11 043 | Maximum wind gust direction |  |  |
|  |  | Wind data | 0 11 041 | Maximum wind gust speed |  |  |
|  |  | Wind data | 0 04 025 | Time period or displacement | = -10 minutes |  |
|  |  | Wind data | 0 11 016 | Extreme counterclockwise wind direction of a variable wind |  |  |
|  |  | Wind data | 0 11 017 | Extreme clockwise wind direction of a variable wind |  |  |
|  |  | Extreme temperature data | 3 02 077 | Extreme temperature data |  |  |
|  |  | Extreme temperature data | 0 07 033 | Height of sensor above water surface | Set to missing (cancel) |  |
|  |  | Extreme temperature data | 3 02 041 | Extreme temperature data |  |  |
|  |  | Precipitation measurement | 1 06 000 | Delayed replication of 6 descriptors |  |  |
|  |  | Precipitation measurement | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Precipitation measurement | 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
|  |  | Precipitation measurement | 0 02 175 | Method of precipitation measurement |  |  |
|  |  | Precipitation measurement | 0 02 178 | Method of liquid content measurement of precipitation |  |  |
|  |  | Precipitation measurement | 1 02 005 | Replicate 2 descriptors 5 times |  |  |
|  |  | Precipitation measurement | 0 04 024 | Time period or displacement | = -1 hour in the first replication, = -3, -6, -12 and -24 hours in the other replications |  |
|  |  | Precipitation measurement | 0 13 011 | Total precipitation/total water equivalent |  |  |
|  |  | Precipitation measurement | 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
|  |  | Evaporation data | 1 03 000 | Delayed replication of 3 descriptors |  |  |
|  |  | Evaporation data | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Evaporation data | 0 02 185 | Method of evaporation measurement |  |  |
|  |  | Evaporation data | 1 01 002 | Replicate 1 descriptor 2 times |  |  |
|  |  | Evaporation data | 3 02 044 | Evaporation data |  |  |
|  |  | Total sunshine data | 1 02 000 | Delayed replication of 2 descriptors |  |  |
|  |  | Total sunshine data | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Total sunshine data | 1 01 002 | Replicate 1 descriptor 2 times |  |  |
|  |  | Total sunshine data | 3 02 039 | Sunshine data (from 1 hour and 24 hour period) |  |  |
|  |  | Radiation data | 1 02 000 | Delayed replication of 2 descriptors |  |  |
|  |  | Radiation data | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Radiation data | 1 01 002 | Replicate 1 descriptor 2 times |  |  |
|  |  | Radiation data | 3 02 045 | Radiation data (from 1 hour and 24 hour period) |  |  |
|  |  | Temperature change group 54g0sndT | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  | Temperature change group 54g0sndT | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Temperature change group 54g0sndT | 3 02 046 | Temperature change |  |  |
|  |  | First-order statistics of P, W, T, U data | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  | First-order statistics of P, W, T, U data | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | First-order statistics of P, W, T, U data | 3 02 083 | First-order statistics of P, W, T, U data |  |  |
| 3 02 089 | (Locust information) |  | 0 20 101 | Locust (acridian) name | Ln |  |
|  |  |  | 0 20 102 | Locust (maturity) colour | Lc |  |
|  |  |  | 0 20 103 | Stage of development of locusts | Ld |  |
|  |  |  | 0 20 104 | Organization state of swarm or band of locusts | Lg |  |
|  |  |  | 0 20 105 | Size of swarm or band of locusts and duration of passage of swarm | sL |  |
|  |  |  | 0 20 106 | Locust population density | dL |  |
|  |  |  | 0 20 107 | Direction of movements of locust swarm | DL |  |
|  |  |  | 0 20 108 | Extent of vegetation | ve |  |
| 3 02 090 | (Sea/water temperature high precision) |  | 0 02 038 | Method of water temperature and/or salinity measurement |  |  |
|  |  |  | 0 07 063 | Depth below sea/water surface (cm) | Sea-surface temperature measurement |  |
|  |  |  | 0 22 045 | Sea/water temperature |  |  |
| 3 02 091 | (Sequence for representation of ancillary meteorological observations) |  | 0 20 001 | Horizontal visibility |  |  |
|  |  |  | 0 04 024 | Time period or displacement |  |  |
|  |  |  | 0 13 011 | Total precipitation/total water equivalent |  |  |
| 3 02 092 | (VOSClim data elements) |  | 0 11 104 | True heading of aircraft, ship or other mobile platform | Ship’s true heading |  |
|  |  |  | 0 01 012 | Direction of motion of moving observing platform | Ship’s course over ground |  |
|  |  |  | 0 01 013 | Speed of motion of moving observing platform | Ship’s speed over ground |  |
|  |  |  | 0 10 038 | Maximum height of deck cargo above summer load line |  |  |
|  |  |  | 0 10 039 | Departure of reference level (summer maximum load line) from actual sea level |  |  |
|  |  |  | 0 11 007 | Relative wind direction (in degrees off bow) |  |  |
|  |  |  | 0 11 008 | Relative wind speed |  |  |
| 3 02 093 | (Extended ship temperature and humidity data) |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
|  |  |  | 0 07 033 | Height of sensor above water surface |  |  |
|  |  |  | 3 03 099 | Metadata common to temperature/humidity sensors |  |  |
|  |  |  | 0 12 101 | Temperature/air temperature |  |  |
|  |  |  | 1 03 000 | Delayed replication of 3 descriptors |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor | Set to 0 if no change from previous values, 1 if changes |  |
|  |  |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
|  |  |  | 0 07 033 | Height of sensor above water surface |  |  |
|  |  |  | 3 03 099 | Metadata common to temperature/humidity sensors |  |  |
|  |  |  | 0 02 039 | Method of wet-bulb temperature measurement |  |  |
|  |  |  | 0 02 097 | Type of humidity sensor |  |  |
|  |  |  | 0 03 024 | Psychrometric coefficient | Set to missing if type of humidity sensor is not psychrometer |  |
|  |  |  | 0 03 021 | Hygrometer heating |  |  |
|  |  |  | 0 12 102 | Wet-bulb temperature |  |  |
|  |  |  | 0 12 103 | Dewpoint temperature |  |  |
|  |  |  | 0 13 003 | Relative humidity |  |  |
|  |  |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
|  |  |  | 0 07 033 | Height of sensor above water surface | Set to missing (cancel) |  |
|  |  |  | 3 03 099 | Metadata common to temperature/humidity sensors | Set all elements in sequence to missing (cancel) |  |
|  |  |  | 0 02 039 | Method of wet-bulb temperature measurement | Set to missing (cancel) |  |
|  |  |  | 0 02 097 | Type of humidity sensor | Set to missing (cancel) |  |
|  |  |  | 0 03 021 | Hygrometer heating | Set to missing (cancel) |  |
|  |  |  | 0 03 024 | Psychrometric coefficient | Set to missing (cancel) |  |
| 3 02 175 | (Intensity of precipitation, size of precipitation element) |  | 0 08 021 | Time significance |  |  |
|  |  |  | 0 04 025 | Time period or displacement |  |  |
|  |  |  | 0 13 155 | Intensity of precipitation (high accuracy) |  |  |
|  |  |  | 0 13 058 | Size of precipitating element |  |  |
|  |  |  | 0 08 021 | Time significance |  |  |