Category 15: Oceanographic report sequences

| FXY1 | TITLE | SUBTITLE | FXY2 | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| 3 15 001 | (Typically reported underwater sounding without optional fields) |  | 0 01 011 | Ship or mobile land station identifier | Ship's call sign |  |
|  |  |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 3 01 012 | Hour, minute |  |  |
|  |  |  | 3 01 023 | Latitude/longitude (coarse accuracy) |  |  |
|  |  |  | 3 06 001 | Depth, temperature |  |  |
| 3 15 002 | (Typically reported underwater sounding without optional fields) |  | 0 01 011 | Ship or mobile land station identifier | Ship's call sign |  |
|  |  |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 3 01 012 | Hour, minute |  |  |
|  |  |  | 3 01 023 | Latitude/longitude (coarse accuracy) |  |  |
|  |  |  | 3 06 004 | Depth, temperature, salinity |  |  |
| 3 15 003 | (Temperature and salinity profile observed by profile floats) |  | 0 01 087 | WMO marine observing platform extended identifier |  |  |
|  |  |  | 0 01 085 | Observing platform manufacturer's model |  |  |
|  |  |  | 0 01 086 | Observing platform manufacturer's serial number |  |  |
|  |  |  | 0 02 036 | Buoy type |  |  |
|  |  |  | 0 02 148 | Data collection and/or location system |  |  |
|  |  |  | 0 02 149 | Type of data buoy |  |  |
|  |  |  | 0 22 055 | Float cycle number |  |  |
|  |  |  | 0 22 056 | Direction of profile |  |  |
|  |  |  | 0 22 067 | Instrument type for water temperature/salinity profile measurement |  |  |
|  |  |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 3 01 012 | Hour, minute |  |  |
|  |  |  | 3 01 021 | Latitude/longitude (high accuracy) |  |  |
|  |  |  | 0 08 080 | Qualifier for GTSPP quality flag |  |  |
|  |  |  | 0 33 050 | Global GTSPP quality flag |  |  |
|  |  |  | 1 09 000 | Delayed replication of 9 descriptors |  |  |
|  |  |  | 0 31 002 | Extended delayed descriptor replication factor |  |  |
|  |  |  | 0 07 065 | Water pressure |  |  |
|  |  |  | 0 08 080 | Qualifier for GTSPP quality flag |  |  |
|  |  |  | 0 33 050 | Global GTSPP quality flag |  |  |
|  |  |  | 0 22 045 | Sea/water temperature |  |  |
|  |  |  | 0 08 080 | Qualifier for GTSPP quality flag |  |  |
|  |  |  | 0 33 050 | Global GTSPP quality flag |  |  |
|  |  |  | 0 22 064 | Salinity |  |  |
|  |  |  | 0 08 080 | Qualifier for GTSPP quality flag |  |  |
|  |  |  | 0 33 050 | Global GTSPP quality flag |  |  |
| 3 15 004 | (XBT temperature profile data sequence) |  | 0 01 079 | Unique identifier for the profile | Hexadecimal string |  |
|  |  |  | 0 01 011 | Ship or mobile land station identifier | Ship's call sign |  |
|  |  |  | 0 01 103 | IMO Number. Unique Lloyd's register | = 0 to 9999999 |  |
|  |  |  | 0 01 087 | WMO marine observing platform extended identifier |  | Note 24: If field 0 01 011 is used, this field will be left missing and vice versa. |
|  |  |  | 0 01 019 | Long station or site name | Ship name |  |
|  |  |  | 0 01 080 | Ship line number according to SOOP |  |  |
|  |  |  | 0 05 036 | Ship transect number according to SOOP |  | Note 31: Integer, assigned by the operator, incremented for each new transect (i.e. all drops have the same transect number while the ship is moving from one end point of the line to the other end point; as soon as the ship arrived to port and goes back to start a new transect then transect number is incremented). The initial value and subsequent values for transect numbers do not matter provided that each new transect by a ship on a line has a transect number higher than previous transect numbers for the same line and the same ship. In case a single cruise follows more than one SOOP line in a row, then the transect number should be incremented each time the cruise changes line. |
|  |  |  | 0 01 036 | Agency in charge of operating the observing platform |  |  |
|  |  |  | 0 01 013 | Speed of motion of moving observing platform |  |  |
|  |  |  | 0 01 012 | Direction of motion of moving observing platform |  |  |
|  |  |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 3 01 012 | Hour, minute |  |  |
|  |  |  | 3 01 021 | Latitude/longitude (high accuracy) |  |  |
|  |  |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
|  |  |  | 0 07 033 | Height of sensor above water surface |  |  |
|  |  |  | 0 02 002 | Type of instrumentation for wind measurement |  |  |
|  |  |  | 0 11 002 | Wind speed |  |  |
|  |  |  | 0 11 001 | Wind direction |  |  |
|  |  |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
|  |  |  | 0 07 033 | Height of sensor above water surface |  |  |
|  |  |  | 0 12 101 | Temperature/air temperature |  |  |
|  |  |  | 0 12 103 | Dewpoint temperature |  |  |
|  |  |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
|  |  |  | 0 07 033 | Height of sensor above water surface | Set to missing (cancel) |  |
|  |  |  | 3 02 021 | Waves |  |  |
|  |  |  | 0 02 031 | Duration and time of current measurement |  |  |
|  |  |  | 0 02 030 | Method of current measurement |  |  |
|  |  |  | 0 22 005 | Direction of sea-surface current |  |  |
|  |  |  | 0 22 032 | Speed of sea-surface current |  |  |
|  |  |  | 0 22 063 | Total water depth |  |  |
|  |  |  | 0 08 080 | Qualifier for GTSPP quality flag |  |  |
|  |  |  | 0 33 050 | Global GTSPP quality flag |  |  |
|  |  |  | 0 22 178 | XBT/XCTD launcher type |  |  |
|  |  |  | 0 22 177 | Height of XBT/XCTD launcher | Above sea level ¦ 0 to 50 m in units of whole m |  |
|  |  |  | 0 22 067 | Instrument type for water temperature/salinity profile measurement |  |  |
|  |  |  | 0 08 041 | Data significance |  |  |
|  |  |  | 0 26 021 | Year |  |  |
|  |  |  | 0 26 022 | Month |  |  |
|  |  |  | 0 26 023 | Day |  |  |
|  |  |  | 0 22 068 | Water temperature profile recorder types |  |  |
|  |  |  | 0 25 061 | Software identification and version number |  |  |
|  |  |  | 0 08 041 | Data significance | Set to missing (cancel) |  |
|  |  |  | 0 08 080 | Qualifier for GTSPP quality flag | Set to missing (cancel) |  |
|  |  |  | 0 02 171 | Instrument serial number for water temperature profile measurement |  |  |
|  |  |  | 3 02 090 | Sea/water temperature high precision |  |  |
|  |  |  | 0 02 171 | Instrument serial number for water temperature profile measurement |  |  |
|  |  |  | 0 02 032 | Indicator for digitization |  | Note 64: This descriptor applies to the method used to select depths for the temperature profile encoded through 3 15 005. If temperatures are reported at significant depths, the values shall: (a) Be sufficient to reproduce basic features of the profile; (b) Define the top and the bottom of isothermal layers. |
|  |  |  | 3 15 005 | Water temperature profile (temperature profile observed by XBT or buoy) |  |  |
| 3 15 005 | (Water temperature profile (temperature profile observed by XBT or buoy) |  | 1 06 000 | Delayed replication of 6 descriptors |  |  |
|  |  |  | 0 31 002 | Extended delayed descriptor replication factor |  |  |
|  |  |  | 0 07 063 | Depth below sea/water surface (cm) |  |  |
|  |  |  | 0 08 080 | Qualifier for GTSPP quality flag | = 13 Water depth at a level |  |
|  |  |  | 0 33 050 | Global GTSPP quality flag |  |  |
|  |  |  | 0 22 043 | Sea/water temperature |  |  |
|  |  |  | 0 08 080 | Qualifier for GTSPP quality flag | = 11 Water temperature at a level |  |
|  |  |  | 0 33 050 | Global GTSPP quality flag |  |  |
| 3 15 006 | (XCTD temperature and salinity profile data) |  | 0 01 079 | Unique identifier for the profile | Hexadecimal string |  |
|  |  |  | 0 01 011 | Ship or mobile land station identifier | Ship's call sign |  |
|  |  |  | 0 01 103 | IMO Number. Unique Lloyd's register | = 0 to 9999999 |  |
|  |  |  | 0 01 087 | WMO marine observing platform extended identifier |  | Note 24: If field 0 01 011 is used, this field will be left missing and vice versa. |
|  |  |  | 0 01 019 | Long station or site name | Ship name |  |
|  |  |  | 0 01 080 | Ship line number according to SOOP |  |  |
|  |  |  | 0 05 036 | Ship transect number according to SOOP |  | Note 31: Integer, assigned by the operator, incremented for each new transect (i.e. all drops have the same transect number while the ship is moving from one end point of the line to the other end point; as soon as the ship arrived to port and goes back to start a new transect then transect number is incremented). The initial value and subsequent values for transect numbers do not matter provided that each new transect by a ship on a line has a transect number higher than previous transect numbers for the same line and the same ship. In case a single cruise follows more than one SOOP line in a row, then the transect number should be incremented each time the cruise changes line. |
|  |  |  | 0 01 036 | Agency in charge of operating the observing platform |  |  |
|  |  |  | 0 01 013 | Speed of motion of moving observing platform |  |  |
|  |  |  | 0 01 012 | Direction of motion of moving observing platform |  |  |
|  |  |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 3 01 012 | Hour, minute |  |  |
|  |  |  | 3 01 021 | Latitude/longitude (high accuracy) |  |  |
|  |  |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
|  |  |  | 0 07 033 | Height of sensor above water surface |  |  |
|  |  |  | 0 02 002 | Type of instrumentation for wind measurement |  |  |
|  |  |  | 0 11 002 | Wind speed |  |  |
|  |  |  | 0 11 001 | Wind direction |  |  |
|  |  |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
|  |  |  | 0 07 033 | Height of sensor above water surface |  |  |
|  |  |  | 0 12 101 | Temperature/air temperature |  |  |
|  |  |  | 0 12 103 | Dewpoint temperature |  |  |
|  |  |  | 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
|  |  |  | 0 07 033 | Height of sensor above water surface | Set to missing (cancel) |  |
|  |  |  | 3 02 021 | Waves |  |  |
|  |  |  | 0 02 031 | Duration and time of current measurement |  |  |
|  |  |  | 0 02 030 | Method of current measurement |  |  |
|  |  |  | 0 22 005 | Direction of sea-surface current |  |  |
|  |  |  | 0 22 032 | Speed of sea-surface current |  |  |
|  |  |  | 0 22 063 | Total water depth |  |  |
|  |  |  | 0 08 080 | Qualifier for GTSPP quality flag |  |  |
|  |  |  | 0 33 050 | Global GTSPP quality flag |  |  |
|  |  |  | 0 22 178 | XBT/XCTD launcher type |  |  |
|  |  |  | 0 22 177 | Height of XBT/XCTD launcher | Above sea level ¦ 0 to 50 m in units of whole m |  |
|  |  |  | 0 22 067 | Instrument type for water temperature/salinity profile measurement |  |  |
|  |  |  | 0 08 041 | Data significance |  |  |
|  |  |  | 0 26 021 | Year |  |  |
|  |  |  | 0 26 022 | Month |  |  |
|  |  |  | 0 26 023 | Day |  |  |
|  |  |  | 0 22 068 | Water temperature profile recorder types |  |  |
|  |  |  | 0 25 061 | Software identification and version number |  |  |
|  |  |  | 0 08 041 | Data significance | Set to missing (cancel) |  |
|  |  |  | 0 08 080 | Qualifier for GTSPP quality flag | Set to missing (cancel) |  |
|  |  |  | 0 02 171 | Instrument serial number for water temperature profile measurement |  |  |
|  |  |  | 3 02 090 | Sea/water temperature high precision |  |  |
|  |  |  | 0 02 171 | Instrument serial number for water temperature profile measurement |  |  |
|  |  |  | 0 02 032 | Indicator for digitization |  | Note 64: This descriptor applies to the method used to select depths for the temperature profile encoded through 3 15 005. If temperatures are reported at significant depths, the values shall: (a) Be sufficient to reproduce basic features of the profile; (b) Define the top and the bottom of isothermal layers. |
|  |  |  | 1 09 000 | Delayed replication of 9 descriptors |  |  |
|  |  |  | 0 31 002 | Extended delayed descriptor replication factor |  |  |
|  |  |  | 0 07 063 | Depth below sea/water surface (cm) |  |  |
|  |  |  | 0 08 080 | Qualifier for GTSPP quality flag | = 13 Water depth at a level |  |
|  |  |  | 0 33 050 | Global GTSPP quality flag |  |  |
|  |  |  | 0 22 043 | Sea/water temperature |  |  |
|  |  |  | 0 08 080 | Qualifier for GTSPP quality flag | = 11 Water temperature at a level |  |
|  |  |  | 0 33 050 | Global GTSPP quality flag |  |  |
|  |  |  | 0 22 064 | Salinity |  |  |
|  |  |  | 0 08 080 | Qualifier for GTSPP quality flag | = 12 Salinity at a level |  |
|  |  |  | 0 33 050 | Global GTSPP quality flag |  |  |
| 3 15 007 | (Sequence for representation of data derived from a ship-based lowered instrument measuring subsurface sea/water temperature, salinity and current profiles) |  | 3 01 003 | Ship’s call sign and motion |  |  |
|  |  | Extended identification | 0 01 019 | Long station or site name |  |  |
|  |  | Extended identification | 0 01 103 | IMO Number. Unique Lloyd's register | Values are restricted to between 0 and 9999999 |  |
|  |  | Extended identification | 0 01 087 | WMO marine observing platform extended identifier | Set to missing, if ship's call sign is reported |  |
|  |  | Cruise/ship line information | 0 01 036 | Agency in charge of operating the observing platform |  |  |
|  |  | Cruise/ship line information | 0 01 115 | Identifier of the cruise or mission under which the data were collected | Set to missing, if no cruise identifier is reported |  |
|  |  | Cruise/ship line information | 0 01 080 | Ship line number according to SOOP |  |  |
|  |  | Cruise/ship line information | 0 05 036 | Ship transect number according to SOOP |  |  |
|  |  | Cruise/ship line information | 3 01 011 | Year, month, day |  |  |
|  |  | Cruise/ship line information | 3 01 012 | Hour, minute |  |  |
|  |  | Cruise/ship line information | 3 01 021 | Latitude/longitude (high accuracy) |  |  |
|  |  | Profile information | 0 01 079 | Unique identifier for the profile |  |  |
|  |  | Profile information | 0 01 023 | Observation sequence number | Cast/station number along the line/transect |  |
|  |  | Profile information | 0 22 063 | Total water depth |  |  |
|  |  | Surface pressure | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  | Surface pressure | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Surface pressure | 3 02 001 | Pressure and 3-hour pressure change |  |  |
|  |  | Waves | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  | Waves | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Waves | 3 02 021 | Waves |  |  |
|  |  | Temperature and humidity data | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  | Temperature and humidity data | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Temperature and humidity data | 3 02 052 | Ship temperature and humidity data |  |  |
|  |  | Wind data | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  | Wind data | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Wind data | 3 02 059 | Ship wind data |  |  |
|  |  | Surface temperature, salinity and current | 0 22 067 | Instrument type for water temperature/salinity profile measurement |  |  |
|  |  | Surface temperature, salinity and current | 0 02 171 | Instrument serial number for water temperature profile measurement |  |  |
|  |  | Surface temperature, salinity and current | 3 02 090 | Sea/water temperature high precision | Surface temperature |  |
|  |  | Surface temperature, salinity and current | 3 06 033 | Surface salinity |  |  |
|  |  | Surface temperature, salinity and current | 3 06 034 | Surface current |  |  |
|  |  | Surface temperature, salinity and current | 0 02 171 | Instrument serial number for water temperature profile measurement | Set to missing (cancel) |  |
|  |  | Surface temperature, salinity and current | 0 22 067 | Instrument type for water temperature/salinity profile measurement | Set to missing (cancel) |  |
|  |  | Temperature and salinity profile data | 0 02 038 | Method of water temperature and/or salinity measurement |  |  |
|  |  | Temperature and salinity profile data | 0 22 067 | Instrument type for water temperature/salinity profile measurement |  |  |
|  |  | Temperature and salinity profile data | 0 22 068 | Water temperature profile recorder types |  |  |
|  |  | Temperature and salinity profile data | 0 02 171 | Instrument serial number for water temperature profile measurement |  |  |
|  |  | Temperature and salinity profile data | 0 02 033 | Method of salinity/depth measurement |  |  |
|  |  | Temperature and salinity profile data | 0 02 032 | Indicator for digitization |  |  |
|  |  | Temperature and salinity profile data | 0 22 056 | Direction of profile |  |  |
|  |  | Temperature and salinity profile data | 0 03 011 | Method of depth calculation |  |  |
|  |  | Temperature and salinity profile data | 3 06 035 | Temperature and salinity profile |  |  |
|  |  | Current profile data | 1 07 000 | Delayed replication of 7 descriptors |  |  |
|  |  | Current profile data | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Current profile data | 0 02 032 | Indicator for digitization |  |  |
|  |  | Current profile data | 0 03 010 | Method of sea/water current measurement |  |  |
|  |  | Current profile data | 0 02 031 | Duration and time of current measurement |  |  |
|  |  | Current profile data | 0 02 040 | Method of removing velocity and motion of platform from current |  |  |
|  |  | Current profile data | 0 22 056 | Direction of profile |  |  |
|  |  | Current profile data | 0 03 011 | Method of depth calculation |  |  |
|  |  | Current profile data | 3 06 036 | Current profile |  |  |
|  |  | Dissolved oxygen profile data | 1 04 000 | Delayed replication of 4 descriptors |  |  |
|  |  | Dissolved oxygen profile data | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Dissolved oxygen profile data | 0 02 032 | Indicator for digitization |  |  |
|  |  | Dissolved oxygen profile data | 0 03 012 | Instrument type/sensor for dissolved oxygen measurement |  |  |
|  |  | Dissolved oxygen profile data | 0 03 011 | Method of depth calculation |  |  |
|  |  | Dissolved oxygen profile data | 3 06 037 | Dissolved oxygen profile data |  |  |
| 3 15 008 | (Sequence for the representation of data from moored buoys) | Buoy identification and location | 3 01 126 | Sequence for representation of moored buoy identification |  |  |
|  |  | Standard meteorological data | 3 06 038 | Sequence for representation of standard surface marine meteorological observations from moored buoys | For buoys equipped with more than 1 anemometer, the height of sensor should relate to the one being used |  |
|  |  | Optional ancillary meteorological data | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  | Optional ancillary meteorological data | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Optional ancillary meteorological data | 3 02 091 | Sequence for representation of ancillary meteorological observations |  |  |
|  |  | Optional radiation measurements | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  | Optional radiation measurements | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Optional radiation measurements | 3 02 082 | Radiation data |  |  |
|  |  | Optional basic wave measurements | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  | Optional basic wave measurements | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Optional basic wave measurements | 3 06 039 | Sequence for representation of basic wave measurements |  |  |
|  |  | Optional spectral wave measurements | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  | Optional spectral wave measurements | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Optional spectral wave measurements | 3 06 040 | Sequence for representation of detailed spectral wave measurements |  |  |
|  |  | Optional temperature profile measurements | 1 02 000 | Delayed replication of 2 descriptors |  |  |
|  |  | Optional temperature profile measurements | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Optional temperature profile measurements | 0 02 005 | Precision of temperature observation |  |  |
|  |  | Optional temperature profile measurements | 3 06 041 | Depth and temperature profile (high accuracy/precision) |  |  |
|  |  | Optional temperature and salinity profile measurements | 1 02 000 | Delayed replication of 2 descriptors |  |  |
|  |  | Optional temperature and salinity profile measurements | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Optional temperature and salinity profile measurements | 0 02 005 | Precision of temperature observation |  |  |
|  |  | Optional temperature and salinity profile measurements | 3 06 004 | Depth, temperature, salinity |  |  |
|  |  | Optional subsurface current measurements | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  | Optional subsurface current measurements | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Optional subsurface current measurements | 3 06 005 | Subsurface current measurements |  |  |
| 3 15 009 | (Sequence for the representation of data from drifting buoys) |  | 0 01 087 | WMO marine observing platform extended identifier |  |  |
|  |  |  | 0 01 019 | Long station or site name |  |  |
|  |  |  | 0 02 149 | Type of data buoy |  |  |
|  |  | Time/date of last known position | 0 08 021 | Time significance | = 26 Time of last known position |  |
|  |  | Time/date of last known position | 3 01 011 | Year, month, day | Date of last known position |  |
|  |  | Time/date of last known position | 3 01 012 | Hour, minute | Time of last known position |  |
|  |  | Location and location quality | 3 01 021 | Latitude/longitude (high accuracy) |  |  |
|  |  | Location and location quality | 2 08 016 | Change width of CCITT IA5 field | Change to 16 characters |  |
|  |  | Location and location quality | 0 01 051 | Platform Transmitter ID number |  |  |
|  |  | Location and location quality | 2 08 000 | Change width of CCITT IA5 field | Cancel |  |
|  |  | Location and location quality | 0 02 148 | Data collection and/or location system | = 1 Argos, = 8 Iridium and GPS, = 9 Argos-3 |  |
|  |  | Location and location quality | 0 01 012 | Direction of motion of moving observing platform | Platform drift direction |  |
|  |  | Location and location quality | 0 01 014 | Platform drift speed (high precision) | Speed of motion of moving observing platform |  |
|  |  | Location and location quality | 0 33 022 | Quality of buoy satellite transmission |  |  |
|  |  | Location and location quality | 0 33 023 | Quality of buoy location |  |  |
|  |  | Location and location quality | 0 33 027 | Location quality class (range of radius of 66% confidence) |  |  |
|  |  | Location and location quality | 0 25 026 | Battery voltage (large range) | Platform battery voltage |  |
|  |  | Drogue status | 0 02 034 | Drogue type |  |  |
|  |  | Drogue status | 0 22 060 | Lagrangian drifter drogue status |  |  |
|  |  | Drogue status | 0 07 070 | Drogue depth |  |  |
|  |  | Drogue status | 0 02 190 | Lagrangian drifter submergence (% time submerged) |  |  |
|  |  | Time/date of observation | 0 08 021 | Time significance | = 25 Nominal reporting time |  |
|  |  | Time/date of observation | 3 01 011 | Year, month, day | Date of observation |  |
|  |  | Time/date of observation | 3 01 012 | Hour, minute | Time of observation |  |
|  |  | Surface temperature and salinity | 0 02 005 | Precision of temperature observation |  |  |
|  |  | Surface temperature and salinity | 0 22 043 | Sea/water temperature |  |  |
|  |  | Surface temperature and salinity | 0 02 033 | Method of salinity/depth measurement |  |  |
|  |  | Surface temperature and salinity | 0 22 059 | Sea-surface salinity |  |  |
|  |  | Surface type/ice information | 0 08 029 | Surface type |  |  |
|  |  | Surface type/ice information | 0 13 115 | Ice thickness |  |  |
|  |  | Optional temperature and salinity profile measurements | 1 03 000 | Delayed replication of 3 descriptors |  |  |
|  |  | Optional temperature and salinity profile measurements | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Optional temperature and salinity profile measurements | 0 02 005 | Precision of temperature observation |  |  |
|  |  | Optional temperature and salinity profile measurements | 3 06 004 | Depth, temperature, salinity |  |  |
|  |  | Optional temperature and salinity profile measurements | 0 02 005 | Precision of temperature observation | Set to missing (cancel) |  |
|  |  | Pressure and air temperature data | 0 10 004 | Pressure |  |  |
|  |  | Pressure and air temperature data | 0 10 051 | Pressure reduced to mean sea level |  |  |
|  |  | Pressure and air temperature data | 1 02 000 | Delayed replication of 2 descriptors |  |  |
|  |  | Pressure and air temperature data | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Pressure and air temperature data | 0 07 033 | Height of sensor above water surface |  |  |
|  |  | Pressure and air temperature data | 0 12 101 | Temperature/air temperature |  |  |
|  |  | Wind data | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  | Wind data | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Wind data | 3 06 042 | Wind measurement from drifting buoy |  |  |
|  |  | Wave data | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  | Wave data | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  | Wave data | 3 06 039 | Sequence for representation of basic wave measurements |  |  |
| 3 15 010 | (First five Fourier components of the wave spectrum) |  | 1 12 000 | Delayed replication of 12 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 2 01 144 | Change data width | Add 16 bits to the width for each data element in Table B |  |
|  |  |  | 2 02 133 | Change scale | Add 5 to the scale for each data element in Table B |  |
|  |  |  | 0 22 080 | Waveband central frequency (Hz) |  |  |
|  |  |  | 0 22 096 | Spectral band width (Hz) |  |  |
|  |  |  | 0 22 069 | Spectral wave density (m2 Hz-1) |  |  |
|  |  |  | 2 02 000 | Change data width | Cancel |  |
|  |  |  | 2 01 000 | Change scale | Cancel |  |
|  |  |  | 0 42 011 | a1 coefficient of the directional Fourier series | First moment of the directional wave spectrum |  |
|  |  |  | 0 42 012 | b1 coefficient of the directional Fourier series | First moment of the directional wave spectrum |  |
|  |  |  | 0 42 013 | a2 coefficient of the directional Fourier series | Second moment of the directional wave spectrum |  |
|  |  |  | 0 42 014 | b2 coefficient of the directional Fourier series | Second moment of the directional wave spectrum |  |
|  |  |  | 0 42 015 | Check factor K | Inverse of wave ellipticity |  |
| 3 15 011 | (Met-ocean observations from autonomous surface vehicles) |  | 3 01 150 | WIGOS identifier |  |  |
|  |  |  | 0 01 087 | WMO marine observing platform extended identifier |  |  |
|  |  |  | 0 01 036 | Agency in charge of operating the observing platform |  |  |
|  |  |  | 0 01 085 | Observing platform manufacturer’s model |  |  |
|  |  |  | 0 01 086 | Observing platform manufacturer’s serial number |  |  |
|  |  |  | 0 03 001 | Surface station type |  |  |
|  |  |  | 2 08 032 | Change width of CCITT IA5 field |  |  |
|  |  |  | 0 01 079 | Unique ID for profile | UUID for report, 32-character hex string |  |
|  |  |  | 2 08 000 | Change width of CCITT IA5 field |  |  |
|  |  |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 3 01 012 | Hour, minute |  |  |
|  |  |  | 3 01 021 | Latitude/longitude (high accuracy) |  |  |
|  |  |  | 0 01 012 | Direction of motion of moving observing platform |  |  |
|  |  |  | 0 01 014 | Platform drift speed (high precision) |  |  |
|  |  |  | 0 11 104 | True heading of aircraft, ship or other mobile platform |  |  |
|  |  |  | 1 03 000 | Delayed replication of 3 descriptor |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 0 07 031 | Height of barometer above mean sea level |  |  |
|  |  |  | 3 06 038 | Sequence for representation of standard surface marine meteorological observations from moored buoys |  |  |
|  |  |  | 0 12 161 | Skin temperature |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 3 06 034 | Surface current |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 3 06 039 | Sequence for representation of basic wave measurements |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 3 06 033 | Surface salinity |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 3 06 041 | Depth and temperature profile (high accuracy/precision) |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 3 06 004 | Depth, temperature, salinity |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 3 06 005 | Sub-surface current measurements |  |  |
|  |  |  | 1 05 000 | Delayed replication of 5 descriptors |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 0 41 001 | pCO2 |  |  |
|  |  |  | 0 08 043 | Atmospheric chemical or physical constituent type |  |  |
|  |  |  | 0 15 028 | Mole fraction of atmospheric constituent/pollutant in dry air |  |  |
|  |  |  | 0 08 043 | Atmospheric chemical or physical constituent type |  |  |
|  |  |  | 0 13 080 | Water pH |  |  |
|  |  |  | 1 04 000 | Delayed replication of 4 descriptors |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 0 41 005 | Turbidity |  |  |
|  |  |  | 0 41 003 | Dissolved nitrates |  |  |
|  |  |  | 0 22 188 | Dissolved oxygen |  |  |
|  |  |  | 0 41 002 | Fluorescence |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 3 06 040 | Sequence for representation of detailed spectral wave measurements |  |  |
|  |  |  | 1 04 000 | Delayed replication of 4 descriptors |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 0 08 021 | Time significance |  |  |
|  |  |  | 0 04 025 | Time period or displacement |  |  |
|  |  |  | 0 14 017 | Instantaneous long-wave radiation |  |  |
|  |  |  | 0 14 018 | Instantaneous short-wave radiation |  |  |
| 3 15 012 | (Sequence for reporting observations from a single glider trajectory profile) |  | 3 01 150 | WIGOS identifier |  |  |
|  |  |  | 2 01 129 | Change data width | Add one bit to width of following descriptor |  |
|  |  |  | 0 01 087 | WMO marine observing platform extended identifier |  |  |
|  |  |  | 2 01 000 | Change data width | Cancel change width |  |
|  |  |  | 0 01 019 | Long station or site name |  |  |
|  |  |  | 0 01 036 | Agency in charge of operating the observing platform |  |  |
|  |  |  | 0 02 148 | Data collection and/or location system |  |  |
|  |  |  | 0 01 085 | Observing platform manufacturer's model |  |  |
|  |  |  | 0 01 086 | Observing platform manufacturer's serial number |  |  |
|  |  |  | 0 08 021 | Time significance | Set to 25, nominal reporting time |  |
|  |  |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 3 01 013 | Hour, minute, second |  |  |
|  |  |  | 3 01 021 | Latitude/longitude (high accuracy) |  |  |
|  |  |  | 0 11 104 | True heading of aircraft, ship or other mobile platform |  |  |
|  |  |  | 0 02 169 | Anemometer type | Set to 2, wind observation through ambient noise (WOTAN) |  |
|  |  |  | 0 11 002 | Wind speed |  |  |
|  |  |  | 0 11 001 | Wind direction |  |  |
|  |  |  | 0 02 169 | Anemometer type | Set to missing / cancel previous value |  |
|  |  |  | 0 22 032 | Speed of sea-surface current |  |  |
|  |  |  | 0 22 005 | Direction of sea-surface current |  |  |
|  |  |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 3 01 013 | Hour, minute, second |  |  |
|  |  |  | 0 08 021 | Time significance | Set to 2, time averaged |  |
|  |  |  | 0 04 025 | Time period or displacement | Duration of dive, e.g. -10 (minutes) |  |
|  |  |  | 3 01 021 | Latitude/longitude (high accuracy) |  |  |
|  |  |  | 0 22 031 | Speed of current |  |  |
|  |  |  | 0 22 004 | Direction of current |  |  |
|  |  |  | 0 08 021 | Time significance | Set to missing / cancel previous value |  |
|  |  |  | 0 05 068 | Profile number |  |  |
|  |  |  | 0 01 079 | Unique identifier for the profile |  |  |
|  |  |  | 1 26 000 | Delayed replication of 26 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor | Number of profile sections (e.g. descending, horizontal, ascending) |  |
|  |  |  | 0 22 056 | Direction of profile | 0 (ascending / upwards profile), 1 (descending / downwards profile), or 2 (horizontal) |  |
|  |  |  | 1 23 000 | Delayed replication of 23 descriptors |  |  |
|  |  |  | 0 31 002 | Extended delayed descriptor replication factor |  |  |
|  |  |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 3 01 013 | Hour, minute, second |  |  |
|  |  |  | 3 01 021 | Latitude/longitude (high accuracy) |  |  |
|  |  |  | 0 08 080 | Qualifier for GTSPP quality flag | 20, position |  |
|  |  |  | 0 33 050 | Global GTSPP quality flag |  |  |
|  |  |  | 0 07 062 | Depth below sea/water surface |  |  |
|  |  |  | 0 08 080 | Qualifier for GTSPP quality flag | 13, water depth at a level |  |
|  |  |  | 0 33 050 | Global GTSPP quality flag |  |  |
|  |  |  | 0 22 065 | Water pressure |  |  |
|  |  |  | 0 08 080 | Qualifier for GTSPP quality flag | 10, water pressure at a level |  |
|  |  |  | 0 33 050 | Global GTSPP quality flag |  |  |
|  |  |  | 0 22 045 | Sea/water temperature |  |  |
|  |  |  | 0 08 080 | Qualifier for GTSPP quality flag | 11, water temperature at a level |  |
|  |  |  | 0 33 050 | Global GTSPP quality flag |  |  |
|  |  |  | 0 22 066 | Water conductivity |  |  |
|  |  |  | 0 08 080 | Qualifier for GTSPP quality flag | 25, water conductivity at a level |  |
|  |  |  | 0 33 050 | Global GTSPP quality flag |  |  |
|  |  |  | 0 22 064 | Salinity |  |  |
|  |  |  | 0 08 080 | Qualifier for GTSPP quality flag | 12, salinity at a level |  |
|  |  |  | 0 33 050 | Global GTSPP quality flag |  |  |
|  |  |  | 0 42 016 | Sea water potential density referenced to sea surface |  |  |
|  |  |  | 0 08 080 | Qualifier for GTSPP quality flag | 26, sea water potential density at a level |  |
|  |  |  | 0 33 050 | Global GTSPP quality flag |  |  |
| 3 15 013 | (Sequence for reporting trajectory profile data from marine animal tags) |  | 3 01 150 | WIGOS identifier |  | Note 79: It is recommended to use 3 15 023 instead of 3 15 013. |
|  |  |  | 0 01 087 | WMO marine observing platform extended identifier | WMO number where assigned |  |
|  |  |  | 2 08 032 | Change width of CCITT IA5 | Change width to 32 characters |  |
|  |  |  | 0 01 019 | Long station or site name | Platform ID, e.g. ct145-933-BAT2-18 (max 32 characters) |  |
|  |  |  | 2 08 000 | Change width of CCITT IA5 | Cancel change width |  |
|  |  |  | 0 03 001 | Surface station type | 10 (Marine animal) |  |
|  |  |  | 0 22 067 | Instrument type for water temperature and/or salinity measurement | Set to 995 (attached to marine mammals) or 996 (other animals) |  |
|  |  |  | 0 01 051 | Platform transmitter ID number | e.g. Argos PTT |  |
|  |  |  | 0 02 148 | Data collection and/or location system |  |  |
|  |  |  | 1 12 000 | Delayed replication of 12 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 08 021 | Time significance | Set to 26, time of last known position |  |
|  |  |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 3 01 012 | Hour, minute |  |  |
|  |  |  | 3 01 021 | Latitude/longitude (high accuracy) |  |  |
|  |  |  | 0 01 012 | Direction of motion of moving observing platform |  |  |
|  |  |  | 0 01 014 | Platform drift speed (high precision) |  |  |
|  |  |  | 0 33 022 | Quality of buoy satellite transmission |  |  |
|  |  |  | 0 33 023 | Quality buoy location |  |  |
|  |  |  | 0 33 027 | Location quality class (range of radius of 66% confidence) |  |  |
|  |  |  | 0 07 063 | Depth below sea/water surface |  |  |
|  |  |  | 0 22 045 | Sea/water temperature |  |  |
|  |  |  | 0 08 021 | Time significance | Set to missing/cancel |  |
|  |  |  | 1 07 000 | Delayed replication of 7 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 3 01 012 | Hour, minute |  |  |
|  |  |  | 3 01 021 | Latitude/longitude (high accuracy) |  |  |
|  |  |  | 0 01 079 | Unique identifier for the profile | Profile ID |  |
|  |  |  | 0 01 023 | Observation sequence number | Upcast number |  |
|  |  |  | 0 22 056 | Direction of profile | Set to 0 (ascending/upwards) |  |
|  |  |  | 3 06 035 | Temperature and salinity profile |  |  |
| 3 15 023 | (Sequence for reporting trajectory profile data from marine animal tags) |  | 3 01 150 | WIGOS identifier |  |  |
|  |  |  | 2 01 129 | Change width | Add one bit to width of following descriptor |  |
|  |  |  | 0 01 087 | WMO marine observing platform extended identifier | WMO number where assigned |  |
|  |  |  | 2 01 000 | Change width | Cancel change width |  |
|  |  |  | 2 08 032 | Change width of CCITT IA5 | Change width to 32 characters |  |
|  |  |  | 0 01 019 | Long station or site name | Platform ID, e.g. ct145-933-BAT2-18 (max 32 characters) |  |
|  |  |  | 2 08 000 | Change width of CCITT IA5 | Cancel change width |  |
|  |  |  | 0 03 001 | Surface station type | 10 (Marine animal) |  |
|  |  |  | 0 22 067 | Instrument type for water temperature and/or salinity measurement | Set to 995 (attached to marine animal) |  |
|  |  |  | 0 01 051 | Platform transmitter ID number | e.g. Argos PTT |  |
|  |  |  | 0 02 148 | Data collection and/or location system |  |  |
|  |  |  | 1 12 000 | Delayed replication of 12 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 08 021 | Time significance | Set to 26, time of last known position |  |
|  |  |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 3 01 012 | Hour, minute |  |  |
|  |  |  | 3 01 021 | Latitude/longitude (high accuracy) |  |  |
|  |  |  | 0 01 012 | Direction of motion of moving observing platform |  |  |
|  |  |  | 0 01 014 | Platform drift speed (high precision) |  |  |
|  |  |  | 0 33 022 | Quality of buoy satellite transmission |  |  |
|  |  |  | 0 33 023 | Quality buoy location |  |  |
|  |  |  | 0 33 027 | Location quality class (range of radius of 66% confidence) |  |  |
|  |  |  | 0 07 063 | Depth below sea/water surface |  |  |
|  |  |  | 0 22 045 | Sea/water temperature |  |  |
|  |  |  | 0 08 021 | Time significance | Set to missing/cancel |  |
|  |  |  | 1 07 000 | Delayed replication of 7 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 3 01 012 | Hour, minute |  |  |
|  |  |  | 3 01 021 | Latitude/longitude (high accuracy) |  |  |
|  |  |  | 0 01 079 | Unique identifier for the profile | Profile ID |  |
|  |  |  | 0 01 023 | Observation sequence number | Upcast number |  |
|  |  |  | 0 22 056 | Direction of profile | Set to 0 (ascending/upwards) |  |
|  |  |  | 3 06 035 | Temperature and salinity profile |  |  |