Class 12: Temperature

| FXY | ELEMENT NAME | BUFR UNIT | SCALE | REF VALUE | BITS | CREX UNIT | SCALE | CHARS | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 12 001 | Temperature/air temperature | K | 1 | 0 | 12 | C | 1 | 3 |  |
| 0 12 002 | Wet-bulb temperature | K | 1 | 0 | 12 | C | 1 | 3 |  |
| 0 12 003 | Dewpoint temperature | K | 1 | 0 | 12 | C | 1 | 3 |  |
| 0 12 004 | Air temperature at 2 m | K | 1 | 0 | 12 | C | 1 | 3 |  |
| 0 12 005 | Wet-bulb temperature at 2 m | K | 1 | 0 | 12 | C | 1 | 3 |  |
| 0 12 006 | Dewpoint temperature at 2 m | K | 1 | 0 | 12 | C | 1 | 3 |  |
| 0 12 007 | Virtual temperature | K | 1 | 0 | 12 | C | 1 | 3 |  |
| 0 12 008 | Uncertainty in virtual temperature | K | 1 | 0 | 12 | C | 1 | 4 |  |
| 0 12 011 | Maximum temperature, at height and over period specified | K | 1 | 0 | 12 | C | 1 | 3 | Note 177: Where the expression "at height and over period specified" is entered under element name, an appropriate vertical location shall be specified using descriptors from Class 07, together with an appropriate period using descriptors from Class 04. |
| 0 12 012 | Minimum temperature, at height and over period specified | K | 1 | 0 | 12 | C | 1 | 3 | Note 177: Where the expression "at height and over period specified" is entered under element name, an appropriate vertical location shall be specified using descriptors from Class 07, together with an appropriate period using descriptors from Class 04. |
| 0 12 013 | Ground minimum temperature, past 12 hours | K | 1 | 0 | 12 | C | 1 | 3 |  |
| 0 12 014 | Maximum temperature at 2 m, past 12 hours | K | 1 | 0 | 12 | C | 1 | 3 |  |
| 0 12 015 | Minimum temperature at 2 m, past 12 hours | K | 1 | 0 | 12 | C | 1 | 3 |  |
| 0 12 016 | Maximum temperature at 2 m, past 24 hours | K | 1 | 0 | 12 | C | 1 | 3 |  |
| 0 12 017 | Minimum temperature at 2 m, past 24 hours | K | 1 | 0 | 12 | C | 1 | 3 |  |
| 0 12 021 | Maximum temperature at 2 m | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 022 | Minimum temperature at 2 m | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 023 | Temperature | C | 0 | -99 | 8 | C | 0 | 2 |  |
| 0 12 024 | Dewpoint temperature | C | 0 | -99 | 8 | C | 0 | 2 |  |
| 0 12 030 | Soil temperature | K | 1 | 0 | 12 | C | 1 | 3 |  |
| 0 12 049 | Temperature change over specified period | K | 0 | -30 | 6 | C | 0 | 2 |  |
| 0 12 051 | Standard deviation temperature | K | 1 | 0 | 10 | C | 1 | 3 |  |
| 0 12 052 | Highest daily mean temperature | K | 1 | 0 | 12 | C | 1 | 3 |  |
| 0 12 053 | Lowest daily mean temperature | K | 1 | 0 | 12 | C | 1 | 3 |  |
| 0 12 060 | AWS enclosure internal temperature | K | 1 | 0 | 12 | C | 1 | 3 |  |
| 0 12 061 | Skin temperature | K | 1 | 0 | 12 | C | 1 | 3 |  |
| 0 12 062 | Equivalent black body temperature | K | 1 | 0 | 12 | C | 1 | 3 |  |
| 0 12 063 | Brightness temperature | K | 1 | 0 | 12 | C | 1 | 3 |  |
| 0 12 064 | Instrument temperature | K | 1 | 0 | 12 | K | 1 | 4 |  |
| 0 12 065 | Standard deviation brightness temperature | K | 1 | 0 | 12 | K | 1 | 4 |  |
| 0 12 066 | Antenna temperature | K | 2 | 0 | 16 | C | 2 | 5 |  |
| 0 12 070 | Warm load temperature | K | 2 | 0 | 16 | K | 2 | 5 |  |
| 0 12 071 | Coldest cluster temperature | K | 1 | 0 | 12 | K | 1 | 4 |  |
| 0 12 072 | Radiance | W m-2 sr-1 | 6 | 0 | 31 | W m-2 sr-1 | 6 | 9 |  |
| 0 12 075 | Spectral radiance | W m-3 sr-1 | -3 | 0 | 16 | W m-3 sr-1 | -3 | 5 |  |
| 0 12 076 | Radiance | W m-2 sr-1 | 3 | 0 | 16 | W m-2 sr-1 | 3 | 5 | Note 52: The Quality information descriptor 0 33 002 at this point relates to both u-component 0 11 003 and v-component 0 11 004. It is given for each respective gate of the profile measurement. |
| 0 12 080 | Brightness temperature real part | K | 2 | -10000 | 16 | K | 2 | 5 |  |
| 0 12 081 | Brightness temperature imaginary part | K | 2 | -10000 | 16 | K | 2 | 5 |  |
| 0 12 082 | Pixel radiometric accuracy | K | 2 | 0 | 12 | K | 2 | 4 |  |
| 0 12 083 | Temperature difference between observed and computed | K | 2 | -10000 | 16 | K | 2 | 6 |  |
| 0 12 101 | Temperature/air temperature | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 102 | Wet-bulb temperature | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 103 | Dewpoint temperature | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 104 | Air temperature at 2 m | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 105 | Web-bulb temperature at 2 m | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 106 | Dewpoint temperature at 2 m | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 107 | Virtual temperature | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 111 | Maximum temperature, at height and over period specified | K | 2 | 0 | 16 | C | 2 | 4 | Note 177: Where the expression "at height and over period specified" is entered under element name, an appropriate vertical location shall be specified using descriptors from Class 07, together with an appropriate period using descriptors from Class 04. |
| 0 12 112 | Minimum temperature, at height and over period specified | K | 2 | 0 | 16 | C | 2 | 4 | Note 177: Where the expression "at height and over period specified" is entered under element name, an appropriate vertical location shall be specified using descriptors from Class 07, together with an appropriate period using descriptors from Class 04. |
| 0 12 113 | Ground minimum temperature, past 12 hours | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 114 | Maximum temperature at 2 m, past 12 hours | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 115 | Minimum temperature at 2 m, past 12 hours | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 116 | Maximum temperature at 2 m, past 24 hours | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 117 | Minimum temperature at 2 m, past 24 hours | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 118 | Maximum temperature at height specified, past 24 hours | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 119 | Minimum temperature at height specified, past 24 hours | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 120 | Ground temperature | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 121 | Ground minimum temperature | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 122 | Ground minimum temperature of the preceding night | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 128 | Road surface temperature | K | 2 | 0 | 16 | C | 2 | 5 |  |
| 0 12 129 | Road subsurface temperature | K | 2 | 0 | 16 | C | 2 | 5 |  |
| 0 12 130 | Soil temperature | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 131 | Snow temperature | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 132 | Ice surface temperature | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 133 | Ice temperature | K | 2 | 0 | 16 |  |  |  |  |
| 0 12 151 | Standard deviation of daily mean temperature | K | 2 | 0 | 12 | C | 2 | 4 |  |
| 0 12 152 | Highest daily mean temperature | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 153 | Lowest daily mean temperature | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 158 | Noise-equivalent delta temperature while viewing cold target | K | 2 | 0 | 12 | C | 2 | 4 |  |
| 0 12 159 | Noise-equivalent delta temperature while viewing warm target | K | 2 | 0 | 12 | C | 2 | 4 |  |
| 0 12 161 | Skin temperature | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 162 | Equivalent black body temperature | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 163 | Brightness temperature | K | 2 | 0 | 16 | C | 2 | 4 |  |
| 0 12 164 | Instrument temperature | K | 2 | 0 | 16 | K | 2 | 5 |  |
| 0 12 165 | Direct sun brightness temperature | K | 0 | 0 | 23 | K | 0 | 7 |  |
| 0 12 166 | Snapshot accuracy | K | 1 | -4000 | 13 | K | 1 | 4 |  |
| 0 12 167 | Radiometric accuracy (pure polarization) | K | 1 | 0 | 9 | K | 1 | 3 |  |
| 0 12 168 | Radiometric accuracy (cross polarization) | K | 1 | 0 | 9 | K | 1 | 3 |  |
| 0 12 169 | Brightness temperature difference | K | 3 | -100000 | 18 | K | 3 | 6 |  |
| 0 12 171 | Coldest cluster temperature | K | 2 | 0 | 16 | K | 2 | 5 |  |
| 0 12 180 | Averaged 12 micron BT for all clear pixels at nadir | K | 2 | 0 | 16 | K | 2 | 5 |  |
| 0 12 181 | Averaged 11 micron BT for all clear pixels at nadir | K | 2 | 0 | 16 | K | 2 | 5 |  |
| 0 12 182 | Averaged 3.7 micron BT for all clear pixels at nadir | K | 2 | 0 | 16 | K | 2 | 5 |  |
| 0 12 183 | Averaged 12 micron BT for all clear pixels, forward view | K | 2 | 0 | 16 | K | 2 | 5 |  |
| 0 12 184 | Averaged 11 micron BT for all clear pixels, forward view | K | 2 | 0 | 16 | K | 2 | 5 |  |
| 0 12 185 | Averaged 3.7 micron BT for all clear pixels, forward view | K | 2 | 0 | 16 | K | 2 | 5 |  |
| 0 12 186 | Mean nadir sea-surface temperature | K | 2 | 0 | 16 | K | 2 | 5 |  |
| 0 12 187 | Mean dual view sea-surface temperature | K | 2 | 0 | 16 | K | 2 | 5 |  |
| 0 12 188 | Interpolated 23.8 GHz brightness T from MWR | K | 2 | 0 | 16 | K | 2 | 5 |  |
| 0 12 189 | Interpolated 36.5 GHz brightness T from MWR | K | 2 | 0 | 16 | K | 2 | 5 |  |
| 0 12 190 | Brightness temperature error covariance | K2 | 3 | -15000 | 15 | K2 | 3 | 8 |  |