Class 30: Image

| FXY | ELEMENT NAME | BUFR UNIT | SCALE | REF VALUE | BITS | CREX UNIT | SCALE | CHARS | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 30 001 | Pixel value (4 bits) | Numeric | 0 | 0 | 4 | Numeric | 0 | 2 |  |
| 0 30 002 | Pixel value (8 bits) | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 |  |
| 0 30 004 | Pixel value (16 bits) | Numeric | 0 | 0 | 16 | Numeric | 0 | 5 |  |
| 0 30 010 | Number of grid points | Numeric | 0 | 0 | 13 | Numeric | 0 | 4 |  |
| 0 30 021 | Number of pixels per row | Numeric | 0 | 0 | 12 | Numeric | 0 | 4 |  |
| 0 30 022 | Number of pixels per column | Numeric | 0 | 0 | 12 | Numeric | 0 | 4 |  |
| 0 30 031 | Picture type | Code table | 0 | 0 | 4 | Code table | 0 | 2 |  |
| 0 30 032 | Combination with other data | Flag table | 0 | 0 | 16 | Flag table | 0 | 6 |  |
| 0 30 033 | Number of bins along the radial | Numeric | 0 | 0 | 12 | Numeric | 0 | 4 |  |
| 0 30 034 | Number of azimuths | Numeric | 0 | 0 | 12 | Numeric | 0 | 4 |  |

Note 92: In order to distinguish unambiguously the cases of missing data and saturated pixels, n-bit image data should be encoded using a data width of n+1. Where such a descriptor is not already available in Class 30, operator descriptor 2 01 YYY should be used to modify the data width of the existing entry as required.

Note 115: Pixel data width can be changed with descriptor 2 01 YYY.