Class 03: Instrumentation

| FXY | ELEMENT NAME | BUFR UNIT | SCALE | REF VALUE | BITS | CREX UNIT | SCALE | CHARS | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 03 001 | Surface station type | Code table | 0 | 0 | 5 | Code table | 0 | 2 |  |
| 0 03 003 | Thermometer/hygrometer housing | Code table | 0 | 0 | 4 | Code table | 0 | 2 |  |
| 0 03 004 | Type of screen/shelter/radiation shield | Code table | 0 | 0 | 4 | Code table | 0 | 2 |  |
| 0 03 005 | Horizontal width of screen or shield (x) | m | 3 | 0 | 16 | m | 3 | 5 |  |
| 0 03 006 | Horizontal depth of screen or shield (y) | m | 3 | 0 | 16 | m | 3 | 5 |  |
| 0 03 007 | Vertical height of screen or shield (z) | m | 3 | 0 | 16 | m | 3 | 5 |  |
| 0 03 008 | Artificially ventilated screen or shield | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 03 009 | Amount of forced ventilation at time of reading | m/s | 1 | 0 | 9 | m/s | 1 | 3 |  |
| 0 03 010 | Method of sea/water current measurement | Code table | 0 | 0 | 4 | Code table | 0 | 2 |  |
| 0 03 011 | Method of depth calculation | Code table | 0 | 0 | 2 | Code table | 0 | 1 |  |
| 0 03 012 | Instrument type/sensor for dissolved oxygen measurement | Code table | 0 | 0 | 4 | Code table | 0 | 2 |  |
| 0 03 016 | Position of road sensors | Code table | 0 | 0 | 4 | Code table | 0 | 2 |  |
| 0 03 017 | Extended type of station | Flag table | 0 | 0 | 6 | Flag table | 0 | 2 |  |
| 0 03 018 | Type of road | Code table | 0 | 0 | 5 | Code table | 0 | 2 |  |
| 0 03 019 | Type of construction | Code table | 0 | 0 | 4 | Code table | 0 | 2 |  |
| 0 03 020 | Material for thermometer/hygrometer housing | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 03 021 | Hygrometer heating | Code table | 0 | 0 | 2 | Code table | 0 | 1 |  |
| 0 03 022 | Instrument owner | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 03 023 | Configuration of louvers for thermometer/hygrometer screen | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 03 024 | Psychrometric coefficient | K-1 | 6 | 0 | 10 | K-1 | 6 | 3 |  |
| 0 03 025 | Cross-track estimation area size | m | 0 | 5000 | 16 | m | 0 | 5 |  |
| 0 03 026 | Along-track estimation area size | m | 0 | 5000 | 16 | m | 0 | 5 |  |
| 0 03 027 | Type of flight rig | Code table | 0 | 0 | 4 | Code table | 0 | 2 |  |
| 0 03 028 | Method of snow water equivalent measurement | Code table | 0 | 0 | 6 | Code table | 0 | 2 |  |
| 0 03 029 | Swath | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 03 030 | Antenna status | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 03 031 | Channel bandwidth, full width half maximum (FWHM) | m | 11 | 0 | 16 | m | 11 | 13 |  |
| 0 03 032 | Channel centre frequency | Hz | -6 | 0 | 26 | Hz | -6 | 10 |  |
| 0 03 033 | Channel bandwidth | Hz | -6 | 0 | 26 | Hz | -6 | 10 |  |
| 0 03 034 | Channel wavelength | m | 9 | 0 | 16 | m | 9 | 5 |  |