Category 16: Synoptic feature sequences

| FXY1 | TITLE | SUBTITLE | FXY2 | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| 3 16 001 |  |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 0 04 004 | Hour |  |  |
|  |  |  | 3 01 023 | Latitude/longitude (coarse accuracy) |  |  |
|  |  |  | 0 01 021 | Synoptic feature identifier |  |  |
|  |  |  | 0 02 041 | Method for estimating reports related to synoptic features |  |  |
|  |  |  | 0 19 001 | Type of synoptic feature |  |  |
|  |  |  | 0 10 051 | Pressure reduced to mean sea level |  |  |
|  |  |  | 0 19 002 | Effective radius of feature |  |  |
|  |  |  | 0 19 003 | Wind speed threshold | 15 m s-1 typically |  |
|  |  |  | 0 19 004 | Effective radius with respect to wind speeds above threshold |  |  |
| 3 16 002 | (Header) |  | 0 08 021 | Time significance | Data time (analysis) |  |
|  |  |  | 0 04 001 | Year |  |  |
|  |  |  | 0 04 002 | Month |  |  |
|  |  |  | 0 04 003 | Day |  |  |
|  |  |  | 0 04 004 | Hour |  |  |
|  |  |  | 0 04 005 | Minute |  |  |
|  |  |  | 0 01 033 | Identification of originating/generating centre |  |  |
|  |  |  | 0 08 021 | Time significance | Validity time (forecast) |  |
|  |  |  | 0 04 001 | Year |  |  |
|  |  |  | 0 04 002 | Month |  |  |
|  |  |  | 0 04 003 | Day |  |  |
|  |  |  | 0 04 004 | Hour |  |  |
|  |  |  | 0 04 005 | Minute |  |  |
|  |  |  | 0 07 002 | Height or altitude | Flight level (base of chart layer) |  |
|  |  |  | 0 07 002 | Height or altitude | Flight level (top of chart layer) |  |
| 3 16 003 | (Jet stream) |  | 1 10 000 | Delayed replication of 10 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 08 011 | Meteorological feature | Jet stream value |  |
|  |  |  | 0 08 007 | Dimensional significance | Value for line |  |
|  |  |  | 1 04 000 | Delayed replication of 4 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 05 002 | Latitude (coarse accuracy) |  |  |
|  |  |  | 0 06 002 | Longitude (coarse accuracy) |  |  |
|  |  |  | 0 10 002 | Height | Flight level |  |
|  |  |  | 0 11 002 | Wind speed |  |  |
|  |  |  | 0 08 007 | Dimensional significance | Cancel |  |
|  |  |  | 0 08 011 | Meteorological feature | Cancel ¦ End of object |  |
| 3 16 004 | (Turbulence) |  | 1 11 000 | Delayed replication of 11 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 08 011 | Meteorological feature | Value for turbulence |  |
|  |  |  | 0 08 007 | Dimensional significance | Value for area |  |
|  |  |  | 0 07 002 | Height or altitude | Flight level (base of layer) |  |
|  |  |  | 0 07 002 | Height or altitude | Flight level (top of layer) |  |
|  |  |  | 1 02 000 | Delayed replication of 2 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 05 002 | Latitude (coarse accuracy) |  |  |
|  |  |  | 0 06 002 | Longitude (coarse accuracy) |  |  |
|  |  |  | 0 11 031 | Degree of turbulence |  | Note 20: For MOD OCNL SEV code as 12 (extreme in clear air) or 13 (extreme in cloud). |
|  |  |  | 0 08 007 | Dimensional significance | Cancel |  |
|  |  |  | 0 08 011 | Meteorological feature | Cancel ¦ End of object |  |
| 3 16 005 | (Storm) |  | 1 08 000 | Delayed replication of 8 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 08 005 | Meteorological attribute significance | Storm centre |  |
|  |  |  | 0 08 007 | Dimensional significance | Value for point |  |
|  |  |  | 0 05 002 | Latitude (coarse accuracy) |  |  |
|  |  |  | 0 06 002 | Longitude (coarse accuracy) |  |  |
|  |  |  | 0 01 026 | WMO storm name | Use "UNKNOWN" for a sandstorm |  |
|  |  |  | 0 19 001 | Type of synoptic feature | Value for type of storm |  |
|  |  |  | 0 08 007 | Dimensional significance | Cancel |  |
|  |  |  | 0 08 005 | Meteorological attribute significance | Cancel ¦ End of object |  |
| 3 16 006 | (Cloud) |  | 1 12 000 | Delayed replication of 12 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 08 011 | Meteorological feature | Value for cloud |  |
|  |  |  | 0 08 007 | Dimensional significance | Value for area |  |
|  |  |  | 0 07 002 | Height or altitude | Flight level (base of layer) |  |
|  |  |  | 0 07 002 | Height or altitude | Flight level (top of layer) |  |
|  |  |  | 1 02 000 | Delayed replication of 2 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 05 002 | Latitude (coarse accuracy) |  |  |
|  |  |  | 0 06 002 | Longitude (coarse accuracy) |  |  |
|  |  |  | 0 20 011 | Cloud amount |  | Note 6: Code table values: FRQ = code figure 8 (8 oktas) OCNL EMBD = code figure 6 (6 oktas) ISOL = code figure 2 (2 oktas) when the cloud = Cb. |
|  |  |  | 0 20 012 | Cloud type |  |  |
|  |  |  | 0 08 007 | Dimensional significance | Cancel |  |
|  |  |  | 0 08 011 | Meteorological feature | Cancel ¦ End of object |  |
| 3 16 007 | (Front) |  | 1 10 000 | Delayed replication of 10 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 08 011 | Meteorological feature | Value for type of front | Note 21: Front direction (towards which the front is moving) must always be given as it is needed for plotting purposes. A front direction with a front speed of zero would indicate a slow front. A value in the code table exists to represent a quasi-stationary front. |
|  |  |  | 0 08 007 | Dimensional significance | Value for line |  |
|  |  |  | 1 04 000 | Delayed replication of 4 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 05 002 | Latitude (coarse accuracy) |  |  |
|  |  |  | 0 06 002 | Longitude (coarse accuracy) |  |  |
|  |  |  | 0 19 005 | Direction of motion of feature |  |  |
|  |  |  | 0 19 006 | Speed of motion of feature |  |  |
|  |  |  | 0 08 007 | Dimensional significance | Cancel |  |
|  |  |  | 0 08 011 | Meteorological feature | Cancel ¦ End of object |  |
| 3 16 008 | (Tropopause) |  | 1 11 000 | Delayed replication of 11 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 08 001 | Vertical sounding significance | Bit 3 set for tropopause |  |
|  |  |  | 0 08 007 | Dimensional significance | Value for point |  |
|  |  |  | 0 08 023 | First-order statistics | Type of tropopause value | Note 58: The statistic is to determine whether the following tropopause levels are minimum, maximum or spot values (missing code value). |
|  |  |  | 1 03 000 | Delayed replication of 3 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 05 002 | Latitude (coarse accuracy) |  |  |
|  |  |  | 0 06 002 | Longitude (coarse accuracy) |  |  |
|  |  |  | 0 10 002 | Height |  |  |
|  |  |  | 0 08 023 | First-order statistics | Cancel |  |
|  |  |  | 0 08 007 | Dimensional significance | Cancel |  |
|  |  |  | 0 08 001 | Vertical sounding significance | Cancel ¦ End of object |  |
| 3 16 009 | (Airframe icing area) |  | 1 11 000 | Delayed replication of 11 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 08 011 | Meteorological feature | Value for airframe icing |  |
|  |  |  | 0 08 007 | Dimensional significance | Value for area |  |
|  |  |  | 0 07 002 | Height or altitude | Flight level (base of layer) |  |
|  |  |  | 0 07 002 | Height or altitude | Flight level (top of layer) |  |
|  |  |  | 1 02 000 | Delayed replication of 2 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 05 002 | Latitude (coarse accuracy) |  |  |
|  |  |  | 0 06 002 | Longitude (coarse accuracy) |  |  |
|  |  |  | 0 20 041 | Airframe icing | Type of airframe icing |  |
|  |  |  | 0 08 007 | Dimensional significance | Cancel |  |
|  |  |  | 0 08 011 | Meteorological feature | Cancel ¦ End of object |  |
| 3 16 010 | (Name of feature) |  | 1 07 000 | Delayed replication of 7 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 08 011 | Meteorological feature |  |  |
|  |  |  | 0 08 007 | Dimensional significance | Value for point |  |
|  |  |  | 0 01 022 | Name of feature |  |  |
|  |  |  | 0 05 002 | Latitude (coarse accuracy) |  |  |
|  |  |  | 0 06 002 | Longitude (coarse accuracy) |  |  |
|  |  |  | 0 08 007 | Dimensional significance | Cancel |  |
|  |  |  | 0 08 011 | Meteorological feature | Cancel ¦ End of object |  |
| 3 16 011 | (Volcano erupting) |  | 1 17 000 | Delayed replication of 17 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 08 011 | Meteorological feature | Value for special clouds |  |
|  |  |  | 0 01 022 | Name of feature | Volcano name |  |
|  |  |  | 0 08 007 | Dimensional significance | Value for point |  |
|  |  |  | 1 02 000 | Delayed replication of 2 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 05 002 | Latitude (coarse accuracy) |  |  |
|  |  |  | 0 06 002 | Longitude (coarse accuracy) |  |  |
|  |  |  | 0 08 021 | Time significance | Eruption starting time |  |
|  |  |  | 0 04 001 | Year |  |  |
|  |  |  | 0 04 002 | Month |  |  |
|  |  |  | 0 04 003 | Day |  |  |
|  |  |  | 0 04 004 | Hour |  |  |
|  |  |  | 0 04 005 | Minute |  |  |
|  |  |  | 0 20 090 | Special clouds | Clouds from volcanic eruptions |  |
|  |  |  | 0 08 021 | Time significance | Cancel |  |
|  |  |  | 0 08 007 | Dimensional significance | Cancel |  |
|  |  |  | 0 08 011 | Meteorological feature | Cancel ¦ End of object |  |
| 3 16 020 | (Tropical storm identification) |  | 0 01 033 | Identification of originating/generating centre |  |  |
|  |  |  | 0 01 025 | Storm identifier |  |  |
|  |  |  | 0 01 027 | WMO long storm name |  |  |
|  |  |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 3 01 012 | Hour, minute |  |  |
| 3 16 021 | (Analysis data) |  | 3 01 023 | Latitude/longitude (coarse accuracy) |  |  |
|  |  |  | 0 02 041 | Method for estimating reports related to synoptic features |  |  |
|  |  |  | 0 19 001 | Type of synoptic feature |  |  |
|  |  |  | 0 19 007 | Effective radius of feature |  |  |
|  |  |  | 0 19 005 | Direction of motion of feature |  |  |
|  |  |  | 0 19 006 | Speed of motion of feature |  |  |
|  |  |  | 0 19 008 | Vertical extent of circulation |  |  |
|  |  |  | 0 08 005 | Meteorological attribute significance | = 1 Storm centre |  |
|  |  |  | 0 10 004 | Pressure | Storm centre by virtue of preceding significance qualifier |  |
|  |  |  | 0 08 005 | Meteorological attribute significance | = 2 Outer limit or edge of feature |  |
|  |  |  | 0 10 004 | Pressure | Outer limit |  |
|  |  |  | 0 19 007 | Effective radius of feature | Outer limit |  |
|  |  |  | 0 08 005 | Meteorological attribute significance | = 3 Location of maximum wind |  |
|  |  |  | 0 08 021 | Time significance | Time averaged |  |
|  |  |  | 0 04 075 | Short time period or displacement | Minutes |  |
|  |  |  | 0 11 040 | Maximum wind speed (mean wind) |  |  |
|  |  |  | 0 19 007 | Effective radius of feature | Maximum wind |  |
|  |  |  | 1 05 004 | Replicate 5 descriptors 4 times |  |  |
|  |  |  | 0 05 021 | Bearing or azimuth | Starting |  |
|  |  |  | 0 05 021 | Bearing or azimuth | Ending |  |
|  |  |  | 1 02 002 | Replicate 2 descriptors 2 times |  |  |
|  |  |  | 0 19 003 | Wind speed threshold |  |  |
|  |  |  | 0 19 004 | Effective radius with respect to wind speeds above threshold |  |  |
| 3 16 022 | (Forecast data) |  | 0 01 032 | Generating application | NWP model name, etc. code table defined by originating/generating centre |  |
|  |  |  | 0 02 041 | Method for estimating reports related to synoptic features |  |  |
|  |  |  | 0 19 001 | Type of synoptic feature |  |  |
|  |  |  | 0 19 010 | Method for tracking the centre of synoptic feature |  |  |
|  |  |  | 1 18 000 | Delayed replication of 18 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 08 021 | Time significance | Forecast |  |
|  |  |  | 0 04 014 | Time increment | Hours |  |
|  |  |  | 0 08 005 | Meteorological attribute significance | Surface synoptic feature |  |
|  |  |  | 3 01 023 | Latitude/longitude (coarse accuracy) |  |  |
|  |  |  | 0 19 005 | Direction of motion of feature |  |  |
|  |  |  | 0 19 006 | Speed of motion of feature |  |  |
|  |  |  | 0 10 004 | Pressure |  |  |
|  |  |  | 0 11 041 | Maximum wind gust speed | For example, used in the United States |  |
|  |  |  | 0 08 021 | Time significance | Forecast time averaged |  |
|  |  |  | 0 04 075 | Short time period or displacement | Minutes |  |
|  |  |  | 0 11 040 | Maximum wind speed (mean wind) |  |  |
|  |  |  | 0 19 008 | Vertical extent of circulation |  |  |
|  |  |  | 1 05 004 | Replicate 5 descriptors 4 times |  |  |
|  |  |  | 0 05 021 | Bearing or azimuth | Starting |  |
|  |  |  | 0 05 021 | Bearing or azimuth | Ending |  |
|  |  |  | 1 02 002 | Replicate 2 descriptors 2 times |  |  |
|  |  |  | 0 19 003 | Wind speed threshold |  |  |
|  |  |  | 0 19 004 | Effective radius with respect to wind speeds above threshold |  |  |
| 3 16 026 | (Tropical storm analysis information) |  | 3 16 020 | Tropical storm identification |  |  |
|  |  |  | 3 16 021 | Analysis data |  |  |
| 3 16 027 | (Tropical storm forecast information) |  | 3 16 020 | Tropical storm identification |  |  |
|  |  |  | 3 16 022 | Forecast data |  |  |
| 3 16 030 | (SIGMET header) |  | 3 01 014 | Time period | For which SIGMET is valid |  |
|  |  |  | 0 01 037 | SIGMET sequence identifier |  |  |
|  |  |  | 0 10 064 | SIGMET cruising level |  |  |
|  |  |  | 0 08 019 | Qualifier for following centre identifier | = 1 ATS unit serving FIR |  |
|  |  |  | 0 01 062 | Short ICAO location indicator |  |  |
|  |  |  | 0 08 019 | Qualifier for following centre identifier | = 2 FIR, = 3 UIR, = 4 CTA |  |
|  |  |  | 0 01 065 | ICAO region identifier |  |  |
|  |  |  | 0 08 019 | Qualifier for following centre identifier | = 6 MWO |  |
|  |  |  | 0 01 062 | Short ICAO location indicator |  |  |
|  |  |  | 0 08 019 | Qualifier for following centre identifier | Set to missing (cancel) |  |
| 3 16 031 | (SIGMET, Observed or forecast location and motion) |  | 0 08 021 | Time significance | = 16 Analysis, = 4 Forecast |  |
|  |  |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 3 01 012 | Hour, minute |  |  |
|  |  |  | 3 01 027 | Description of a feature in 3-D or 2-D |  |  |
|  |  |  | 0 19 005 | Direction of motion of feature |  |  |
|  |  |  | 0 19 006 | Speed of motion of feature |  |  |
|  |  |  | 0 20 028 | Expected change in intensity |  |  |
|  |  |  | 0 08 021 | Time significance | Set to missing (cancel) |  |
| 3 16 032 | (SIGMET, Forecast position) |  | 0 08 021 | Time significance | = 4 Forecast |  |
|  |  |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 3 01 012 | Hour, minute |  |  |
|  |  |  | 3 01 027 | Description of a feature in 3-D or 2-D |  |  |
|  |  |  | 0 08 021 | Time significance | Set to missing (cancel) |  |
| 3 16 033 | (SIGMET, Outlook) |  | 0 08 021 | Time significance | = 4 Forecast |  |
|  |  |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 3 01 012 | Hour, minute |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 3 01 027 | Description of a feature in 3-D or 2-D |  |  |
|  |  |  | 0 08 021 | Time significance | Set to missing (cancel) |  |
| 3 16 034 | (Volcanic Ash SIGMET) |  | 0 08 079 | Product status | = 0 Normal issue, = 1 Correction |  |
|  |  |  | 3 16 030 | SIGMET header |  |  |
|  |  |  | 0 08 011 | Meteorological feature | = 17 Volcano |  |
|  |  |  | 0 01 022 | Name of feature |  |  |
|  |  |  | 0 08 007 | Dimensional significance | = 0 Point |  |
|  |  |  | 3 01 023 | Latitude/longitude (coarse accuracy) |  |  |
|  |  |  | 0 08 007 | Dimensional significance | Set to missing (cancel) |  |
|  |  |  | 0 20 090 | Special clouds | = 5 Clouds from volcanic eruptions |  |
|  |  |  | 3 16 031 | SIGMET, Observed or forecast location and motion |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 3 16 032 | SIGMET, Forecast position |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 3 16 033 | SIGMET, Outlook |  |  |
|  |  |  | 0 08 011 | Meteorological feature | Set to missing (cancel) |  |
|  |  |  | 0 08 079 | Product status | Set to missing (cancel) |  |
| 3 16 035 | (Thunderstorm SIGMET) |  | 0 08 079 | Product status | = 0 Normal issue, = 1 Correction |  |
|  |  |  | 3 16 030 | SIGMET header |  |  |
|  |  |  | 0 08 011 | Meteorological feature | = 21 Thunderstorm |  |
|  |  |  | 0 20 023 | Other weather phenomena | Bit 2 = Squalls or all 18 bits = Missing |  |
|  |  |  | 0 20 021 | Type of precipitation | Bit 14 = Hail or all 30 bits = Missing |  |
|  |  |  | 0 20 008 | Cloud distribution for aviation | = 15 OBSC, = 16 EMBD, = 12 FRQ, = 31 Missing |  |
|  |  |  | 3 16 031 | SIGMET, Observed or forecast location and motion |  |  |
|  |  |  | 0 08 011 | Meteorological feature | Set to missing (cancel) |  |
|  |  |  | 0 08 079 | Product status | Set to missing (cancel) |  |
| 3 16 036 | (Tropical cyclone SIGMET) |  | 0 08 079 | Product status | = 0 Normal issue, = 1 Correction |  |
|  |  |  | 3 16 030 | SIGMET header |  |  |
|  |  |  | 0 08 011 | Meteorological feature | = 22 Tropical cyclone |  |
|  |  |  | 0 01 027 | WMO long storm name |  |  |
|  |  |  | 3 16 031 | SIGMET, Observed or forecast location and motion |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 000 | Short delayed descriptor replication factor |  |  |
|  |  |  | 3 16 032 | SIGMET, Forecast position |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 3 16 033 | SIGMET, Outlook |  |  |
|  |  |  | 0 08 011 | Meteorological feature | Set to missing (cancel) |  |
|  |  |  | 0 08 079 | Product status | Set to missing (cancel) |  |
| 3 16 037 | (Turbulence SIGMET) |  | 0 08 079 | Product status | = 0 Normal issue, = 1 Correction |  |
|  |  |  | 3 16 030 | SIGMET header |  |  |
|  |  |  | 0 08 011 | Meteorological feature | = 13 Turbulence |  |
|  |  |  | 0 11 031 | Degree of turbulence | = 10 Moderate, = 11 Severe |  |
|  |  |  | 3 16 031 | SIGMET, Observed or forecast location and motion |  |  |
|  |  |  | 0 08 011 | Meteorological feature | Set to missing (cancel) |  |
|  |  |  | 0 08 079 | Product status | Set to missing (cancel) |  |
| 3 16 038 | (Icing SIGMET) |  | 0 08 079 | Product status | = 0 Normal issue, = 1 Correction |  |
|  |  |  | 3 16 030 | SIGMET header |  |  |
|  |  |  | 0 08 011 | Meteorological feature | = 15 Airframe icing |  |
|  |  |  | 0 20 041 | Airframe icing | = 7 Severe |  |
|  |  |  | 0 20 021 | Type of precipitation | Bit 3 = Liquid freezing or all 30 bits = Missing |  |
|  |  |  | 3 16 031 | SIGMET, Observed or forecast location and motion |  |  |
|  |  |  | 0 08 011 | Meteorological feature | Set to missing (cancel) |  |
|  |  |  | 0 08 079 | Product status | Set to missing (cancel) |  |
| 3 16 039 | (Mountain wave, duststorm or sandstorm SIGMET) |  | 0 08 079 | Product status | = 0 Normal issue, = 1 Correction |  |
|  |  |  | 3 16 030 | SIGMET header |  |  |
|  |  |  | 0 08 011 | Meteorological feature | = 23 Mountain wave, = 24 Duststorm, = 25 Sandstorm |  |
|  |  |  | 0 20 024 | Intensity of phenomena | = 3 Heavy, = 5 Severe |  |
|  |  |  | 3 16 031 | SIGMET, Observed or forecast location and motion |  |  |
|  |  |  | 0 08 011 | Meteorological feature | Set to missing (cancel) |  |
|  |  |  | 0 08 079 | Product status | Set to missing (cancel) |  |
| 3 16 040 | (Cancellation of SIGMET) |  | 3 16 030 | SIGMET header |  |  |
|  |  |  | 0 08 079 | Product status | = 4 Cancellation |  |
|  |  |  | 3 01 014 | Time period | Of the SIGMET to be cancelled |  |
|  |  |  | 0 01 037 | SIGMET sequence identifier | Of the SIGMET to be cancelled |  |
|  |  |  | 0 10 064 | SIGMET cruising level | Of the SIGMET to be cancelled |  |
|  |  |  | 0 08 079 | Product status | Set to missing (cancel) |  |
| 3 16 050 | (RADOB template - Part A: Information on tropical cyclone) |  | 3 01 001 | WMO block and station numbers |  |  |
|  |  |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 3 01 012 | Hour, minute |  |  |
|  |  |  | 0 02 160 | Wavelength of the radar |  |  |
|  |  |  | 0 08 005 | Meteorological attribute significance | = 1 |  |
|  |  |  | 0 05 002 | Latitude (coarse accuracy) |  |  |
|  |  |  | 0 06 002 | Longitude (coarse accuracy) |  |  |
|  |  |  | 0 08 005 | Meteorological attribute significance | Cancel |  |
|  |  |  | 0 19 100 | Time interval to calculate the movement of the tropical cyclone |  |  |
|  |  |  | 0 19 005 | Direction of motion of feature |  |  |
|  |  |  | 0 19 006 | Speed of motion of feature |  |  |
|  |  |  | 0 19 101 | Accuracy of the position of the centre of the tropical cyclone |  |  |
|  |  |  | 0 19 102 | Shape and definition of the eye of the tropical cyclone |  |  |
|  |  |  | 0 19 103 | Diameter of major axis of the eye of the tropical cyclone |  |  |
|  |  |  | 0 19 104 | Change in character of the eye during the 30 minutes |  |  |
|  |  |  | 0 19 105 | Distance between the end of spiral band and the centre |  |  |
| 3 16 052 | (SAREP template - Part A: Information on tropical cyclone) |  | 3 01 005 | Originating centre/sub-centre |  |  |
|  |  |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 3 01 012 | Hour, minute |  |  |
|  |  |  | 0 01 007 | Satellite identifier |  |  |
|  |  |  | 0 25 150 | Method of tropical cyclone intensity analysis using satellite data |  |  |
|  |  |  | 1 22 000 | Delayed replication of 22 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 01 027 | WMO long storm name |  |  |
|  |  |  | 0 19 150 | Typhoon International Common Number (Typhoon Committee) |  |  |
|  |  |  | 0 19 106 | Identification number of tropical cyclone |  |  |
|  |  |  | 0 08 005 | Meteorological attribute significance | = 1 |  |
|  |  |  | 0 05 002 | Latitude (coarse accuracy) |  |  |
|  |  |  | 0 06 002 | Longitude (coarse accuracy) |  |  |
|  |  |  | 0 08 005 | Meteorological attribute significance | Cancel |  |
|  |  |  | 0 19 107 | Time interval over which the movement of the tropical cyclone has been calculated |  |  |
|  |  |  | 0 19 005 | Direction of motion of feature |  |  |
|  |  |  | 0 19 006 | Speed of motion of feature |  |  |
|  |  |  | 0 19 108 | Accuracy of geographical position of the tropical cyclone |  |  |
|  |  |  | 0 19 109 | Mean diameter of the overcast cloud of the tropical cyclone |  |  |
|  |  |  | 0 19 110 | Apparent 24-hour change in intensity of the tropical cyclone |  |  |
|  |  |  | 0 19 111 | Current Intensity (CI) number of the tropical cyclone |  |  |
|  |  |  | 0 19 112 | Data Tropical (DT) number of the tropical cyclone |  |  |
|  |  |  | 0 19 113 | Cloud pattern type of the DT-number |  |  |
|  |  |  | 0 19 114 | Model Expected Tropical (MET) number of the tropical cyclone |  |  |
|  |  |  | 0 19 115 | Trend of the past 24-hour change (+: Developed, -: Weakened) |  |  |
|  |  |  | 0 19 116 | Pattern Tropical (PT) number of the tropical cyclone |  |  |
|  |  |  | 0 19 117 | Cloud picture type of the PT-number |  |  |
|  |  |  | 0 19 118 | Final Tropical (T) number of the tropical cyclone |  |  |
|  |  |  | 0 19 119 | Type of the final T-number |  |  |
| 3 16 060 | (Definition of squall line (by 3 points: Centre, North, South) and forecasted trajectory and evolution) |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 3 01 012 | Hour, minute |  |  |
|  |  | Position of squall line centre | 0 05 002 | Latitude (coarse accuracy) |  |  |
|  |  | Position of squall line centre | 0 06 002 | Longitude (coarse accuracy) |  |  |
|  |  | Position of squall line centre | 0 19 005 | Direction of motion of feature |  |  |
|  |  | Position of squall line centre | 0 19 006 | Speed of motion of feature |  |  |
|  |  | Amplitude of feature from most external points to centre point - North point | 0 05 002 | Latitude (coarse accuracy) |  |  |
|  |  | Amplitude of feature from most external points to centre point - North point | 0 06 002 | Longitude (coarse accuracy) |  |  |
|  |  | Amplitude of feature from most external points to centre point - South point | 0 05 002 | Latitude (coarse accuracy) |  |  |
|  |  | Amplitude of feature from most external points to centre point - South point | 0 06 002 | Longitude (coarse accuracy) |  |  |
|  |  | Amplitude of feature from most external points to centre point - Evolution | 0 04 074 | Short time period or displacement | Period of validity |  |
|  |  | Amplitude of feature from most external points to centre point - Evolution | 0 20 048 | Evolution of feature |  |  |
|  |  | Amplitude of feature from most external points to centre point - Evolution | 0 11 041 | Maximum wind gust speed | Maximum burst expected |  |
|  |  | Amplitude of feature from most external points to centre point - Evolution | 0 13 055 | Intensity of precipitation | Intensity of rain expected |  |
| 3 16 061 | (Definition of squall line (by centre and several points: North points and South points) and forecasted trajectory and evolution) |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 3 01 012 | Hour, minute |  |  |
|  |  | Position of squall line centre | 0 05 002 | Latitude (coarse accuracy) |  |  |
|  |  | Position of squall line centre | 0 06 002 | Longitude (coarse accuracy) |  |  |
|  |  | Position of squall line centre | 0 19 005 | Direction of motion of feature |  |  |
|  |  | Position of squall line centre | 0 19 006 | Speed of motion of feature |  |  |
|  |  | Amplitude of feature from most external points to centre point - North points | 1 02 000 | Delayed replication of 2 descriptors |  |  |
|  |  | Amplitude of feature from most external points to centre point - North points | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  | Amplitude of feature from most external points to centre point - North points | 0 05 002 | Latitude (coarse accuracy) |  |  |
|  |  | Amplitude of feature from most external points to centre point - North points | 0 06 002 | Longitude (coarse accuracy) |  |  |
|  |  | Amplitude of feature from most external points to centre point - South points | 1 02 000 | Delayed replication of 2 descriptors |  |  |
|  |  | Amplitude of feature from most external points to centre point - South points | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  | Amplitude of feature from most external points to centre point - South points | 0 05 002 | Latitude (coarse accuracy) |  |  |
|  |  | Amplitude of feature from most external points to centre point - South points | 0 06 002 | Longitude (coarse accuracy) |  |  |
|  |  | Amplitude of feature from most external points to centre point - Evolution | 0 04 074 | Short time period or displacement | Period of validity |  |
|  |  | Amplitude of feature from most external points to centre point - Evolution | 0 20 048 | Evolution of feature |  |  |
|  |  | Amplitude of feature from most external points to centre point - Evolution | 0 11 041 | Maximum wind gust speed | Maximum burst expected |  |
|  |  | Amplitude of feature from most external points to centre point - Evolution | 0 13 055 | Intensity of precipitation | Intensity of rain expected |  |
| 3 16 071 | (Graphical AIRMET Sierra) |  | 3 01 014 | Time period | For which AIRMET is valid |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 002 | Extended delayed descriptor replication factor |  |  |
|  |  |  | 3 16 075 | GFA IFR ceiling and visibility |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 002 | Extended delayed descriptor replication factor |  |  |
|  |  |  | 3 16 076 | GFA mountain obscuration |  |  |
| 3 16 072 | (Graphical AIRMET Tango) |  | 3 01 014 | Time period | For which AIRMET is valid |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 002 | Extended delayed descriptor replication factor |  |  |
|  |  |  | 3 16 077 | GFA turbulence |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 002 | Extended delayed descriptor replication factor |  |  |
|  |  |  | 3 16 078 | GFA strong surface wind |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 002 | Extended delayed descriptor replication factor |  |  |
|  |  |  | 3 16 079 | GFA low-level wind shear |  |  |
| 3 16 073 | (Graphical AIRMET Zulu) |  | 3 01 014 | Time period | For which AIRMET is valid |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 002 | Extended delayed descriptor replication factor |  |  |
|  |  |  | 3 16 080 | GFA icing |  |  |
|  |  |  | 1 01 000 | Delayed replication of 1 descriptor |  |  |
|  |  |  | 0 31 002 | Extended delayed descriptor replication factor |  |  |
|  |  |  | 3 16 081 | GFA freezing level |  |  |
| 3 16 074 | (GFA identifier and observed/forecast location) |  | 0 01 039 | Graphical Area Forecast (GFA) sequence identifier |  |  |
|  |  |  | 0 08 021 | Time significance | = 4 Forecast, = 16 Analysis |  |
|  |  |  | 3 01 014 | Time period | For which hazard is being observed/forecast |  |
|  |  |  | 3 01 027 | Description of a feature in 3-D or 2-D |  |  |
|  |  |  | 0 08 021 | Time significance | Set to missing (cancel) |  |
| 3 16 075 | (GFA IFR ceiling and visibility) |  | 0 08 079 | Product status | = 0 Normal, = 1 COR, = 2 AMD, = 3 COR AMD, = 4 CNL |  |
|  |  |  | 0 08 041 | Data significance | = 8 IFR ceiling and visibility |  |
|  |  |  | 3 16 074 | GFA identifier and observed/forecast location |  |  |
|  |  |  | 0 20 006 | Flight rules | = 1 IFR |  |
|  |  |  | 0 33 042 | Type of limit represented by following value | = 2 Exclusive upper limit, = 7 Missing |  |
|  |  |  | 0 20 013 | Height of base of cloud |  |  |
|  |  |  | 0 33 042 | Type of limit represented by following value | = 2 Exclusive upper limit, = 7 Missing |  |
|  |  |  | 0 20 001 | Horizontal visibility |  |  |
|  |  |  | 0 20 025 | Obscuration |  |  |
|  |  |  | 0 20 026 | Character of obscuration | = 6 Blowing, = 15 Missing |  |
|  |  |  | 0 08 041 | Data significance | Set to missing (cancel) |  |
|  |  |  | 0 08 079 | Product status | Set to missing (cancel) |  |
| 3 16 076 | (GFA mountain obscuration) |  | 0 08 079 | Product status | = 0 Normal, = 1 COR, = 2 AMD, = 3 COR AMD, = 4 CNL |  |
|  |  |  | 0 08 041 | Data significance | = 9 Mountain obscuration |  |
|  |  |  | 3 16 074 | GFA identifier and observed/forecast location |  |  |
|  |  |  | 0 20 006 | Flight rules | = 1 IFR |  |
|  |  |  | 0 20 025 | Obscuration |  |  |
|  |  |  | 0 20 026 | Character of obscuration | = 6 Blowing, = 15 Missing |  |
|  |  |  | 0 08 041 | Data significance | Set to missing (cancel) |  |
|  |  |  | 0 08 079 | Product status | Set to missing (cancel) |  |
| 3 16 077 | (GFA turbulence) |  | 0 08 079 | Product status | = 0 Normal, = 1 COR, = 2 AMD, = 3 COR AMD, = 4 CNL |  |
|  |  |  | 0 08 011 | Meteorological feature | = 13 Turbulence |  |
|  |  |  | 3 16 074 | GFA identifier and observed/forecast location |  |  |
|  |  |  | 0 11 031 | Degree of turbulence | = 6 Moderate |  |
|  |  |  | 0 08 011 | Meteorological feature | Set to missing (cancel) |  |
|  |  |  | 0 08 079 | Product status | Set to missing (cancel) |  |
| 3 16 078 | (GFA strong surface wind) |  | 0 08 079 | Product status | = 0 Normal, = 1 COR, = 2 AMD, = 3 COR AMD, = 4 CNL |  |
|  |  |  | 0 08 041 | Data significance | = 10 Strong surface wind |  |
|  |  |  | 3 16 074 | GFA identifier and observed/forecast location |  |  |
|  |  |  | 0 33 042 | Type of limit represented by following value | = 0 Exclusive lower limit |  |
|  |  |  | 0 11 012 | Wind speed at 10 m |  |  |
|  |  |  | 0 08 041 | Data significance | Set to missing (cancel) |  |
|  |  |  | 0 08 079 | Product status | Set to missing (cancel) |  |
| 3 16 079 | (GFA low-level wind shear) |  | 0 08 079 | Product status | = 0 Normal, = 1 COR, = 2 AMD, = 3 COR AMD, = 4 CNL |  |
|  |  |  | 0 08 011 | Meteorological feature | = 16 Phenomenon |  |
|  |  |  | 3 16 074 | GFA identifier and observed/forecast location |  |  |
|  |  |  | 0 20 023 | Other weather phenomena | Bit 12 = Wind shear |  |
|  |  |  | 0 20 024 | Intensity of phenomena |  |  |
|  |  |  | 0 08 011 | Meteorological feature | Set to missing (cancel) |  |
|  |  |  | 0 08 079 | Product status | Set to missing (cancel) |  |
| 3 16 080 | (GFA icing) |  | 0 08 079 | Product status | = 0 Normal, = 1 COR, = 2 AMD, = 3 COR AMD, = 4 CNL |  |
|  |  |  | 0 08 011 | Meteorological feature | = 15 Airframe icing |  |
|  |  |  | 3 16 074 | GFA identifier and observed/forecast location |  |  |
|  |  |  | 0 20 041 | Airframe icing | = 4 Moderate icing |  |
|  |  |  | 0 08 011 | Meteorological feature | Set to missing (cancel) |  |
|  |  |  | 0 08 079 | Product status | Set to missing (cancel) |  |
| 3 16 081 | (GFA freezing level) |  | 0 08 079 | Product status | = 0 Normal, = 1 COR, = 2 AMD, = 3 COR AMD, = 4 CNL |  |
|  |  |  | 0 08 041 | Data significance | = 11 Freezing level, = 12 Multiple freezing level |  |
|  |  |  | 3 16 074 | GFA identifier and observed/forecast location |  |  |
|  |  |  | 0 08 041 | Data significance | Set to missing (cancel) |  |
|  |  |  | 0 08 079 | Product status | Set to missing (cancel) |  |
| 3 16 082 | (Tropical cyclone track and wind radii) |  | 0 01 033 | Identification of originating/generating centre |  |  |
|  |  |  | 0 01 034 | Identification of originating/generating sub-centre |  |  |
|  |  |  | 0 01 032 | Generating application |  |  |
|  |  |  | 0 01 025 | Storm identifier |  |  |
|  |  |  | 0 01 027 | WMO long storm name |  |  |
|  |  |  | 0 01 090 | Technique for making up initial perturbations |  |  |
|  |  |  | 0 01 091 | Ensemble member number |  |  |
|  |  |  | 0 01 092 | Type of ensemble forecast |  |  |
|  |  |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 3 01 012 | Hour, minute |  |  |
|  |  |  | 0 08 005 | Meteorological attribute significance |  |  |
|  |  |  | 3 01 023 | Latitude/longitude (coarse accuracy) |  |  |
|  |  |  | 0 08 005 | Meteorological attribute significance |  |  |
|  |  |  | 3 01 023 | Latitude/longitude (coarse accuracy) |  |  |
|  |  |  | 0 10 051 | Pressure reduced to mean sea level |  |  |
|  |  |  | 0 08 005 | Meteorological attribute significance |  |  |
|  |  |  | 3 01 023 | Latitude/longitude (coarse accuracy) |  |  |
|  |  |  | 0 11 012 | Wind speed at 10 m |  |  |
|  |  |  | 1 07 003 | Delayed replication of 7 descriptors |  |  |
|  |  |  | 0 19 003 | Wind speed threshold |  |  |
|  |  |  | 1 05 004 | Delayed replication of 5 descriptors |  |  |
|  |  |  | 0 05 021 | Bearing or azimuth |  |  |
|  |  |  | 0 05 021 | Bearing or azimuth |  |  |
|  |  |  | 2 01 131 | Change data width |  |  |
|  |  |  | 0 19 004 | Effective radius with respect to wind speeds above threshold |  |  |
|  |  |  | 2 01 000 | Change data width | Set to missing (cancel) |  |
|  |  |  | 1 16 000 | Delayed replication of 16 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 08 021 | Time significance |  |  |
|  |  |  | 0 04 024 | Time period or displacement |  |  |
|  |  |  | 0 08 005 | Meteorological attribute significance |  |  |
|  |  |  | 3 01 023 | Latitude/longitude (coarse accuracy) |  |  |
|  |  |  | 0 10 051 | Pressure reduced to mean sea level |  |  |
|  |  |  | 0 08 005 | Meteorological attribute significance |  |  |
|  |  |  | 3 01 023 | Latitude/longitude (coarse accuracy) |  |  |
|  |  |  | 0 11 012 | Wind speed at 10 m |  |  |
|  |  |  | 1 07 003 | Delayed replication of 7 descriptors |  |  |
|  |  |  | 0 19 003 | Wind speed threshold |  |  |
|  |  |  | 1 05 004 | Delayed replication of 5 descriptors |  |  |
|  |  |  | 0 05 021 | Bearing or azimuth |  |  |
|  |  |  | 0 05 021 | Bearing or azimuth |  |  |
|  |  |  | 2 01 131 | Change data width |  |  |
|  |  |  | 0 19 004 | Effective radius with respect to wind speeds above threshold |  |  |
|  |  |  | 2 01 000 | Change data width | Set to missing (cancel) |  |
| 3 16 083 | (Tropical cyclone track, wind radii and vortex) |  | 0 01 033 | Identification of originating/generating centre |  |  |
|  |  |  | 0 01 034 | Identification of originating/generating sub-centre |  |  |
|  |  |  | 0 01 032 | Generating application |  |  |
|  |  |  | 0 01 130 | Storm identifier (long) |  |  |
|  |  |  | 0 01 027 | WMO long storm name |  |  |
|  |  |  | 0 01 030 | Numerical model identifier |  |  |
|  |  |  | 0 01 090 | Technique for making up initial perturbations |  |  |
|  |  |  | 0 01 091 | Ensemble member number |  |  |
|  |  |  | 0 01 092 | Type of ensemble forecast |  |  |
|  |  |  | 3 01 011 | Year, month, day |  |  |
|  |  |  | 3 01 012 | Hour, minute |  |  |
|  |  |  | 0 08 005 | Meteorological attribute significance |  |  |
|  |  |  | 3 01 023 | Latitude/longitude (coarse accuracy) |  |  |
|  |  |  | 0 08 005 | Meteorological attribute significance |  |  |
|  |  |  | 3 01 023 | Latitude/longitude (coarse accuracy) |  |  |
|  |  |  | 0 10 051 | Pressure reduced to mean sea level |  |  |
|  |  |  | 0 08 005 | Meteorological attribute significance |  |  |
|  |  |  | 3 01 023 | Latitude/longitude (coarse accuracy) |  |  |
|  |  |  | 0 11 012 | Wind speed at 10 m |  |  |
|  |  |  | 1 07 004 | Replicate 7 descriptors 4 times |  |  |
|  |  |  | 0 19 003 | Wind speed threshold |  |  |
|  |  |  | 1 05 004 | Replicate 5 descriptors 4 times |  |  |
|  |  |  | 0 05 021 | Bearing or azimuth |  |  |
|  |  |  | 0 05 021 | Bearing or azimuth |  |  |
|  |  |  | 2 01 131 | Change data width |  |  |
|  |  |  | 0 19 004 | Effective radius with respect to wind speeds above threshold |  |  |
|  |  |  | 2 01 000 | Change data width | Set to missing (cancel) |  |
|  |  |  | 1 16 000 | Delayed replication of 16 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 08 021 | Time significance |  |  |
|  |  |  | 0 04 024 | Time period or displacement |  |  |
|  |  |  | 0 08 005 | Meteorological attribute significance |  |  |
|  |  |  | 3 01 023 | Latitude/longitude (coarse accuracy) |  |  |
|  |  |  | 0 10 051 | Pressure reduced to mean sea level |  |  |
|  |  |  | 0 08 005 | Meteorological attribute significance |  |  |
|  |  |  | 3 01 023 | Latitude/longitude (coarse accuracy) |  |  |
|  |  |  | 0 11 012 | Wind speed at 10 m |  |  |
|  |  |  | 1 07 004 | Replicate 7 descriptors 4 times |  |  |
|  |  |  | 0 19 003 | Wind speed threshold |  |  |
|  |  |  | 1 05 004 | Replicate 5 descriptors 4 times |  |  |
|  |  |  | 0 05 021 | Bearing or azimuth |  |  |
|  |  |  | 0 05 021 | Bearing or azimuth |  |  |
|  |  |  | 2 01 131 | Change data width |  |  |
|  |  |  | 0 19 004 | Effective radius with respect to wind speeds above threshold |  |  |
|  |  |  | 2 01 000 | Change data width | Set to missing (cancel) |  |
|  |  |  | 1 04 000 | Delayed replication of 4 descriptors |  |  |
|  |  |  | 0 31 001 | Delayed descriptor replication factor |  |  |
|  |  |  | 0 08 055 | Method used to calculate the average wind components after removing vortex |  |  |
|  |  |  | 0 07 004 | Pressure |  |  |
|  |  |  | 0 11 024 | Average u-component after the vortex removal |  |  |
|  |  |  | 0 11 025 | Average v-component after the vortex removal |  |  |