Class 31: Data description operator qualifiers

| FXY | ELEMENT NAME | BUFR UNIT | SCALE | REF VALUE | BITS | CREX UNIT | SCALE | CHARS | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 31 000 | Short delayed descriptor replication factor | Numeric | 0 | 0 | 1 |  |  |  |  |
| 0 31 001 | Delayed descriptor replication factor | Numeric | 0 | 0 | 8 |  |  |  |  |
| 0 31 002 | Extended delayed descriptor replication factor | Numeric | 0 | 0 | 16 |  |  |  |  |
| 0 31 011 | Delayed descriptor and data repetition factor | Numeric | 0 | 0 | 8 |  |  |  | Note B127: The "delayed descriptor and data repetition factor" is intended for run-length encoding (e.g. scanning an image). It specifies a count N which applies to both descriptor and data, i.e. the value of the single element defined by the following descriptor is repeated N times (at intervals already specified). |
| 0 31 012 | Extended delayed descriptor and data repetition factor | Numeric | 0 | 0 | 16 |  |  |  |  |
| 0 31 021 | Associated field significance | Code table | 0 | 0 | 6 |  |  |  |  |
| 0 31 031 | Data present indicator | Flag table | 0 | 0 | 1 |  |  |  | Note B59: Descriptor 0 31 031, used in conjunction with quality control or statistics operators 2 22 YYY through 2 32 YYY, shall indicate the presence of quality control information when the indicator value is set to zero. It may be used in conjunction with the replication operator 1 01 YYY to construct a table of data present/not present indicators, forming a data present bit-map as defined in Regulation 94.5.5.3. This makes it possible to present quality control information and statistical information for selected data corresponding to element descriptors which precede the 2 22 YYY to 2 32 YYY operators. ¦ Note B113: Other applications of the data present indicator may be developed. |