Class 40: Satellite data

| FXY | ELEMENT NAME | BUFR UNIT | SCALE | REF VALUE | BITS | CREX UNIT | SCALE | CHARS | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 40 001 | Surface soil moisture (ms) | % | 1 | 0 | 10 | % | 1 | 4 |  |
| 0 40 002 | Estimated error in surface soil moisture | % | 1 | 0 | 10 | % | 1 | 4 |  |
| 0 40 003 | Mean surface soil moisture | Numeric | 3 | 0 | 10 | Numeric | 3 | 4 |  |
| 0 40 004 | Rain fall detection | Numeric | 3 | 0 | 10 | Numeric | 3 | 4 |  |
| 0 40 005 | Soil moisture correction flag | Flag table | 0 | 0 | 8 | Flag table | 0 | 3 |  |
| 0 40 006 | Soil moisture processing flag | Flag table | 0 | 0 | 16 | Flag table | 0 | 6 |  |
| 0 40 007 | Soil moisture quality | % | 1 | 0 | 10 | % | 1 | 4 |  |
| 0 40 008 | Frozen land surface fraction | % | 1 | 0 | 10 | % | 1 | 4 |  |
| 0 40 009 | Inundation and wetland fraction | % | 1 | 0 | 10 | % | 1 | 4 |  |
| 0 40 010 | Topographic complexity | % | 1 | 0 | 10 | % | 1 | 4 |  |
| 0 40 011 | Interpolation flag | Flag table | 0 | 0 | 8 | Flag table | 0 | 3 |  |
| 0 40 012 | Radiometer data quality flag | Flag table | 0 | 0 | 8 | Flag table | 0 | 3 |  |
| 0 40 013 | Radiometer brightness temperature interpretation flag | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 40 014 | High-frequency fluctuations of the sea-surface topography correction | m | 4 | -3000 | 13 | m | 4 | 4 |  |
| 0 40 015 | Normalized differential vegetation index (NDVI) | Numeric | 2 | -100 | 8 | Numeric | 2 | 3 |  |
| 0 40 016 | Residual RMS in band | Numeric | 3 | 0 | 14 | Numeric | 3 | 5 |  |
| 0 40 017 | Non-normalized principal component score | Numeric | 0 | -1073741824 | 31 | Numeric | 0 | 10 |  |
| 0 40 018 | GIacAvgImagIIS - average of imager measurements | W m-2 sr-1 m | 6 | 0 | 24 | W m-2 sr-1 m | 6 | 8 |  |
| 0 40 019 | GIacVarImagIIS - variance of imager measurements | W m-2 sr-1 m | 6 | 0 | 24 | W m-2 sr-1 m | 6 | 8 |  |
| 0 40 020 | GqisFlagQualDetailed - quality flag for the system | Flag table | 0 | 0 | 17 | Flag table | 0 | 6 |  |
| 0 40 021 | Fraction of weighted AVHRR pixel in IASI FOV covered with snow/ice | % | 0 | 0 | 7 | % | 0 | 3 |  |
| 0 40 022 | Number of missing, bad or failed AVHRR pixels | Numeric | 0 | 0 | 7 | Numeric | 0 | 3 |  |
| 0 40 023 | Auxiliary altimeter state flags | Flag table | 0 | 0 | 5 | Flag table | 0 | 2 |  |
| 0 40 024 | Meteorological map availability | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 40 025 | Interpolation flag for mean diurnal tide | Code table | 0 | 0 | 2 | Code table | 0 | 1 |  |
| 0 40 026 | Score quantization factor | Numeric | 2 | 0 | 16 | Numeric | 2 | 5 |  |
| 0 40 027 | Sun glint angle | deg | 2 | -18000 | 16 | deg | 2 | 5 |  |
| 0 40 028 | GMI quality flag | Code table | 0 | 0 | 4 | Code table | 0 | 2 |  |
| 0 40 029 | Horizontal observation integration length | m | 0 | 0 | 26 | m | 0 | 8 |  |
| 0 40 030 | Horizontal line of sight wind | m/s | 2 | -32767 | 16 | m/s | 2 | 5 |  |
| 0 40 031 | Error estimate of horizontal line of sight wind | m/s | 2 | 0 | 15 | m/s | 2 | 5 |  |
| 0 40 032 | Derivative wind to pressure | m s-1 Pa-1 | 3 | -100000 | 18 | m s-1 Pa-1 | 3 | 6 |  |
| 0 40 033 | Derivative wind to temperature | m s-1 K-1 | 3 | -100000 | 18 | m s-1 K-1 | 3 | 6 |  |
| 0 40 034 | Derivative wind to backscatter ratio | m/s | 3 | -200000 | 19 | m/s | 3 | 6 |  |
| 0 40 035 | Satellite range | m | 0 | 380000 | 18 | m | 0 | 6 |  |
| 0 40 036 | Lidar L2b classification type | Code table | 0 | 0 | 4 | Code table | 0 | 2 |  |
| 0 40 037 | Backscatter ratio | Numeric | 3 | 500 | 20 | Numeric | 3 | 7 |  |
| 0 40 038 | Cloud particle size | m | 7 | 0 | 28 | m | 7 | 6 |  |
| 0 40 039 | Single look complex image intensity | Numeric | 0 | -25 | 5 | Numeric | 0 | 3 |  |
| 0 40 040 | Single look complex image skewness | Numeric | 2 | 1 | 13 | Numeric | 0 | 4 |  |
| 0 40 041 | Single look complex image kurtosis | Numeric | 2 | 1 | 13 | Numeric | 0 | 4 |  |
| 0 40 042 | Single look complex image variance | Numeric | 2 | 1 | 13 | Numeric | 0 | 4 |  |
| 0 40 043 | Satellite manoeuvre indicator | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 40 044 | Dust index | Numeric | 1 | 0 | 8 | Numeric | 1 | 3 |  |
| 0 40 045 | Cloud formation and height assignment | Flag table | 0 | 0 | 5 | Flag table | 0 | 2 |  |
| 0 40 046 | Cloudiness summary | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 40 047 | Validation flag for IASI or IASI-NG level 1 product | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 40 048 | Validation flag of AMSU-A level 1 data flow | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 40 049 | Cloud tests executed and results | Flag table | 0 | 0 | 16 | Flag table | 0 | 5 |  |
| 0 40 050 | Retrieval initialisation | Flag table | 0 | 0 | 8 | Flag table | 0 | 3 |  |
| 0 40 051 | Convergence of the iterative retrieval | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 40 052 | Indication of super-adiabatic and super-saturation in final retrieval | Flag table | 0 | 0 | 8 | Flag table | 0 | 3 |  |
| 0 40 053 | Number of iterations used for retrieval | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 |  |
| 0 40 054 | Potential processing and inputs errors | Flag table | 0 | 0 | 13 | Flag table | 0 | 4 |  |
| 0 40 055 | Diagnostics on the retrieval | Flag table | 0 | 0 | 21 | Flag table | 0 | 7 |  |
| 0 40 056 | General retrieval quality | Code table | 0 | 0 | 3 | Code table | 0 | 1 |  |
| 0 40 057 | IASI level 2 retrieval flags | Flag table | 0 | 0 | 31 | Flag table | 0 | 10 |  |
| 0 40 058 | Number of vectors describing the characterization matrices | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 |  |
| 0 40 059 | Number of layers actually retrieved | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 |  |
| 0 40 060 | Number of profiles retrieved in scanline | Numeric | 0 | 0 | 8 | Numeric | 0 | 3 |  |
| 0 40 061 | Air partial columns on each retrieved layer | mol cm-2 | 3 | 0 | 16 | mol cm-2 | 3 | 5 |  |
| 0 40 062 | A priori partial columns on each retrieved layer | mol cm-2 | 10 | 0 | 16 | mol cm-2 | 10 | 5 |  |
| 0 40 063 | Scaling vector multiplying the a priori vector in order to define the retrieved vector | Numeric | 5 | 0 | 26 | Numeric | 5 | 8 |  |
| 0 40 064 | Main eigenvalues of the sensitivity matrix | Numeric | 6 | 0 | 31 | Numeric | 6 | 10 |  |
| 0 40 065 | Main eigenvectors of the sensitivity matrix | Numeric | 6 | -1000000000 | 31 | Numeric | 6 | 10 |  |
| 0 40 066 | Quality indicator for atmospheric water vapour | Numeric | 1 | 0 | 8 | Numeric | 1 | 3 |  |
| 0 40 067 | Quality indicator for atmospheric temperature | Numeric | 1 | 0 | 8 | Numeric | 1 | 3 |  |
| 0 40 068 | General retrieval quality flag for SO2 | Code table | 0 | 0 | 4 | Code table | 0 | 2 |  |
| 0 40 069 | PWLR estimated retrieval error for surface air temperature | K | 4 | -1000000 | 21 | K | 4 | 7 | Note B114: Piece-wise linear regression cube is a first-guess optimal estimation method. |
| 0 40 070 | PWLR estimated retrieval error of surface dewpoint | K | 4 | -1000000 | 21 | K | 4 | 7 |  |
| 0 40 071 | Retrieval error covariance matrix for ozone in principal component domain | Numeric | 4 | -1000000 | 21 | Numeric | 4 | 7 |  |
| 0 40 072 | PWLR estimated retrieval quality indicator of atmospheric ozone | Numeric | 1 | 0 | 8 | Numeric | 1 | 3 |  |
| 0 40 073 | PWLR estimated retrieval error of surface skin temperature | K | 1 | 0 | 8 | K | 1 | 3 |  |
| 0 40 074 | General interferometry quality flags | Flag table | 0 | 0 | 16 |  |  |  |  |
| 0 40 075 | Retrieval error covariance matrix for humidity in principal component domain | Numeric | 0 | 0 | 21 | Numeric | 0 | 7 |  |
| 0 40 076 | Retrieval error covariance matrix for temperature in principal component domain | Numeric | 0 | 0 | 21 | Numeric | 0 | 7 |  |
| 0 40 077 | Ocean product quality flag | Flag table | 0 | 0 | 8 | Flag table | 0 | 3 |  |
| 0 40 080 | Cloud test results for IASI-NG level 2 | Flag table | 0 | 0 | 5 | Flag table | 0 | 2 |  |
| 0 40 081 | OLCI Level 1 classification and quality | Flag table | 0 | 0 | 31 |  |  |  |  |