Category 02: Meteorological sequences common to surface data

3 02 001 (Pressure and 3-hour pressure change)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 10 004 | Pressure | Station level |  |
| 0 10 051 | Pressure reduced to mean sea level |  |  |
| 0 10 061 | 3-hour pressure change |  |  |
| 0 10 063 | Characteristic of pressure tendency |  |  |

3 02 002 (High altitude station)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 10 004 | Pressure | Station level |  |
| 0 07 004 | Pressure | Pressure level |  |
| 0 10 003 | Geopotential | Pressure level |  |
| 0 10 061 | 3-hour pressure change |  |  |
| 0 10 063 | Characteristic of pressure tendency |  |  |

3 02 003 (Wind, temperature, humidity, visibility, weather)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 11 011 | Wind direction at 10 m |  |  |
| 0 11 012 | Wind speed at 10 m |  |  |
| 0 12 004 | Air temperature at 2 m |  |  |
| 0 12 006 | Dewpoint temperature at 2 m |  |  |
| 0 13 003 | Relative humidity |  |  |
| 0 20 001 | Horizontal visibility |  |  |
| 0 20 003 | Present weather |  |  |
| 0 20 004 | Past weather (1) |  |  |
| 0 20 005 | Past weather (2) |  |  |

3 02 004 (General cloud information)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 20 010 | Cloud cover (total) |  |  |
| 0 08 002 | Vertical significance (surface observations) |  |  |
| 0 20 011 | Cloud amount |  |  |
| 0 20 013 | Height of base of cloud |  |  |
| 0 20 012 | Cloud type |  |  |
| 0 20 012 | Cloud type |  |  |
| 0 20 012 | Cloud type |  |  |

3 02 005 (Cloud layer)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 08 002 | Vertical significance (surface observations) |  |  |
| 0 20 011 | Cloud amount |  |  |
| 0 20 012 | Cloud type |  |  |
| 0 20 013 | Height of base of cloud |  |  |

3 02 006 (Pressure and 24-hour pressure change)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 10 004 | Pressure | Station level |  |
| 0 10 051 | Pressure reduced to mean sea level |  |  |
| 0 10 062 | 24-hour pressure change |  |  |
| 0 10 063 | Characteristic of pressure tendency |  |  |

3 02 011 (Low altitude station)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 02 001 | Pressure and 3-hour pressure change |  |  |
| 3 02 003 | Wind, temperature, humidity, visibility, weather |  |  |
| 3 02 004 | General cloud information | Significant cloud layer |  |

3 02 012 (High altitude station)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 02 002 | High altitude station | Pressure and pressure change |  |
| 3 02 003 | Wind, temperature, humidity, visibility, weather |  |  |
| 3 02 004 | General cloud information | Significant cloud layer |  |

3 02 013 (Basic surface report)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 02 006 | Pressure and 24-hour pressure change |  |  |
| 3 02 003 | Wind, temperature, humidity, visibility, weather |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 02 005 | Cloud layer |  |  |

3 02 021 (Waves)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 22 001 | Direction of waves |  |  |
| 0 22 011 | Period of waves |  |  |
| 0 22 021 | Height of waves |  |  |

3 02 022 (Wind waves)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 22 002 | Direction of wind waves |  |  |
| 0 22 012 | Period of wind waves |  |  |
| 0 22 022 | Height of wind waves |  |  |

3 02 023 (Swell waves)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 22 003 | Direction of swell waves |  |  |
| 0 22 013 | Period of swell waves |  |  |
| 0 22 023 | Height of swell waves |  |  |

3 02 024 (Wind and swell waves)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 02 022 | Wind waves |  |  |
| 1 01 002 | Replicate 1 descriptor 2 times |  |  |
| 3 02 023 | Swell waves | 2 systems of swell |  |

3 02 031 (Pressure information)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 02 001 | Pressure and 3-hour pressure change |  |  |
| 0 10 062 | 24-hour pressure change |  |  |
| 0 07 004 | Pressure | Standard level |  |
| 0 10 009 | Geopotential height |  |  |

3 02 032 (Temperature and humidity data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Temperature and humidity measurement |  |
| 0 12 101 | Temperature/air temperature | Scale: 2 |  |
| 0 12 103 | Dewpoint temperature | Scale: 2 |  |
| 0 13 003 | Relative humidity |  |  |

3 02 033 (Visibility data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Visibility measurement |  |
| 0 20 001 | Horizontal visibility |  |  |

3 02 034 (Precipitation past 24 hours)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Precipitation measurement |  |
| 0 13 023 | Total precipitation past 24 hours |  |  |

3 02 035 (Basic synoptic "instantaneous" data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 02 032 | Temperature and humidity data |  |  |
| 3 02 033 | Visibility data |  |  |
| 3 02 034 | Precipitation past 24 hours |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 3 02 004 | General cloud information |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 02 005 | Cloud layer | Individual cloud layer or mass |  |

3 02 036 (Clouds with bases below station level)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 1 05 000 | Delayed replication of 5 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 08 002 | Vertical significance (surface observations) |  |  |
| 0 20 011 | Cloud amount |  |  |
| 0 20 012 | Cloud type |  |  |
| 0 20 014 | Height of top of cloud |  |  |
| 0 20 017 | Cloud top description |  |  |

3 02 037 (State of ground, snow depth, ground minimum temperature)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 20 062 | State of the ground (with or without snow) |  |  |
| 0 13 013 | Total snow depth |  |  |
| 0 12 113 | Ground minimum temperature, past 12 hours | Scale: 2 |  |

3 02 038 (Present and past weather)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 20 003 | Present weather |  |  |
| 0 04 024 | Time period or displacement | Hours |  |
| 0 20 004 | Past weather (1) |  |  |
| 0 20 005 | Past weather (2) |  |  |

3 02 039 (Sunshine data (from 1 hour and 24 hour period))

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 04 024 | Time period or displacement | Hours |  |
| 0 14 031 | Total sunshine |  |  |

3 02 040 (Precipitation measurement)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Precipitation measurement |  |
| 1 02 002 | Replicate 2 descriptors 2 times |  |  |
| 0 04 024 | Time period or displacement | Hours |  |
| 0 13 011 | Total precipitation/total water equivalent |  |  |

3 02 041 (Extreme temperature data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Temperature measurement |  |
| 0 04 024 | Time period or displacement |  |  |
| 0 04 024 | Time period or displacement |  | Note D78: Within RA IV, the maximum temperature at 1200 UTC is reported for the previous calendar day (i.e. the ending time of the period is not equal to the nominal time of the report). To construct the required time range, descriptor 0 04 024 has to be included two times. If the period ends at the nominal time of the report, value of the second 0 04 024 shall be set to 0 ¦ Note D77: Within RA III, the maximum daytime temperature and the minimum night-time temperature is reported (i.e. the ending time of the period may not be equal to the nominal time of the report). To construct the required time range, descriptor 0 04 024 has to be included two times. If the period ends at the nominal time of the report,value of the second 0 04 024 shall be set to 0. |
| 0 12 111 | Maximum temperature, at height and over period specified | Scale: 2 |  |
| 0 04 024 | Time period or displacement |  |  |
| 0 04 024 | Time period or displacement |  | Note D77: Within RA III, the maximum daytime temperature and the minimum night-time temperature is reported (i.e. the ending time of the period may not be equal to the nominal time of the report). To construct the required time range, descriptor 0 04 024 has to be included two times. If the period ends at the nominal time of the report,value of the second 0 04 024 shall be set to 0. |
| 0 12 112 | Minimum temperature, at height and over period specified | Scale: 2 |  |

3 02 042 (Wind data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Wind measurement |  |
| 0 02 002 | Type of instrumentation for wind measurement |  |  |
| 0 08 021 | Time significance | = 2 Time averaged |  |
| 0 04 025 | Time period or displacement | = -10 minutes, or number of minutes after a significant change of wind |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 08 021 | Time significance | Set to missing |  |
| 1 03 002 | Replicate 3 descriptors 2 times |  |  |
| 0 04 025 | Time period or displacement | Minutes |  |
| 0 11 043 | Maximum wind gust direction |  |  |
| 0 11 041 | Maximum wind gust speed |  |  |

3 02 043 (Basic synoptic "period" data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 02 038 | Present and past weather |  |  |
| 1 01 002 | Replicate 1 descriptor 2 times |  |  |
| 3 02 039 | Sunshine data (from 1 hour and 24 hour period) |  |  |
| 3 02 040 | Precipitation measurement |  |  |
| 3 02 041 | Extreme temperature data |  |  |
| 3 02 042 | Wind data |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |

3 02 044 (Evaporation data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 04 024 | Time period or displacement | Hours |  |
| 0 02 004 | Type of instrumentation for evaporation measurement or type of crop for which evapotranspiration is reported |  |  |
| 0 13 033 | Evaporation/evapotranspiration |  |  |

3 02 045 (Radiation data (from 1 hour and 24 hour period))

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 04 024 | Time period or displacement | Hours |  |
| 0 14 002 | Long-wave radiation, integrated over period specified |  |  |
| 0 14 004 | Short-wave radiation, integrated over period specified |  |  |
| 0 14 016 | Net radiation, integrated over period specified |  |  |
| 0 14 028 | Global solar radiation (high accuracy), integrated over period specified |  |  |
| 0 14 029 | Diffuse solar radiation (high accuracy), integrated over period specified |  |  |
| 0 14 030 | Direct solar radiation (high accuracy), integrated over period specified |  |  |

3 02 046 (Temperature change)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 04 024 | Time period or displacement |  |  |
| 0 04 024 | Time period or displacement |  | Note D73: To construct the required time range, descriptor 0 04 024 has to be included two times. |
| 0 12 049 | Temperature change over specified period |  |  |

3 02 047 (Direction of cloud drift)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 1 02 003 | Replicate 2 descriptors 3 times |  |  |
| 0 08 002 | Vertical significance (surface observations) |  |  |
| 0 20 054 | True direction from which a phenomenon or clouds are moving or in which they are observed |  |  |

3 02 048 (Direction and elevation of cloud)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 05 021 | Bearing or azimuth |  |  |
| 0 07 021 | Elevation | Elevation angle |  |
| 0 20 012 | Cloud type |  |  |
| 0 05 021 | Bearing or azimuth | Set to missing (cancel) |  |
| 0 07 021 | Elevation | Set to missing (cancel) |  |

3 02 049 (Cloud information reported with vertical soundings)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 08 002 | Vertical significance (surface observations) |  |  |
| 0 20 011 | Cloud amount | Low or middle clouds Nh |  |
| 0 20 013 | Height of base of cloud | h |  |
| 0 20 012 | Cloud type | Low clouds CL |  |
| 0 20 012 | Cloud type | Middle clouds CM |  |
| 0 20 012 | Cloud type | High clouds CH |  |
| 0 08 002 | Vertical significance (surface observations) | Set to missing |  |

3 02 050 (Radiosonde surface observation)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 08 041 | Data significance | = 5 Surface observation displacement from launch point |  |
| 0 05 021 | Bearing or azimuth |  |  |
| 0 07 005 | Height increment |  |  |
| 2 02 130 | Change scale |  |  |
| 0 06 021 | Distance |  |  |
| 2 02 000 | Change scale | Cancel |  |
| 0 08 041 | Data significance | = 4 Surface observation |  |
| 2 01 131 | Change data width |  |  |
| 2 02 129 | Change scale |  |  |
| 0 02 115 | Type of surface observing equipment |  |  |
| 0 10 004 | Pressure |  |  |
| 0 02 115 | Type of surface observing equipment |  |  |
| 0 13 003 | Relative humidity |  |  |
| 2 02 000 | Change scale | Cancel |  |
| 2 01 000 | Change data width | Cancel |  |
| 0 02 115 | Type of surface observing equipment |  |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 02 115 | Type of surface observing equipment |  |  |
| 1 02 002 | Replicate 2 descriptors 2 times |  |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 04 024 | Time period or displacement | Hours |  |
| 0 02 115 | Type of surface observing equipment |  |  |
| 0 12 103 | Dewpoint temperature |  |  |
| 0 12 102 | Wet-bulb temperature |  |  |
| 1 01 003 | Replicate 1 descriptor 3 times |  |  |
| 0 20 012 | Cloud type |  |  |
| 0 20 011 | Cloud amount |  |  |
| 0 20 013 | Height of base of cloud |  |  |
| 1 01 002 | Replicate 1 descriptor 2 times |  |  |
| 0 20 003 | Present weather |  |  |

3 02 051

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 10 004 | Pressure |  |  |
| 0 10 051 | Pressure reduced to mean sea level |  |  |
| 0 07 004 | Pressure | Vertical location |  |
| 0 10 003 | Geopotential |  |  |
| 0 12 004 | Air temperature at 2 m |  |  |
| 0 12 051 | Standard deviation temperature |  |  |
| 0 12 016 | Maximum temperature at 2 m, past 24 hours |  |  |
| 0 12 017 | Minimum temperature at 2 m, past 24 hours |  |  |
| 0 13 004 | Vapour pressure |  |  |
| 1 02 004 | Replicate 2 descriptors 4 times |  |  |
| 0 08 051 | Qualifier for number of missing values in calculation of statistic |  |  |
| 0 08 020 | Total number of missing entities (with respect to accumulation or average) |  |  |

3 02 052 (Ship temperature and humidity data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Temperature and humidity measurement |  |
| 0 07 033 | Height of sensor above water surface | Temperature and humidity measurement |  |
| 0 12 101 | Temperature/air temperature | Scale: 2 |  |
| 0 02 039 | Method of wet-bulb temperature measurement |  |  |
| 0 12 102 | Wet-bulb temperature | Scale: 2 |  |
| 0 12 103 | Dewpoint temperature | Scale: 2 |  |
| 0 13 003 | Relative humidity |  |  |

3 02 053 (Ship visibility data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Visibility measurement |  |
| 0 07 033 | Height of sensor above water surface | Visibility measurement |  |
| 0 20 001 | Horizontal visibility |  |  |

3 02 054 (Ship "instantaneous" data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 02 052 | Ship temperature and humidity data |  |  |
| 3 02 053 | Ship visibility data |  |  |
| 0 07 033 | Height of sensor above water surface | Set to missing (cancel) |  |
| 3 02 034 | Precipitation past 24 hours |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 3 02 004 | General cloud information |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 02 005 | Cloud layer |  |  |

3 02 055 (Icing and ice)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 20 031 | Ice deposit (thickness) |  |  |
| 0 20 032 | Rate of ice accretion (estimated) |  |  |
| 0 20 033 | Cause of ice accretion |  |  |
| 0 20 034 | Sea ice concentration |  |  |
| 0 20 035 | Amount and type of ice |  |  |
| 0 20 036 | Ice situation |  |  |
| 0 20 037 | Ice development |  |  |
| 0 20 038 | Bearing of ice edge |  |  |

3 02 056 (Sea/water temperature)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 02 038 | Method of water temperature and/or salinity measurement |  |  |
| 0 07 063 | Depth below sea/water surface (cm) | Sea-surface temperature measurement |  |
| 0 22 043 | Sea/water temperature |  |  |
| 0 07 063 | Depth below sea/water surface (cm) | Set to missing (cancel) |  |

3 02 057 (Ship marine data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 02 056 | Sea/water temperature | Sea-surface temperature, method of measurement, and depth below sea surface |  |
| 3 02 021 | Waves |  |  |
| 3 02 024 | Wind and swell waves |  |  |

3 02 058 (Ship extreme temperature data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Temperature measurement |  |
| 0 07 033 | Height of sensor above water surface | Temperature measurement |  |
| 0 04 024 | Time period or displacement |  |  |
| 0 04 024 | Time period or displacement |  | Note D78: Within RA IV, the maximum temperature at 1200 UTC is reported for the previous calendar day (i.e. the ending time of the period is not equal to the nominal time of the report). To construct the required time range, descriptor 0 04 024 has to be included two times. If the period ends at the nominal time of the report, value of the second 0 04 024 shall be set to 0 ¦ Note D77: Within RA III, the maximum daytime temperature and the minimum night-time temperature is reported (i.e. the ending time of the period may not be equal to the nominal time of the report). To construct the required time range, descriptor 0 04 024 has to be included two times. If the period ends at the nominal time of the report,value of the second 0 04 024 shall be set to 0. |
| 0 12 111 | Maximum temperature, at height and over period specified | Scale: 2 |  |
| 0 04 024 | Time period or displacement |  |  |
| 0 04 024 | Time period or displacement |  | Note D77: Within RA III, the maximum daytime temperature and the minimum night-time temperature is reported (i.e. the ending time of the period may not be equal to the nominal time of the report). To construct the required time range, descriptor 0 04 024 has to be included two times. If the period ends at the nominal time of the report,value of the second 0 04 024 shall be set to 0. |
| 0 12 112 | Minimum temperature, at height and over period specified | Scale: 2 |  |

3 02 059 (Ship wind data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Wind measurement |  |
| 0 07 033 | Height of sensor above water surface | Wind measurement |  |
| 0 02 002 | Type of instrumentation for wind measurement |  |  |
| 0 08 021 | Time significance | = 2 Time averaged |  |
| 0 04 025 | Time period or displacement | = -10 minutes, or number of minutes after a significant change of wind |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 08 021 | Time significance | Set to missing |  |
| 1 03 002 | Replicate 3 descriptors 2 times |  |  |
| 0 04 025 | Time period or displacement | Minutes |  |
| 0 11 043 | Maximum wind gust direction |  |  |
| 0 11 041 | Maximum wind gust speed |  |  |

3 02 060 (Ship "period" data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 02 038 | Present and past weather |  |  |
| 3 02 040 | Precipitation measurement |  |  |
| 3 02 058 | Ship extreme temperature data |  |  |
| 3 02 059 | Ship wind data |  |  |

3 02 062 (Ship "instantaneous" data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 25 188 | Method for reducing pressure to sea level |  |  |
| 3 02 001 | Pressure and 3-hour pressure change |  |  |
| 3 02 093 | Extended ship temperature and humidity data |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 053 | Ship visibility data |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 0 07 033 | Height of sensor above water surface | Set to missing (cancel) |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 004 | General cloud information |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 02 005 | Cloud layer |  |  |
| 0 08 002 | Vertical significance (surface observations) | Set to missing (cancel) |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 055 | Icing and ice |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 056 | Sea/water temperature |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 021 | Waves |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 024 | Wind and swell waves |  |  |

3 02 063 (Ship "period" data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 02 038 | Present and past weather |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 040 | Precipitation measurement |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 034 | Precipitation past 24 hours |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 058 | Ship extreme temperature data |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 064 | Ship or other marine platform wind data |  |  |

3 02 064 (Ship or other marine platform wind data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 07 033 | Height of sensor above water surface |  |  |
| 0 02 002 | Type of instrumentation for wind measurement |  |  |
| 0 08 021 | Time significance | = 2 Time averaged |  |
| 0 04 025 | Time period or displacement |  |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 08 021 | Time significance | Set to missing (cancel) |  |
| 1 03 000 | Delayed replication of 3 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 04 025 | Time period or displacement |  |  |
| 0 11 043 | Maximum wind gust direction |  |  |
| 0 11 041 | Maximum wind gust speed |  |  |

3 02 065 (CFOSAT (Chinese-French Oceanography Satellite) : SWIM off-nadir data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 129 | Observing satellite and instruments |  |  |
| 1 04 000 | Delayed replication of 4 descriptors |  |  |
| 0 31 002 | Extended delayed descriptor replication factor |  |  |
| 0 05 074 | Index in range direction |  |  |
| 2 02 133 | Change scale |  |  |
| 0 06 029 | Wave number |  |  |
| 2 02 000 | Cancel change scale |  |  |
| 1 02 024 | Replicate 2 descriptors 24 times |  |  |
| 0 05 075 | Index in azimuthal direction |  |  |
| 0 25 082 | Azimuth angle |  |  |
| 0 05 092 | Side track index or along lateral track row number |  |  |
| 0 01 124 | Grid point identifier |  |  |
| 0 05 001 | Latitude (high accuracy) |  |  |
| 0 06 001 | Longitude (high accuracy) |  |  |
| 3 01 130 | High precision timestamp |  |  |
| 1 10 000 | Delayed replication of 10 descriptors |  |  |
| 0 31 002 | Extended delayed descriptor replication factor |  |  |
| 0 05 074 | Index in range direction |  |  |
| 1 08 024 | Replicate 8 descriptors 24 times |  |  |
| 0 05 075 | Index in azimuthal direction |  |  |
| 0 08 085 | Beam identifier |  |  |
| 0 22 161 | Wave spectra |  |  |
| 0 08 085 | Beam identifier |  |  |
| 1 02 000 | Delayed replication of 2 descriptors |  |  |
| 0 31 002 | Extended delayed descriptor replication factor |  |  |
| 0 02 134 | Antenna beam azimuth |  |  |
| 0 22 161 | Wave spectra |  |  |

3 02 066 (Dangerous weather phenomena)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 20 023 | Other weather phenomena |  |  |
| 0 20 024 | Intensity of phenomena |  |  |
| 0 20 027 | Phenomena occurrence |  |  |
| 0 20 054 | True direction from which a phenomenon or clouds are moving or in which they are observed |  |  |
| 0 20 023 | Other weather phenomena |  |  |
| 0 20 027 | Phenomena occurrence |  |  |
| 0 20 054 | True direction from which a phenomenon or clouds are moving or in which they are observed |  |  |
| 0 20 025 | Obscuration |  |  |
| 0 20 026 | Character of obscuration |  |  |
| 0 20 027 | Phenomena occurrence |  |  |
| 0 20 040 | Evolution of drift snow |  |  |
| 0 20 066 | Maximum diameter of hailstones |  |  |
| 0 20 027 | Phenomena occurrence |  |  |
| 0 20 021 | Type of precipitation |  |  |
| 0 20 067 | Diameter of deposit |  |  |
| 0 20 027 | Phenomena occurrence |  |  |

3 02 067 (Additional synoptical parameters)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 01 023 | Observation sequence number |  |  |
| 0 04 025 | Time period or displacement | = 0 minutes |  |
| 0 02 177 | Method of snow depth measurement | = 0 Manual observation, = 1 Ultrasonic method, = 2 Video camera method, = 3 Laser method, 4-13 Reserved, = 14 Others, = 15 Missing value |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 20 003 | Present weather | 960ww, 961ww |  |
| 1 03 000 | Delayed replication of 3 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 05 021 | Bearing or azimuth | 981VV-988VV |  |
| 0 20 001 | Horizontal visibility | VV |  |
| 0 05 021 | Bearing or azimuth | Set to missing (cancel) |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 056 | Sea/water temperature | Sea-surface temperature, method of measurement, and depth below sea surface |  |
| 1 03 000 | Delayed replication of 3 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 33 041 | Attribute of following value |  |  |
| 0 20 058 | Visibility seawards from a coastal station | 980VsVs |  |
| 0 22 061 | State of the sea | 924SVs |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 022 | Wind waves |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 02 023 | Swell waves |  |  |
| 1 04 000 | Delayed replication of 4 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 20 054 | True direction from which a phenomenon or clouds are moving or in which they are observed | Da, Dp |  |
| 0 20 137 | Evolution of clouds | 940Cn3 |  |
| 0 20 012 | Cloud type | 941CDp, 943CLDp |  |
| 0 20 090 | Special clouds | 993CsDa |  |
| 1 03 000 | Delayed replication of 3 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 20 054 | True direction from which a phenomenon or clouds are moving or in which they are observed |  |  |
| 0 20 137 | Evolution of clouds |  |  |
| 0 20 136 | Supplementary cloud type | 948C0Da, 949CaDa, 950Nmn3, 951Nvn4 |  |
| 0 04 025 | Time period or displacement | Reference period of fresh fallen snow |  |
| 0 13 012 | Depth of fresh snow |  |  |
| 0 04 025 | Time period or displacement | = -60 minutes |  |
| 0 11 042 | Maximum wind speed (10-minute mean wind) | 912ff 902tt 912ff .. mandatory ff >= 18 |  |
| 1 04 000 | Delayed replication of 4 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 08 021 | Time significance | = 30 Time of occurrence, = 17 Start of phenomenon |  |
| 0 04 025 | Time period or displacement | = -xx ¦ 902tt |  |
| 0 11 042 | Maximum wind speed (10-minute mean wind) | 912ff |  |
| 0 08 021 | Time significance | Set to missing (cancel) |  |
| 1 15 000 | Delayed replication of 15 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 08 021 | Time significance | = 30 Time of occurrence, = 17 Start of phenomenon |  |
| 0 04 015 | Time increment | = -xx1 |  |
| 0 08 021 | Time significance | = 2 Time averaged |  |
| 0 04 025 | Time period or displacement | = -10 minutes, or number of minutes after a significant change of wind |  |
| 0 11 001 | Wind direction | 915dd |  |
| 0 11 002 | Wind speed | 913ff |  |
| 0 08 021 | Time significance | = 22 Time of occurrence of wind shift |  |
| 0 04 015 | Time increment | = +xx2 |  |
| 0 08 021 | Time significance | = 2 Time averaged |  |
| 0 04 025 | Time period or displacement | = -10 minutes, or number of minutes after a significant change of wind |  |
| 0 11 001 | Wind direction | 915dd |  |
| 0 11 002 | Wind speed | 913ff |  |
| 0 08 021 | Time significance | Set to missing (cancel) |  |
| 0 04 025 | Time period or displacement | = 0 minutes |  |
| 0 04 015 | Time increment | = +(xx1-xx2) ¦ Non negative to reset the time to the actual time |  |
| 1 03 000 | Delayed replication of 3 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 04 025 | Time period or displacement | = -xx, i.e. from |  |
| 0 04 025 | Time period or displacement | = -xx, i.e. to |  |
| 0 20 003 | Present weather | 962ww, 963w1w1, 964ww, 965w1w1, 966ww, 967w1w1 |  |
| 1 10 000 | Delayed replication of 10 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 04 025 | Time period or displacement | = -xx, i.e. from |  |
| 0 04 025 | Time period or displacement | = -xx, i.e. to |  |
| 0 05 021 | Bearing or azimuth | Da, Dp |  |
| 0 05 021 | Bearing or azimuth | Da, Dp |  |
| 0 20 054 | True direction from which a phenomenon or clouds are moving or in which they are observed | Da, Dp |  |
| 0 20 024 | Intensity of phenomena | = 1 Light, = 2 Moderate, = 3 Heavy, = 4 Violent, = 5 Severe |  |
| 0 20 025 | Obscuration |  |  |
| 0 20 026 | Character of obscuration |  |  |
| 0 20 027 | Phenomena occurrence |  |  |
| 0 20 063 | Special phenomena |  |  |

3 02 069 (Visibility data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 07 033 | Height of sensor above water surface |  |  |
| 0 33 041 | Attribute of following value |  |  |
| 0 20 001 | Horizontal visibility |  |  |

3 02 070 (Wind data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 07 033 | Height of sensor above water surface |  |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 11 043 | Maximum wind gust direction |  |  |
| 0 11 041 | Maximum wind gust speed |  |  |
| 0 11 016 | Extreme counterclockwise wind direction of a variable wind |  |  |
| 0 11 017 | Extreme clockwise wind direction of a variable wind |  |  |

3 02 071 (Wind data from one-hour period)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 07 033 | Height of sensor above water surface |  |  |
| 0 08 021 | Time significance | = 2 Time averaged |  |
| 0 04 025 | Time period or displacement | = -10 minutes, or number of minutes after a significant change of wind, if any |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 08 021 | Time significance | Set to missing |  |
| 1 03 002 | Replicate 3 descriptors 2 times |  |  |
| 0 04 025 | Time period or displacement | = -10 minutes in the first replication, = -60 minutes in the second replication |  |
| 0 11 043 | Maximum wind gust direction |  |  |
| 0 11 041 | Maximum wind gust speed |  |  |
| 0 04 025 | Time period or displacement | = -10 minutes |  |
| 0 11 016 | Extreme counterclockwise wind direction of a variable wind |  |  |
| 0 11 017 | Extreme clockwise wind direction of a variable wind |  |  |

3 02 072 (Temperature and humidity data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 07 033 | Height of sensor above water surface |  |  |
| 0 12 101 | Temperature/air temperature | Scale: 2 |  |
| 0 12 103 | Dewpoint temperature | Scale: 2 |  |
| 0 13 003 | Relative humidity |  |  |

3 02 073 (Cloud data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 20 010 | Cloud cover (total) |  |  |
| 1 05 004 | Replicate 5 descriptors 4 times |  |  |
| 0 08 002 | Vertical significance (surface observations) |  |  |
| 0 20 011 | Cloud amount |  |  |
| 0 20 012 | Cloud type |  |  |
| 0 33 041 | Attribute of following value |  |  |
| 0 20 013 | Height of base of cloud |  |  |

3 02 074 (Present and past weather)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 20 003 | Present weather |  |  |
| 0 04 025 | Time period or displacement |  |  |
| 0 20 004 | Past weather (1) |  |  |
| 0 20 005 | Past weather (2) |  |  |

3 02 075 (Intensity of precipitation, size of precipitation element)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 08 021 | Time significance | = 2 Time averaged |  |
| 0 04 025 | Time period or displacement | = -10 minutes |  |
| 0 13 055 | Intensity of precipitation |  |  |
| 0 13 058 | Size of precipitating element |  |  |
| 0 08 021 | Time significance | Set to missing |  |

3 02 076 (Precipitation, obscuration and other phenomena)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 20 021 | Type of precipitation |  |  |
| 0 20 022 | Character of precipitation |  |  |
| 0 26 020 | Duration of precipitation |  |  |
| 0 20 023 | Other weather phenomena |  |  |
| 0 20 024 | Intensity of phenomena |  |  |
| 0 20 025 | Obscuration |  |  |
| 0 20 026 | Character of obscuration |  |  |

3 02 077 (Extreme temperature data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 07 033 | Height of sensor above water surface |  |  |
| 0 04 025 | Time period or displacement |  |  |
| 0 12 111 | Maximum temperature, at height and over period specified | Scale: 2 |  |
| 0 12 112 | Minimum temperature, at height and over period specified | Scale: 2 |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Ground temperature |  |
| 0 04 025 | Time period or displacement |  |  |
| 0 12 112 | Minimum temperature, at height and over period specified | Scale: 2 ¦ Ground temperature |  |

3 02 078 (State of ground and snow depth measurement)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 02 176 | Method of state of ground measurement |  |  |
| 0 20 062 | State of the ground (with or without snow) |  |  |
| 0 02 177 | Method of snow depth measurement |  |  |
| 0 13 013 | Total snow depth |  |  |

3 02 079 (Precipitation measurement)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 02 175 | Method of precipitation measurement |  |  |
| 0 02 178 | Method of liquid content measurement of precipitation |  |  |
| 0 04 025 | Time period or displacement |  |  |
| 0 13 011 | Total precipitation/total water equivalent |  |  |

3 02 080 (Evaporation measurement)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 02 185 | Method of evaporation measurement |  |  |
| 0 04 025 | Time period or displacement |  |  |
| 0 13 033 | Evaporation/evapotranspiration |  |  |

3 02 081 (Total sunshine data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 04 025 | Time period or displacement |  |  |
| 0 14 031 | Total sunshine |  |  |

3 02 082 (Radiation data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 04 025 | Time period or displacement |  |  |
| 0 14 002 | Long-wave radiation, integrated over period specified |  |  |
| 0 14 004 | Short-wave radiation, integrated over period specified |  |  |
| 0 14 016 | Net radiation, integrated over period specified |  |  |
| 0 14 028 | Global solar radiation (high accuracy), integrated over period specified |  |  |
| 0 14 029 | Diffuse solar radiation (high accuracy), integrated over period specified |  |  |
| 0 14 030 | Direct solar radiation (high accuracy), integrated over period specified |  |  |

3 02 083 (First-order statistics of P, W, T, U data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 04 025 | Time period or displacement |  |  |
| 0 08 023 | First-order statistics |  |  |
| 0 10 004 | Pressure |  |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 12 101 | Temperature/air temperature | Scale: 2 |  |
| 0 13 003 | Relative humidity |  |  |
| 0 08 023 | First-order statistics | Set to missing |  |

3 02 084 ("Instantaneous" data of sequence 3 07 096)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 02 031 | Pressure information |  |  |
| 3 02 072 | Temperature and humidity data |  |  |
| 1 03 000 | Delayed replication of 3 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 1 01 005 | Replicate 1 descriptor 5 times |  |  |
| 3 07 063 | Depth below land surface and soil temperature |  |  |
| 0 07 061 | Depth below land surface | Set to missing (cancel) |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 069 | Visibility data |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 0 07 033 | Height of sensor above water surface | Set to missing (cancel) |  |
| 1 05 000 | Delayed replication of 5 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 20 031 | Ice deposit (thickness) |  |  |
| 0 20 032 | Rate of ice accretion (estimated) |  |  |
| 0 02 038 | Method of water temperature and/or salinity measurement |  |  |
| 0 22 043 | Sea/water temperature | Scale: 2 |  |
| 3 02 021 | Waves |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 078 | State of ground and snow depth measurement |  |  |
| 0 12 113 | Ground minimum temperature, past 12 hours | Scale: 2 |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 004 | General cloud information |  |  |
| 1 05 000 | Delayed replication of 5 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 08 002 | Vertical significance (surface observations) |  |  |
| 0 20 011 | Cloud amount |  |  |
| 0 20 012 | Cloud type |  |  |
| 0 33 041 | Attribute of following value |  |  |
| 0 20 013 | Height of base of cloud |  |  |
| 3 02 036 | Clouds with bases below station level |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 047 | Direction of cloud drift |  |  |
| 0 08 002 | Vertical significance (surface observations) | Set to missing (cancel) |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 048 | Direction and elevation of cloud |  |  |

3 02 085 ("Period" data of sequence 3 07 096)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 1 05 000 | Delayed replication of 5 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 20 003 | Present weather |  |  |
| 1 03 002 | Replicate 3 descriptors 2 times |  |  |
| 0 04 024 | Time period or displacement | = -1 hour in the first replication, = -x hours in the second replication, x corresponding to the time period of W1W2 in the SYNOP report |  |
| 0 20 004 | Past weather (1) |  |  |
| 0 20 005 | Past weather (2) |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 175 | Intensity of precipitation, size of precipitation element |  |  |
| 1 02 000 | Delayed replication of 2 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 04 025 | Time period or displacement | = -10 minutes |  |
| 3 02 076 | Precipitation, obscuration and other phenomena |  |  |
| 1 02 000 | Delayed replication of 2 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 04 025 | Time period or displacement | = -10 minutes |  |
| 0 13 059 | Number of flashes (thunderstorm) |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 07 033 | Height of sensor above water surface |  |  |
| 0 08 021 | Time significance | = 2 Time averaged |  |
| 0 04 025 | Time period or displacement | = -10 minutes, or number of minutes after a significant change of wind |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 08 021 | Time significance | Set to missing |  |
| 1 03 003 | Replicate 3 descriptors 3 times |  |  |
| 0 04 025 | Time period or displacement | = -10 minutes in the first replication, = -60 minutes in the second replication, = -60x3 or 60x6 minutes in the third replication |  |
| 0 11 043 | Maximum wind gust direction |  |  |
| 0 11 041 | Maximum wind gust speed |  |  |
| 0 04 025 | Time period or displacement | = -10 minutes |  |
| 0 11 016 | Extreme counterclockwise wind direction of a variable wind |  |  |
| 0 11 017 | Extreme clockwise wind direction of a variable wind |  |  |
| 3 02 077 | Extreme temperature data |  |  |
| 0 07 033 | Height of sensor above water surface | Set to missing (cancel) |  |
| 3 02 041 | Extreme temperature data |  |  |
| 1 06 000 | Delayed replication of 6 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 02 175 | Method of precipitation measurement |  |  |
| 0 02 178 | Method of liquid content measurement of precipitation |  |  |
| 1 02 005 | Replicate 2 descriptors 5 times |  |  |
| 0 04 024 | Time period or displacement | = -1 hour in the first replication, = -3, -6, -12 and -24 hours in the other replications |  |
| 0 13 011 | Total precipitation/total water equivalent |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 1 03 000 | Delayed replication of 3 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 02 185 | Method of evaporation measurement |  |  |
| 1 01 002 | Replicate 1 descriptor 2 times |  |  |
| 3 02 044 | Evaporation data |  |  |
| 1 02 000 | Delayed replication of 2 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 1 01 002 | Replicate 1 descriptor 2 times |  |  |
| 3 02 039 | Sunshine data (from 1 hour and 24 hour period) |  |  |
| 1 02 000 | Delayed replication of 2 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 1 01 002 | Replicate 1 descriptor 2 times |  |  |
| 3 02 045 | Radiation data (from 1 hour and 24 hour period) |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 046 | Temperature change |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 3 02 083 | First-order statistics of P, W, T, U data |  |  |

3 02 089 (Locust information)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 20 101 | Locust (acridian) name | Ln |  |
| 0 20 102 | Locust (maturity) colour | Lc |  |
| 0 20 103 | Stage of development of locusts | Ld |  |
| 0 20 104 | Organization state of swarm or band of locusts | Lg |  |
| 0 20 105 | Size of swarm or band of locusts and duration of passage of swarm | sL |  |
| 0 20 106 | Locust population density | dL |  |
| 0 20 107 | Direction of movements of locust swarm | DL |  |
| 0 20 108 | Extent of vegetation | ve |  |

3 02 090 (Sea/water temperature high precision)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 02 038 | Method of water temperature and/or salinity measurement |  |  |
| 0 07 063 | Depth below sea/water surface (cm) | Sea-surface temperature measurement |  |
| 0 22 045 | Sea/water temperature |  |  |

3 02 091 (Sequence for representation of ancillary meteorological observations)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 20 001 | Horizontal visibility |  |  |
| 0 04 024 | Time period or displacement |  |  |
| 0 13 011 | Total precipitation/total water equivalent |  |  |

3 02 092 (VOSClim data elements)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 11 104 | True heading of aircraft, ship or other mobile platform | Ship’s true heading |  |
| 0 01 012 | Direction of motion of moving observing platform | Ship’s course over ground |  |
| 0 01 013 | Speed of motion of moving observing platform | Ship’s speed over ground |  |
| 0 10 038 | Maximum height of deck cargo above summer load line |  |  |
| 0 10 039 | Departure of reference level (summer maximum load line) from actual sea level |  |  |
| 0 11 007 | Relative wind direction (in degrees off bow) |  |  |
| 0 11 008 | Relative wind speed |  |  |

3 02 093 (Extended ship temperature and humidity data)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 07 033 | Height of sensor above water surface |  |  |
| 3 03 099 | Metadata common to temperature/humidity sensors |  |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 1 03 000 | Delayed replication of 3 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor | Set to 0 if no change from previous values, 1 if changes |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) |  |  |
| 0 07 033 | Height of sensor above water surface |  |  |
| 3 03 099 | Metadata common to temperature/humidity sensors |  |  |
| 0 02 039 | Method of wet-bulb temperature measurement |  |  |
| 0 02 097 | Type of humidity sensor |  |  |
| 0 03 024 | Psychrometric coefficient | Set to missing if type of humidity sensor is not psychrometer |  |
| 0 03 021 | Hygrometer heating |  |  |
| 0 12 102 | Wet-bulb temperature |  |  |
| 0 12 103 | Dewpoint temperature |  |  |
| 0 13 003 | Relative humidity |  |  |
| 0 07 032 | Height of sensor above local ground (or deck of marine platform) | Set to missing (cancel) |  |
| 0 07 033 | Height of sensor above water surface | Set to missing (cancel) |  |
| 3 03 099 | Metadata common to temperature/humidity sensors | Set all elements in sequence to missing (cancel) |  |
| 0 02 039 | Method of wet-bulb temperature measurement | Set to missing (cancel) |  |
| 0 02 097 | Type of humidity sensor | Set to missing (cancel) |  |
| 0 03 021 | Hygrometer heating | Set to missing (cancel) |  |
| 0 03 024 | Psychrometric coefficient | Set to missing (cancel) |  |

3 02 175 (Intensity of precipitation, size of precipitation element)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 08 021 | Time significance |  |  |
| 0 04 025 | Time period or displacement |  |  |
| 0 13 155 | Intensity of precipitation (high accuracy) |  |  |
| 0 13 058 | Size of precipitating element |  |  |
| 0 08 021 | Time significance |  |  |