Category 11: Single level report sequences (conventional data)

3 11 001 (Aircraft reports)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 051 | Flight number, navigational system, date/time, location, phase of flight | ASDAR |  |
| 0 07 002 | Height or altitude |  |  |
| 0 12 001 | Temperature/air temperature |  |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 11 031 | Degree of turbulence |  |  |
| 0 11 032 | Height of base of turbulence |  |  |
| 0 11 033 | Height of top of turbulence |  |  |
| 0 20 041 | Airframe icing |  |  |

3 11 002 (ACARS reports)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 065 | ACARS identification |  |  |
| 3 01 066 | ACARS location |  |  |
| 3 11 003 | ACARS standard reported variables |  |  |
| 3 11 004 | ACARS supplementary reported variables |  |  |

3 11 003 (ACARS standard reported variables)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 10 070 | Indicated aircraft altitude |  |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 12 001 | Temperature/air temperature |  |  |
| 0 13 002 | Mixing ratio |  |  |

3 11 004 (ACARS supplementary reported variables)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 11 034 | Vertical gust velocity |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 11 035 | Vertical gust acceleration |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 11 075 | Mean turbulence intensity (eddy dissipation rate) |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 11 076 | Peak turbulence intensity (eddy dissipation rate) |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 33 025 | ACARS interpolated values indicator |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 33 026 | Moisture quality |  |  |

3 11 005 (Standard AMDAR reports)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 01 008 | Aircraft registration number or other identification |  |  |
| 0 01 023 | Observation sequence number |  |  |
| 3 01 021 | Latitude/longitude (high accuracy) |  |  |
| 3 01 011 | Year, month, day |  |  |
| 3 01 013 | Hour, minute, second |  |  |
| 0 07 010 | Flight level |  |  |
| 0 08 009 | Detailed phase of flight |  |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 11 031 | Degree of turbulence |  |  |
| 0 11 036 | Maximum derived equivalent vertical gust speed |  |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 33 025 | ACARS interpolated values indicator |  |  |

3 11 006 (AMDAR data or aircraft data for one level without latitude/longitude)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 010 | Flight level |  |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 02 064 | Aircraft roll angle quality |  |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 12 103 | Dewpoint temperature |  |  |

3 11 007 (Aircraft data for one level with latitude/longitude indicated)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 07 010 | Flight level |  |  |
| 3 01 021 | Latitude/longitude (high accuracy) |  |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 02 064 | Aircraft roll angle quality |  |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 12 103 | Dewpoint temperature |  |  |

3 11 008 (Aircraft ascent/descent profile without latitude/longitude indicated at each level)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 01 008 | Aircraft registration number or other identification |  |  |
| 3 01 011 | Year, month, day |  |  |
| 3 01 013 | Hour, minute, second |  |  |
| 3 01 021 | Latitude/longitude (high accuracy) |  |  |
| 0 08 004 | Phase of aircraft flight |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 11 006 | AMDAR data or aircraft data for one level without latitude/longitude |  |  |

3 11 009 (Aircraft ascent/descent profile with latitude/longitude given for each level)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 01 008 | Aircraft registration number or other identification |  |  |
| 3 01 011 | Year, month, day |  |  |
| 3 01 013 | Hour, minute, second |  |  |
| 3 01 021 | Latitude/longitude (high accuracy) |  |  |
| 0 08 004 | Phase of aircraft flight |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 11 007 | Aircraft data for one level with latitude/longitude indicated |  |  |

3 11 010 (BUFR template for AMDAR, version 7)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 01 008 | Aircraft registration number or other identification |  |  |
| 0 01 023 | Observation sequence number |  |  |
| 0 01 006 | Aircraft flight number |  |  |
| 0 01 110 | Aircraft tail number |  |  |
| 0 01 111 | Origination airport |  |  |
| 0 01 112 | Destination airport |  |  |
| 2 04 002 | Add associated field | 2 bits long |  |
| 0 31 021 | Associated field significance | = 8 Two bits quality information |  |
| 3 01 011 | Year, month, day |  |  |
| 3 01 013 | Hour, minute, second |  |  |
| 3 01 021 | Latitude/longitude (high accuracy) |  |  |
| 0 07 010 | Flight level | Pressure altitude |  |
| 0 10 053 | Global navigation satellite system altitude |  |  |
| 0 08 009 | Detailed phase of flight |  |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 02 064 | Aircraft roll angle quality |  |  |
| 0 11 100 | Aircraft true airspeed |  |  |
| 0 11 101 | Aircraft ground speed u-component |  |  |
| 0 11 102 | Aircraft ground speed v-component |  |  |
| 0 11 103 | Aircraft ground speed w-component |  |  |
| 0 11 104 | True heading of aircraft, ship or other mobile platform | True heading of aircraft |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 02 170 | Aircraft humidity sensors |  |  |
| 2 01 144 | Change data width |  |  |
| 2 02 133 | Change scale |  |  |
| 0 13 002 | Mixing ratio |  |  |
| 2 02 000 | Change scale | Cancel |  |
| 2 01 000 | Change data width | Cancel |  |
| 2 01 135 | Change data width |  |  |
| 2 02 130 | Change scale |  |  |
| 0 13 003 | Relative humidity |  |  |
| 2 02 000 | Change scale | Cancel |  |
| 2 01 000 | Change data width | Cancel |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 12 103 | Dewpoint temperature |  |  |
| 0 33 026 | Moisture quality |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 20 042 | Airframe icing present |  |  |
| 1 03 000 | Delayed replication of 3 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 20 043 | Peak liquid water content |  |  |
| 0 20 044 | Average liquid water content |  |  |
| 0 20 045 | Supercooled large droplet (SLD) conditions |  |  |
| 1 01 000 | Delayed replication of 1 descriptor |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 33 025 | ACARS interpolated values indicator |  |  |
| 1 03 000 | Delayed replication of 3 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 11 075 | Mean turbulence intensity (eddy dissipation rate) |  |  |
| 0 11 076 | Peak turbulence intensity (eddy dissipation rate) |  |  |
| 0 11 039 | Extended time of occurrence of peak eddy dissipation rate |  |  |
| 1 02 000 | Delayed replication of 2 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 11 037 | Turbulence index | EDR |  |
| 0 11 077 | Reporting interval or averaging time for eddy dissipation rate |  |  |
| 1 03 000 | Delayed replication of 3 descriptors |  |  |
| 0 31 000 | Short delayed descriptor replication factor |  |  |
| 0 11 034 | Vertical gust velocity |  |  |
| 0 11 035 | Vertical gust acceleration |  |  |
| 0 11 036 | Maximum derived equivalent vertical gust speed |  |  |
| 2 04 000 | Add associated field | Cancel |  |
| 1 19 000 | Delayed replication of 19 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 3 01 011 | Year, month, day |  |  |
| 3 01 013 | Hour, minute, second |  |  |
| 3 01 021 | Latitude/longitude (high accuracy) |  |  |
| 0 07 007 | Height |  |  |
| 0 11 105 | EDR algorithm version |  |  |
| 2 04 007 | Add associated field | 7 bits long |  |
| 0 31 021 | Associated field significance | = 7 Percentage confidence |  |
| 0 11 076 | Peak turbulence intensity (eddy dissipation rate) |  |  |
| 0 11 075 | Mean turbulence intensity (eddy dissipation rate) |  |  |
| 2 04 000 | Add associated field | Cancel |  |
| 0 11 106 | Running minimum confidence |  |  |
| 0 11 107 | Maximum number bad inputs |  |  |
| 0 11 108 | Peak location |  |  |
| 0 11 109 | Number of good EDR |  |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 11 001 | Wind direction |  |  |
| 2 01 130 | Change data width |  |  |
| 0 11 084 | Wind speed |  |  |
| 2 01 000 | Change data width | Cancel |  |

3 11 011 (IAGOS template for a single observation), version 2

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 0 01 023 | Observation sequence number |  |  |
| 0 08 004 | Phase of aircraft flight |  |  |
| 3 01 011 | Year, month, day |  |  |
| 3 01 013 | Hour, minute, second |  |  |
| 0 05 002 | Latitude (coarse accuracy) |  |  |
| 0 06 002 | Longitude (coarse accuracy) |  |  |
| 0 07 004 | Pressure |  |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 1 06 000 | Delayed replication of 6 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 08 046 | Atmospheric chemical or physical constituent type |  |  |
| 2 01 139 | Change data width | 20 bits long |  |
| 2 02 126 | Change scale | Scale: 7 |  |
| 0 15 026 | Concentration of pollutant (mol mol-1) |  |  |
| 2 02 000 | Change scale | Cancel |  |
| 2 01 000 | Change data width | Cancel |  |
| 1 06 000 | Delayed replication of 6 descriptors |  |  |
| 0 31 001 | Delayed descriptor replication factor |  |  |
| 0 08 046 | Atmospheric chemical or physical constituent type |  |  |
| 2 01 138 | Change data width | 19 bits long |  |
| 2 02 130 | Change scale | Scale: 11 |  |
| 0 15 026 | Concentration of pollutant (mol mol-1) |  |  |
| 2 02 000 | Change scale | Cancel |  |
| 2 01 000 | Change data width | Cancel |  |
| 0 15 052 | Log10 of number density of aerosol particles with diameter greater than 5 nm |  |  |
| 0 15 053 | Log10 of number density of aerosol particles with diameter greater than 14 nm |  |  |
| 0 15 054 | Log10 of number density of aerosol particles with diameter between 0.25 and 2.5 µm |  |  |
| 0 15 055 | Non volatile aerosol ratio |  |  |
| 0 07 004 | Pressure |  |  |
| 0 07 004 | Pressure |  |  |
| 0 13 099 | Log10 of integrated cloud particle density |  |  |
| 0 13 100 | Log10 of integrated cloud particle area |  |  |
| 0 13 101 | Log10 of integrated cloud particle volume |  |  |

3 11 012 (BUFR template for aircraft ascent/descent profile with latitude and longitude given for each level)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 150 | WIGOS identifier |  |  |
| 0 01 008 | Aircraft registration number or other identification |  |  |
| 0 01 111 | Origination airport | To be reported in case of an ascent profile |  |
| 0 01 112 | Destination airport | To be reported in case of an descent profile |  |
| 3 01 011 | Year, month, day | Date/time and position of first level in profile (for ascent profile first report/take off, for descent profile last report/touch down) |  |
| 3 01 013 | Hour, Min, second | Date/time and position of first level in profile (for ascent profile first report/take off, for descent profile last report/touch down) |  |
| 3 01 021 | Latitude/Longitude | Date/time and position of first level in profile (for ascent profile first report/take off, for descent profile last report/touch down) |  |
| 0 08 004 | Phase of flight | Ascent or descent profile |  |
| 1 10 000 | Delayed replication of 10 descriptors |  |  |
| 0 31 002 | Extended delayed descriptor replication factor | Number of levels following |  |
| 3 01 011 | Year, month, day | Date of single level report |  |
| 3 01 013 | Hour, Min, second | Time of single level report |  |
| 3 11 007 | Aircraft data for one level with latitude/longitude indicated | Flight level = pressure altitude |  |
| 2 01 144 | Change data width |  |  |
| 2 02 133 | Change scale |  |  |
| 0 13 002 | Mixing ratio | naly measured value |  |
| 2 02 000 | Cancel change scale |  |  |
| 2 01 000 | Cancel change data width |  |  |
| 0 13 003 | Relative humidity | Value derived from Mixing ratio, limited to ≤ 100% |  |
| 0 33 026 | Moisture quality | Code table |  |

3 11 013 (Uncrewed aircraft systems (UAS))

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 150 | WIGOS identifier |  |  |
| 0 01 008 | Aircraft registration number or other identification |  |  |
| 0 01 095 | Observer identification |  |  |
| 3 01 011 | Year, month, day |  |  |
| 3 01 013 | Hour, minute, second |  |  |
| 3 01 021 | Latitude/longitude (high accuracy) |  |  |
| 0 01 013 | Speed of motion of moving observing platform |  |  |
| 0 08 009 | Detailed phase of flight |  |  |
| 0 07 010 | Flight level |  |  |
| 0 33 003 | Quality information |  |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 12 101 | Temperature/air temperature |  |  |
| 0 02 170 | Aircraft humidity sensor |  |  |
| 2 01 144 | Change data width |  |  |
| 2 02 133 | Change scale |  |  |
| 0 13 002 | Mixing ratio |  |  |
| 2 02 000 | Change scale | Cancel |  |
| 2 01 000 | Change data width | Cancel |  |
| 2 01 135 | Change data width |  |  |
| 2 02 130 | Change scale |  |  |
| 0 13 003 | Relative humidity |  |  |
| 2 02 000 | Change scale | Cancel |  |
| 2 01 000 | Change data width | Cancel |  |
| 0 11 073 | Turbulent kinetic energy |  |  |
| 0 11 075 | Mean turbulence intensity (eddy dissipation rate) |  |  |

3 11 014 (Template for drifting balloon systems)

| TABLE REFERENCES | ELEMENT NAME | ELEMENT DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 3 01 150 | WIGOS identifier |  |  |
| 3 01 005 | Originating centre/sub-centre |  |  |
| 3 01 134 | Balloon and instrumentation information |  |  |
| 3 01 011 | Year, month, day |  |  |
| 3 01 013 | Hour, minute, second |  |  |
| 3 01 021 | Latitude/longitude (high accuracy) |  |  |
| 0 07 007 | Height | Is derived from GNSS measurements |  |
| 0 33 024 | Station elevation quality mark (for mobile stations) |  |  |
| 0 08 021 | Time significance | = 2 Time averaged |  |
| 0 25 170 | Sampling interval (time) | Seconds |  |
| 0 04 026 | Time period or displacement |  |  |
| 0 07 077 | Height of sensor with respect to station |  |  |
| 0 07 004 | Pressure |  |  |
| 0 10 009 | Geopotential height |  |  |
| 0 08 021 | Time significance | = 2 Time averaged |  |
| 0 25 170 | Sampling interval (time) | Seconds |  |
| 0 04 026 | Time period or displacement |  |  |
| 0 07 077 | Height of sensor with respect to station |  |  |
| 0 12 001 | Temperature/air temperature |  |  |
| 0 08 021 | Time significance | = 2 Time averaged |  |
| 0 25 170 | Sampling interval (time) | Seconds |  |
| 0 04 026 | Time period or displacement |  |  |
| 0 07 077 | Height of sensor with respect to station |  |  |
| 0 12 003 | Dewpoint temperature |  |  |
| 2 01 135 | Change data width |  |  |
| 2 02 130 | Change scale |  |  |
| 0 13 003 | Relative humidity |  |  |
| 2 02 000 | Change scale | Cancel |  |
| 2 01 000 | Change data width | Cancel |  |
| 2 01 144 | Change data width |  |  |
| 2 02 133 | Change scale |  |  |
| 0 13 002 | Mixing ratio |  |  |
| 2 02 000 | Change scale | Cancel |  |
| 2 01 000 | Change data width | Cancel |  |
| 0 08 021 | Time significance | = 2 Time averaged |  |
| 0 25 170 | Sampling interval (time) | Seconds |  |
| 0 04 026 | Time period or displacement |  |  |
| 0 07 077 | Height of sensor with respect to station |  |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 0 08 023 | First-order statistics | =10 means standard-deviation (N) |  |
| 2 02 130 | Change scale |  |  |
| 0 07 004 | Pressure |  |  |
| 2 02 000 | Change scale | Cancel |  |
| 2 02 129 | Change scale |  |  |
| 0 12 001 | Temperature/air temperature |  |  |
| 0 12 003 | Dewpoint temperature |  |  |
| 0 13 003 | Relative humidity |  |  |
| 2 02 000 | Change scale | Cancel |  |
| 2 02 132 | Change scale |  |  |
| 0 13 002 | Mixing ratio |  |  |
| 2 02 000 | Change scale | Cancel |  |
| 2 02 129 | Change scale |  |  |
| 0 11 001 | Wind direction |  |  |
| 0 11 002 | Wind speed |  |  |
| 2 02 000 | Change scale | Cancel |  |
| 0 08 023 | First-order statistics | Cancel |  |